Faro Mine Complex, Fall 2014 Groundwater Sampling

Prepared for:

Yukon Government

Prepared by:

Hemmera Envirochem Inc.

230 – 2237 2nd Avenue

Whitehorse, YT  Y1A 0K7

Ecological Logistics & Research Ltd.

204-105 Titanium Way

Whitehorse, YT  Y1A 0E7

File: 1343-005.02

November 2014

TABLE OF CONTENTS

1.0	Introduction	1

1.1	Site Location	1

1.2	Scope of Work	1

1.3	Sample Sites	2

2.0	Methodology	10

2.1	Protocols	10

2.2	Well Measurements and Purging	10

2.3	Field Parameters	11

2.4	Groundwater Quality Sample Collection	11

2.5	Data Analysis	12

2.6	Quality Assurance and Quality Control (QA/QC)	12

2.6.1	Field QA/QC	12

2.6.2	Laboratory and Sampling QA/QC	12

3.0	Results	13

3.1	Groundwater Sampling Summary	13

3.2	Analytical Results	18

3.2.1	Cross Valley Dam	18

3.2.2	Down Gradient of Cross Valley Dam	18

3.2.3	ETA Area	19

3.2.4	Haul Road/Near NFRC	19

3.2.5	Intermediate Dam	20

3.2.6	Intermediate Dump	20

3.2.7	Main Dump	20

3.2.8	Mill Area	20

3.2.9	NFRC Rock Drain Pond	21

3.2.10	Northeast Dumps	21

3.2.11	Second Impoundment	21

3.2.12	S-Wells Area	22

3.2.13	NFRC near S-Wells Area	23

3.2.14	Upstream of Tailings	23

3.2.15	Groundwater Vangorda/Grum	23

3.3	Quality Assurance and Quality Control Results	24

3.3.1	Field and Travel Blanks	24

3.3.2	Field Duplicates	24

4.0	Recommendations	26

5.0	Closure	27

6.0	References	28

7.0	Statement of Limitations	29

List of Tables (within text)

Table 1-1 	Summary of Groundwater Sample Sites Identified for 2014 Fall Program	2

Table 2-1 	Groundwater Sampling – Field Parameter Purging Criteria	11

Table 2-2 	Groundwater Sampling – Preservation and Intended Analysis	11

Table 3-1 	Groundwater Field Parameters and Well Measurements for 2014 Fall Sampling Program	14

List of Tables (following text)

Table 3-2 	Groundwater Sampling Analytical Results and CCME Guideline Exceedances for 2014 Spring Sampling Program

Table 3-3 	Quality Assurance and Quality Control Analytical Results for 2014 Spring Groundwater Sampling Program

List of Figures

Figure 1-1 	Site Location – Faro Mine Complex	6

Figure 1-2 	Groundwater Sampling Locations – Faro Mine Area	7

Figure 1-3 	Groundwater Sampling Locations – S-Wells Area	8

Figure 1-4 	Groundwater Sampling Locations – Vangorda/Grum Mine Area	9

List of Appendices

Appendix A	Site Photos

Appendix B	Laboratory Reports

Appendix C	Field Forms

# Introduction

Hemmera and Ecological Logistics & Research Ltd. (ELR) were retained by the Government of Yukon (GY), Assessment and Abandoned Mines (AAM) to conduct the 2014 groundwater sampling program at the Faro Mine Complex (FMC).The program consists of two sampling events: spring (May/June) and fall (September). This report summarizes the activities completed and analytical results from the fall sampling event.

## Site Location

The FMC is located approximately thirteen (13) kilometres (km) northeast of the Town of Faro, Yukon (20 km by road). The FMC consists of two distinct areas, the Faro Mine Area and the Vangorda/Grum Area (Figure 1-1), which are connected by a fourteen (14) km roadway (the Haul Road; Figure 1-1). Groundwater sampling stations exist throughout the FMC and surrounding area, a series of which were sampled during the fall 2014 program. Specific sampling locations and general sample site distribution are described in Sections 1.2 and 1.3.

## Scope of Work

The scope of work (SOW) included the coordination and execution of the fall groundwater sampling program and the preparation of this summary report. This report provides a summary of the sampling program activities, methodologies (including any deviations from standard methodologies), field in-situ and laboratory analytical results, concentrations of contaminants exceeding the applicable guidelines, and recommendations relating to sample procedures and monitoring well conditions. This report does not provide an interpretation of the analytical results or provide recommendations relating to the program. The fall groundwater sampling event at the FMC was conducted over a ten (10) day period between September 9 and October 15, 2014. A total of 119 groundwater wells were specified by AAM for the fall sampling event (Table 1-1), including 18 wells recently installed in 2014. The majority of sampling was completed between September 9 and 17, 2014 (118 wells). One (1) additional well was sampled on October 15, 2014. Sampling was conducted by a team of four (4) field staff from Hemmera/ELR.

At each well (sampling station) the groundwater level and depth to bottom of the well were measured, the well was purged appropriately, and field parameters were measured (pH, water temperature, and conductivity). Groundwater samples were collected following field measurements and purging, and were analysed for general groundwater quality chemistry (major anions/cations and physical parameters), total metals, and dissolved metals. A detailed description of the sampling methodology is provided in Section 2, below.

## Sample Sites

Fall groundwater sampling was conducted at 119 wells across 16 different areas of the FMC (Table 1-1; Figures 1-1 to 1-4). The majority of fall sample sites were located in the Faro Mine Area (96 wells), with the remaining wells located in the Vangorda/Grum Area (23 wells). A large portion of the wells sampled in the Faro Mine Area were located in the S-Wells Area (31 wells; Figure 1-3), with additional wells in the surrounding areas. Wells in the Vangorda/Grum Area were primarily located in the vicinity of the Grum Sulphide Cell (Figure 1-4). In addition to the one hundred (100) wells identified in the SOW document, nineteen (19) wells in the vicinity of North Fork Rose Creek (NFRC) had been installed during 2014 and were added to the fall sampling program at the request of AAM, including four (4) shallow wells downstream of the Haul Road, six (6) shallow wells upstream of the Haul Road, and nine (9) deep wells located along the Haul Road near the NFRC. Table 1-1 summarizes sample sites included in the fall sampling program, while Figures 1-2 through 1-4 show locations and general distribution of the sites. Photographs of each sample site are included as Appendix A.

Table 1-1 	Summary of Groundwater Sample Sites Identified for 2014 Fall Program

| Area | Well Name | UTM (Zone 8N) |  | Well Status | Sample Successfully Collected | QA/QC Sample Collected |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Easting | Northing |  |  |  |
| Cross Valley Dam (CVD) | P01-11 | 580092 | 6914486 | Good |  | Duplicate |
|  | P03-09-6 | 579948 | 6914411 | Good |  | - |
|  | P03-09-9 | 579948 | 6914411 | Good |  | - |
|  | P05-01-03 | 580056 | 6914505 | Good |  | - |
|  | P05-01-05 | 580056 | 6914505 | Good |  | - |
|  | P09-C2 | 580012 | 6914402 | Good |  | - |
|  | P09-C3 | 579970 | 6914316 | Good |  | - |
| Down Gradient of CVD | P01-01A | 579701 | 6914854 | Good |  | - |
|  | P01-01B | 579701 | 6914854 | Good |  | Duplicate |
| Emergency Tailings (ETA) Area | P09-ETA-1 | 582700 | 6913813 | Good |  | - |
|  | P09-ETA-2 | 582700 | 6913812 | Good |  | - |
|  | P96-8A | 583220 | 6914072 | Good |  | - |
|  | P96-8B | 583220 | 6914072 | Good |  | - |
|  | SRK04-3A | 582870 | 6913995 | Good |  | - |
|  | SRK05-ETA-BR1 | 582863 | 6914019 | Good |  | - |
|  | SRK05-ETA-BR2 | 582879 | 6913997 | Good |  | Duplicate |
| Haul Road Near NFRC | MW14-02D | 584758 | 6913127 | Good |  | - |
|  | MW14-02S | 584758 | 6913127 | Good |  | - |
|  | MW14-03 | 584613 | 6913290 | Good |  | Duplicate |
|  | MW14-04D | 584648 | 6913321 | Good |  | - |
|  | MW14-04S | 584658 | 6913321 | Good |  | - |
|  | MW14-05 | 584694 | 6913345 | Good |  | Duplicate |
| Haul Road Near NFRC | PW14-01 | 584752 | 6913151 | Good |  | - |
|  | PW14-06 | 584476 | 6913310 | Good |  | - |
|  | PW14-07 | 584690 | 6913192 | Good |  | - |
| Intermediate Dam | P01-03 | 580516 | 6914255 | Good |  | - |
|  | P01-04A | 580372 | 6914074 | Good |  | - |
|  | P01-04B | 580372 | 6914074 | Good |  | - |
|  | X24-96D | 580544 | 6914298 | Good |  | - |
|  | X25-96A | 580544 | 6914298 | Good |  | - |
|  | X25-96B | 580407 | 6914119 | Good |  | - |
| Intermediate Dump | P96-6 | 584902 | 6913313 | Good |  | - |
| Main Dump | SRK08-P9 | 583682 | 6913620 | Good |  | - |
| Mill Area | SRK08-10A | 582718 | 6914052 | Good |  | - |
|  | SRK08-11A | 582583 | 6914571 | Good | - | - |
|  | SRK08-11B | 582587 | 6914571 | Good |  | - |
| NFRC Rock Drain Pond | MW14-12D | 584858 | 6913264 | Good |  | - |
|  | MW14-12S | 584858 | 6913264 | Good |  | - |
|  | MW14-13 | 584921 | 6913285 | Good |  | Duplicate |
|  | MW14-14 | 584821 | 6913250 | Good |  | - |
|  | MW14-15 | 584831 | 6913263 | Good |  | - |
|  | MW14-16 | 584887 | 6913289 | Good |  | - |
| Northeast Dumps | BH13B | 585746 | 6914494 | Good |  | - |
|  | BH14A | 585584 | 6914005 | Good |  | - |
|  | BH14B | 585584 | 6914005 | Good |  | - |
| Second Impoundment | P03-01-2 | 583179 | 6912761 | Good |  | - |
|  | P03-01-8 | 583179 | 6912761 | Good |  | - |
|  | P03-03-2 | 583068 | 6912698 | Good |  | - |
|  | P03-03-4 | 582484 | 6913117 | Good |  | - |
|  | P03-03-9 | 582948 | 6912879 | Good |  | - |
|  | P03-05-4 | 582605 | 6912934 | Good |  | - |
|  | P03-06-1 | 582452 | 6913496 | Good |  | - |
|  | P03-06-2 | 582452 | 6913496 | Good |  | - |
|  | P03-06-6 | 582452 | 6913496 | Good |  | - |
|  | P03-06-7 | 582452 | 6913496 | Dry | - | - |
| S-Wells Area S-Wells Area | P09-SIS1 | 584478 | 6913128 | Good |  | - |
|  | P09-SIS2 | 584487 | 6913125 | Good |  | Field Blank |
|  | P09-SIS3 | 584493 | 6913117 | Good |  | - |
|  | P09-SIS4 | 584512 | 6913107 | Good |  | - |
|  | P09-SIS5 | 584515 | 6913110 | Good |  | - |
|  | P96-7 | 584127 | 6913287 | Good |  | Duplicate |
|  | S1A | 584433 | 6913114 | Good |  | - |
|  | S1B | 584433 | 6913114 | Good |  | - |
|  | S2A | 584471 | 6913123 | Good |  | - |
|  | S2B | 584471 | 6913123 | Good |  | - |
|  | SRK05-SP-1A | 584621 | 6913075 | Good |  | - |
|  | SRK05-SP-1B | 584621 | 6913075 | Good |  | Duplicate |
|  | SRK05-SP-3A | 584543 | 6913094 | Good |  | - |
|  | SRK05-SP-3B | 584543 | 6913094 | Good |  | - |
|  | SRK05-SP-4A | 584506 | 6913110 | Good |  | - |
|  | SRK05-SP-4B | 584506 | 6913110 | Good |  | - |
|  | SRK05-SP-5 | 584467 | 6913133 | Good |  | - |
|  | SRK05-SP-6 | 584492 | 6912975 | Dry | - | - |
|  | SRK08-SBR1 | 584472 | 6913128 | Broken | - | - |
|  | SRK08-SBR2 | 584484 | 6913123 | Good |  | - |
|  | SRK08-SBR3 | 584394 | 6913146 | Good |  | - |
|  | SRK08-SBR4 | 584447 | 6913140 | Good |  | - |
|  | SRK08-SP-7A | 584437 | 6913095 | Good |  | - |
|  | SRK08-SP-7B | 584437 | 6913095 | Good |  | Duplicate |
|  | SRK08-SP-8A | 584295 | 6912952 | Good |  | - |
|  | SRK08-SP-8B | 584295 | 6912952 | Good |  | - |
| NFRC Near S-Wells Area | MW14-08 | 584701 | 6913035 | Good |  | - |
|  | MW14-09 | 584691 | 6913044 | Good |  | - |
|  | MW14-10 | 584679 | 6913040 | Good |  | - |
|  | MW14-11 | 584677 | 6913028 | Good |  | - |
|  | SRK05-SP-2 | 584681 | 6913035 | Good |  | - |
| Upstream of Tailings | TH86-2 | 583536 | 6912494 | Good |  | - |
|  | TH86-5 | 583587 | 6912570 | Good |  | - |
| Vangorda/Grum Vangorda/Grum | BH05-9B-R | 592640 | 6903347 | Good |  | - |
|  | P09-GS1A | 592494 | 6904832 | Good |  | - |
|  | P09-GS1B | 592485 | 6904833 | Good |  | - |
|  | P09-LCD1 | 593358 | 6903316 | Good |  | - |
|  | P09-LCD4 | 593330 | 6903278 | Good |  | - |
|  | P09-LCD6 | 593313 | 6903251 | Good |  | - |
|  | P09-VC1 | 593518 | 6903418 | Good |  | - |
|  | P09-VC2 | 593517 | 6903431 | Good |  | - |
|  | P2001-02A | 593132 | 6902866 | Good |  | - |
|  | P2001-02B | 593132 | 6902866 | Good |  | - |
|  | P2001-3 | 593099 | 6902881 | Good |  | - |
|  | P96-9A | 592648 | 6903345 | Good |  | - |
|  | SRK05-07 | 592375 | 6903189 | Good |  | - |
|  | SRK05-08 | 592582 | 6903238 | Good |  | - |
|  | SRK05-5C | 592763 | 6903382 | Good |  | - |
|  | SRK05-9 | 592949 | 6903158 | Good |  | - |
|  | SRK08-P14 | 591759 | 6903699 | Good |  | Duplicate |
|  | SRK08-P15 | 591960 | 6903534 | Good |  | - |
|  | SRK08-P16 | 592198 | 6903145 | Insufficient Volume | - | - |
|  | V34 | 593428 | 6902476 | Good |  | Duplicate |
|  | V35 | 593175 | 6902554 | Good |  | - |
|  | V36 | 593133 | 6902916 | Good |  | - |
|  | V37 | 593309 | 6903079 | Good |  | Field Blank |
| Zone 2 Outwash/Pit | BH10A | 585122 | 6913711 | Good |  | - |
|  | BH10B | 585122 | 6913711 | Good |  | - |
|  | BH5 | 585088 | 6913550 | Good |  | - |
|  | BH6 | 585092 | 6913645 | Good |  | Duplicate |
|  | BH8 | 585144 | 6913777 | Good |  | - |
|  | P05-04 | 585115 | 6913650 | Good |  | - |
|  | SRK08-P12A | 585228 | 6913687 | Good |  | - |
|  | SRK08-P12B | 585228 | 6913687 | Good |  | - |

Figure 1-1 	Site Location – Faro Mine Complex

Figure 1-2 	Groundwater Sampling Locations – Faro Mine Area

Figure 1-3 	Groundwater Sampling Locations – S-Wells Area

Figure 1-4 	Groundwater Sampling Locations – Vangorda/Grum Mine Area

# Methodology

## Protocols

Groundwater purging and sampling conducted by Hemmera/ELR was in accordance with Yukon Environment’s Protocol for the Contaminated Sites Regulation #7 – Groundwater Monitoring Well Installation, Sampling and Decommissioning (Yukon Environment, March 2011). Methods used were also consistent with the ASTM D4448-01 Standard Guide for Sampling Groundwater Monitoring Wells (ASTM, 2013), the D6452-99 Guide for Purging Methods for Wells used for Groundwater Quality Investigations (ASTM, 2012) and in accordance with Standard Methods for the Examination of Water and Wastewater (Rice et al., 2012).

## Well Measurements and Purging

Upon arriving at each location, the well structure and casing were inspected for damage, closure, and general conditions. Several measurements were recorded from each well, including Depth-to-Water (DTW; m), Depth-to-Bottom (DTB; m), well diameter (cm), and well stick-up height (m).

DTB and DTW were measured using either a Solinst - Model 102 Water Level Meter (for 2.54 cm diameter wells) or a Heron Water Tape (for wells with diameter greater than 2.54 cm). DTB and DTW were measured from (in order of preference): 1) a black mark drawn on the top of the well; 2) the bottom of the most significant notch found on the top of the PVC if a mark was not present; or 3) a line was drawn on the highest point of the well and measurement taken from that line if no distinguishable point of measure was present. Based on information reviewed by Hemmera/ELR, it is unknown where the point of measurement was for previous sampling programs. Stick-up height was measured from the lowest point on the bottom of the well casing to the highest point (or distinguishing mark) on the well. Water level meters were decontaminated between each sample site using a combination of Alconox low-foaming phosphate-free detergent solution and de-ionized water.

Next, groundwater wells were purged and sampled using one of four (4) techniques: 1) Hydrolift electric pump using dedicated high density polyethelene (HDPE) Waterra tubing and footvalve, 2) manual purging using dedicated HDPE Waterra tubing and footvalve, 3) GeoPump peristaltic pump using dedicated HDPE and silicone tubing, or 4) Grundfos Redi-Flo2 submersible pump using dedicated HDPE tubing. The purging technique chosen for each well was that which would produce the most representative groundwater sample.

Groundwater wells were determined to be sufficiently purged when either three successive field parameter measurements were recorded to be within an allowable tolerance level (as summarized in Table 2-1, below), or when a volume of groundwater equivalent to three standing well volumes of groundwater had been purged. Groundwater turbidity measured in Nephelometric Turbidity Units (NTU) was also measured prior to sampling and was used as an indication of sample quality. Where possible samples were not collected until turbidity was less than 50 NTU.

Purge volume measurements were collected using a graduated container and stop watch. All well measurements, purging details, and additional field notes were recorded on field forms, this information is presented in Table 3-1.

Table 2-1 	Groundwater Sampling – Field Parameter Purging Criteria

| Field Parameter | Allowable Variance in 3 Consecutive Readings |
| --- | --- |
| Temperature (°C) | 3% |
| pH (pH Units) | +/- 0.1 |
| Conductivity (µS/cm) | 3% |

## Field Parameters

Hemmera/ELR measured general field parameters using either a Hanna 991301 field meter or a YSI Professional Plus multi-parameter meter. All field parameters were collected using a flow through cell in order to minimize field parameter variability. Field parameters recorded at each sample site included; groundwater temperature (oC), conductivity (μs/cm), and pH (pH Units). Where possible, field parameters were recorded throughout the purging process at five-minute intervals. For wells with slow recharge field parameters were recorded at volume related intervals (e.g., every 500 mL). Groundwater turbidity was measured at the time of sample collection using either a LaMotte 2020we or a Hach 2100Q Portable turbidity meter.

## Groundwater Quality Sample Collection

Groundwater quality samples were collected and preserved in accordance with laboratory directions, and using techniques consistent with Standard Methods for the Examination of Water and Wastewater (Rice et al., 2012). ALS Global was the analytical subcontractor chosen for this project, and an example summary of the sample set collected at each sample location, including parameters analysed and preservation techniques, is provided in Table 2-2.

Table 2-2 	Groundwater Sampling – Preservation and Intended Analysis

| Bottle Type | Parameters Analyzed | Sample Treatment | Preservation Added |
| --- | --- | --- | --- |
| 120 mL (Plastic) | Total Metals | Preserved | HNO3 |
| 120 mL (Plastic) | Dissolved Metals | Field Filtered and Preserved | HNO3 |
| 1 L (Plastic) | Acidity, alkalinity, chloride, conductivity, pH, sulphate, total suspended solids (TSS) | - | None |

## Data Analysis

Groundwater analytical results were compared to the Canadian Council of Ministers of the Environment (CCME) Water Quality Guidelines for the Protection of Freshwater Aquatic Life (FAL; CCME, 2014). All relevant CCME FAL guidelines are presented in Table 3-2.

## Quality Assurance and Quality Control (QA/QC)

### Field QA/QC

Several controls were used by Hemmera/ELR staff while in the field to help ensure that sample integrity was maintained and that data was recorded completely and accurately. All equipment used during the sampling process was dedicated to individual wells, including HDPE tubing and Waterra footvalves, laboratory provided pre-cleaned sample bottles, disposable filters, and disposable syringes. The only exception to this was a Grundfos Redi-Flo2 submersible pump that was required for several deep wells in the fall program. Field staff wore dedicated disposable nitrile gloves for all measurements, purging, and sampling. Water level meters and the submersible pump were cleaned using Alconox low-foaming phosphate-free detergent and de-ionized water and between wells, and field instruments (Hanna/YSI field meters and turbidity meters) were checked and/or calibrated before each site visit to ensure the parameters recorded were as accurate as possible.

Project-specific field data sheets were created for the sampling event to help ensure that all required measurements were taken, and that information was recorded correctly. Field data sheets have been included as Appendix B of this report.

### Laboratory and Sampling QA/QC

Laboratory and sampling QA/QC measures taken as part of the fall sampling program include the collection of travel blanks, duplicates, and field blanks, as outlined in the SOW and as per standard industry practice. Duplicate samples were collected at a ratio of 10% of the regular samples (12 duplicates were collected in relation to 119 sample sites). Additionally, two field blanks were collected, and two travel blanks accompanied the analytical supplies and samples during shipping to and from the laboratory.

The variation between sample and duplicate values was calculated as relative percent difference (RPD). RPD provides a measure of the relative difference between two values in comparison to their mean value, and is calculated as the difference between a sample and its field duplicate over the average of two values. RPD values greater than 20% indicate a potential error that has affected the precision of sampling or analysis. RPD was calculated according to the following formula:

%RPD=x1-x2x1+x22x 100

RPD is not calculated if either the sample or the field duplicate concentration is less than five times the detection limit. QA/QC analytical results including RPD values are presented in Table 3-3.

Laboratory replicates and additional quality control measures (i.e., measures against lab standards) were conducted by ALS. Laboratory QA/QC analytical results are included as Appendix A and discussed in Section 3.3.

# Results

Summary tables of the laboratory analytical results are presented in Table 3-2 of this report, including comparisons of results to CCME FAL guidelines. A summary of the QA/QC sampling results is also attached, including analytical data for duplicates, field blank, and travel blank (Table 3-3). Laboratory analytical reports are provided as Appendix A.

## Groundwater Sampling Summary

The majority of fall 2014 groundwater sampling was completed between September 9 and September 15, 2014 (110 wells), with the remaining wells sampled between September 16 and 17, 2014 (9 wells). A return trip to locate and sample well SRK08-P-16 was completed on October 15, 2014. Weather conditions varied throughout sampling with ambient air temperature ranging from -2 to 19°C. Weather conditions were pre-dominantly clear and sunny with periods of overcast conditions. All 119 groundwater wells specified for the fall sampling event were located and assessed by Hemmera/ELR. Groundwater samples were successfully collected at 115 of the 119 sampling locations as outlined in Table 1-1. Two (2) wells were dry (P03-06-7 and SRK05-SP-6), one (1) well was damaged (SRK08-SBR1), and one (1) had an insufficient groundwater volume to sample (SRK08-P-16). A summary of groundwater wells sampled during the fall 2014 sampling event, including field parameters and well measurements, is provided in Table 3-1. All samples were received by the laboratory within the required holding times and temperature limits.

A summary of the sampling results and guideline exceedances is provided in the following sections, organized by area.

Table 3-1 	Groundwater Field Parameters and Well Measurements for 2014 Fall Sampling Program

| Area | Well Name | Sample Date | Stick up height (m) | Depth To Water (m) | Depth To Bottom (m) | Standing Water volume (L) | Volume Purged (L) | Purge Start time | Purge End Time | Elapsed Purge Time | Purge Rate (l/min) | Criteria (3WV / PS) | Draw Down (m) | pH | Temperature (°C) | Conductivity (µs/cm)1 | Field Turbidity (NTU)2 | Method Used | Well diameter (inches) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cross Valley Dam (CVD) | P01-11 | 2014-09-10 | 1.20 | 1.202 | 11.060 | 20.0 | 6.00 | 10:15 | 10:35 | 0:20 | 0.30 | PS | 0.010 | 6.63 | 4.6 | 2242.0 | 6.69 | peristaltic | 2 |
|  | P03-09-6 | 2014-09-10 | 0.62 | 3.321 | 19.582 | 2.0 | 1.75 | 12:09 | 12:30 | 0:21 | 0.08 | PS | NR | 6.77 | 4.6 | 1155.0 | 9.95 | peristaltic | 1/2 |
|  | P03-09-9 | 2014-09-10 | 0.67 | 4.058 | 8.383 | 0.5 | 1.50 | 11:32 | 11:50 | 0:18 | 0.08 | 3WV | NR | 6.70 | 4.2 | 184.4 | 45.00 | peristaltic | 1/2 |
|  | P05-01-03 | 2014-09-10 | 0.56 | 1.555 | 17.765 | 2.0 | 1.00 | 14:25 | 14:37 | 0:12 | 0.08 | PS | NR | 6.36 | 6.7 | 2338.0 | 14.20 | peristaltic | 1/2 |
|  | P05-01-05 | 2014-09-10 | 0.62 | 2.036 | 6.548 | 0.6 | 1.00 | 13:52 | 14:04 | 0:12 | 0.08 | PS | NR | 6.42 | 6.8 | 2113.0 | 0.83 | peristaltic | 1/2 |
|  | P09-C2 | 2014-09-10 | 0.87 | 0.625 | 64.580 | 128.0 | 50.00 | 9:25 | 9:50 | 0:25 | 2.00 | PS | 6.769 | 6.45 | 4.1 | 1619.0 | 23.40 | hydrolift | 2 |
|  | P09-C3 | 2014-09-10 | 0.79 | 1.486 | 49.990 | 97.0 | 105.00 | 8:25 | 9:00 | 0:35 | 0.45 | PS | 0.157 | 6.69 | 3.9 | 845.0 | 0.62 | hydrolift | 2 |
| Down Gradient of CVD | P01-01A | 2014-09-09 | 0.64 | 3.652 | 20.345 | 33.5 | 35.00 | 10:35 | 11:05 | 0:30 | 1.17 | PS | 0.013 | 6.73 | 2.1 | 1062.0 | 0.46 | hydrolift | 2 |
|  | P01-01B | 2014-09-09 | 0.59 | 3.740 | 35.535 | 63.5 | 60.00 | 11:15 | 11:55 | 0:40 | 1.50 | PS | 0.020 | 7.40 | 2.4 | 848.0 | 0.63 | hydrolift | 2 |
| Emergency Tailings (ETA) Area | P09-ETA-1 | 2014-09-13 | 0.98 | 8.672 | 33.700 | 50.0 | 38.00 | 15:15 | 16:05 | 0:50 | 0.76 | PS | 7.423 | 7.89 | 7.5 | 275.6 | 11.40 | hydrolift | 2 |
|  | P09-ETA-2 | 2014-09-13 | 0.70 | 12.969 | 18.500 | 11.0 | 24.50 | 14:25 | 15:00 | 0:35 | 0.70 | PS | 0.041 | 6.30 | 5.2 | 4299.0 | 2.95 | hydrolift | 2 |
|  | P96-8A | 2014-09-11 | 0.78 | 2.431 | 4.833 | 4.8 | 5.50 | 16:26 | 16:46 | 0:20 | 0.28 | PS | 0.019 | 3.52 | 9.0 | >4000 | 2.09 | peristaltic | 2 |
|  | P96-8B | 2014-09-11 | 0.69 | 2.349 | 9.428 | 14.2 | 8.00 | 16:52 | 17:21 | 0:29 | 0.28 | PS | 0.011 | 5.03 | 7.7 | >4000 | 2.83 | peristaltic | 2 |
|  | SRK04-3A | 2014-09-13 | 0.65 | 6.133 | 12.380 | 12.5 | 14.00 | 13:10 | 13:45 | 0:35 | 0.40 | PS | 0.035 | 5.42 | 5.1 | 6285.0 | 3.63 | peristaltic | 2 |
|  | SRK05-ETA-BR1 | 2014-09-13 | 0.77 | 7.051 | 13.268 | 7.0 | 6.75 | 11:38 | 12:06 | 0:28 | 0.24 | PS | -0.001 | 5.17 | 5.8 | 6160.0 | 1.16 | peristaltic | 1.5 |
|  | SRK05-ETA-BR2 | 2014-09-13 | 0.47 | 4.939 | 19.342 | 7.0 | 7.00 | 12:15 | 12:50 | 0:35 | 0.20 | PS | 0.693 | 6.83 | 5.1 | 1689.0 | 4.17 | peristaltic | 1 |
| Haul Road/Road Near NFRC | MW14-02D | 2014-09-15 | 0.95 | 58.656 | 78.900 | 41.0 | 116.00 | 10:29 | 11:06 | 0:37 | 3.14 | PS | NR | 5.69 | 4.6 | 492.0 | 6.10 | subm. pump | 2 |
|  | MW14-02S | 2014-09-15 | 0.95 | 58.135 | 78.900 | 147.0 | 185.00 | 8:57 | 9:48 | 0:51 | 3.63 | PS | NR | 7.05 | 6.8 | 157.0 | 8.18 | subm. pump | 3.75 |
|  | MW14-03 | 2014-09-14 | 0.89 | 53.833 | 60.900 | 128.0 | 177.00 | 8:34 | 10:17 | 0:43 | 4.12 | PS | NR | 7.07 | 11.0 | 814.0 | 16.40 | subm. pump | 6 |
|  | MW14-04D | 2014-09-15 | 0.95 | 69.048 | 75.200 | 12.5 | 69.50 | 16:39 | 17:29 | 0:50 | 1.39 | PS | NR | 6.67 | 13.2 | 2855.0 | 13.20 | subm. pump | 2 |
|  | MW14-04S | 2014-09-17 | 0.95 | 61.180 | 62.630 | 7.0 | 10.00 | 15:30 | 16:23 | 0:53 | 0.19 | Criteria not met3 | 1.450 | 6.74 | 7.3 | >4000 | 11.18 | subm. pump | 3.75 |
|  | MW14-05 | 2014-09-15 | 0.86 | 53.350 | 66.900 | 246.0 | 187.50 | 14:55 | 15:41 | 0:46 | 0.76 | PS | NR | 6.42 | 11.3 | 3651.0 | 8.21 | subm. pump | 6 |
|  | PW14-01 | 2014-09-14 | 1.02 | 59.673 | 78.300 | 338.0 | 226.00 | 16:49 | 17:57 | 1:08 | 3.32 | PS | NR | 5.69 | 3.9 | 524.0 | 2.50 | subm. pump | 6 |
|  | PW14-06 | 2014-09-14 | 1.00 | 46.995 | 62.900 | 127.4 | 154.50 | 11:30 | 12:20 | 0:50 | 0.00 | PS | NR | 4.23 | 22.2 | >4000 | 2.52 | subm. pump | 4 |
|  | PW14-07 | 2014-09-14 | 0.93 | 66.297 | 76.800 | 192.0 | 195.00 | 14:28 | 15:08 | 0:40 | 0.81 | PS | NR | 5.52 | 3.8 | 796.0 | 2.54 | subm. pump | 6 |
| Intermediate Dam | P01-03 | 2014-09-09 | 0.44 | 2.936 | 9.645 | 13.5 | 13.50 | 13:45 | 14:20 | 0:35 | 0.39 | PS | 1.504 | 6.04 | 4.2 | 2372.0 | 8.75 | peristaltic | 2 |
|  | P01-04A | 2014-09-09 | 0.32 | 1.535 | 53.150 | 103.5 | 105.00 | 17:35 | 18:10 | 0:35 | 0.45 | PS | 0.060 | 6.52 | 3.6 | 682.0 | 1.11 | hydrolift | 2 |
|  | P01-04B | 2014-09-09 | 0.30 | 2.003 | 34.085 | 64.0 | 70.00 | 16:20 | 16:40 | 0:20 | 3.50 | PS | 0.008 | 6.75 | 3.3 | 1563.0 | 0.67 | hydrolift | 2 |
|  | X24-96D | 2014-09-09 | 0.96 | 3.765 | 28.284 | 48.5 | 50.00 | 12:35 | 13:15 | 0:40 | 1.25 | PS | 16.105 | 6.12 | 3.4 | 1999.0 | 10.00 | hydrolift | 2 |
|  | X25-96A | 2014-09-09 | 0.63 | 3.184 | 9.462 | 12.5 | 13.00 | 14:50 | 15:20 | 0:30 | 0.43 | PS | -0.009 | 6.80 | 3.9 | 1765.0 | 1.13 | peristaltic | 2 |
|  | X25-96B | 2014-09-09 | 0.61 | 3.056 | 19.705 | 33.5 | 11.00 | 15:30 | 15:50 | 0:20 | 0.55 | PS | -0.014 | 7.42 | 3,7 | 1022.0 | 1.17 | peristaltic | 2 |
| Intermediate Dump | P96-6 | 2014-09-12 | 0.25 | 11.898 | 18.340 | 12.9 | 30.00 | 17:48 | 18:15 | 0:27 | 1.11 | PS | 0.102 | 6.45 | 1.2 | 2067.0 | 3.10 | hydrolift | 2 |
| Main Dump | SRK08-P9 | 2014-09-11 | 0.10 | 4.419 | 6.170 | 3.5 | 5.00 | 15:04 | 15:34 | 0:30 | 0.17 | PS | 0.411 | 7.16 | 3.7 | 2323.0 | 24.00 | peristaltic | 2 |
| Mill Area | SRK08-10A | 2014-09-10 | 0.73 | 10.344 | 13.762 | 6.9 | 21.00 | 15:15 | 15:50 | 0:35 | 0.60 | 3WV | 1.054 | 6.39 | 3.0 | 2265.0 | 35.10 | waterra | 2 |
|  | SRK08-11A | 2014-09-10 | 0.69 | 0.765 | 12.467 | 23.5 | 6.00 | 16:50 | 17:10 | 0:20 | 0.30 | PS | 0.067 | 6.97 | 3.4 | 749.0 | 1.52 | peristaltic | 2 |
|  | SRK08-11B | 2014-09-10 | 0.94 | 1.110 | 6.735 | 11.3 | 6.00 | 16:20 | 16:40 | 0:20 | 0.30 | PS | 0.026 | 6.54 | 4.4 | 1140.0 | 1.35 | peristaltic | 2 |
| NFRC Rock Drain Pond | MW14-12D | 2014-09-13 | 0.93 | 1.902 | 6.350 | 8.9 | 10.00 | 11:16 | 11:52 | 0:36 | 0.28 | PS | -0.002 | 5.82 | 2.9 | 780.0 | 1.53 | peristaltic | 2 |
|  | MW14-12S | 2014-09-13 | 1.00 | 2.004 | 3.920 | 3.8 | 7.50 | 10:27 | 11:12 | 0:45 | 0.17 | PS | 0.836 | 6.12 | 5.6 | 621.0 | 22.50 | peristaltic | 2 |
|  | MW14-13 | 2014-09-13 | 0.99 | 3.365 | 5.005 | 3.3 | 6.50 | 9:48 | 10:09 | 0:21 | 0.31 | PS | 0.005 | 5.67 | 2.3 | 1612.0 | 1.51 | peristaltic | 2 |
|  | MW14-14 | 2014-09-16 | 0.89 | 1.531 | 3.500 | 1.0 | 1.00 | 12:22 | 12:29 | 0:07 | 0.14 | 3WV | 1.969 | 6.62 | 6.7 | 2091.0 | 19.10 | waterra | 1.5 |
|  | MW14-15 | 2014-09-14 | 1.15 | 2.032 | 3.295 | 0.4 | 3.00 | 12:00 | 13:56 | 1:56 | 0.03 | 3WV | 0.775 | 6.71 | 9.4 | 699.0 | 71.00 | peristaltic | 1 |
|  | MW14-16 | 2014-09-13 | 0.98 | 4.985 | 6.900 | 3.8 | 5.00 | 9:11 | 9:32 | 0:21 | 0.24 | PS | -0.005 | 5.92 | 7.3 | 1510.0 | 2.14 | peristaltic | 2 |
| Northeast Dumps | BH13B | 2014-09-12 | 0.75 | 2.690 | 4.400 | 3.4 | 6.50 | 14:33 | 15:04 | 0:31 | 0.21 | PS | 0.015 | 6.58 | 1.6 | 1150.0 | 2.06 | peristaltic | 2 |
|  | BH14A | 2014-09-12 | 0.06 | 3.420 | 6.441 | 6.0 | 6.50 | 15:28 | 16:00 | 0:32 | 0.20 | PS | 0.445 | 6.57 | 4.5 | 3950.0 | 8.19 | peristaltic | 2 |
|  | BH14B | 2014-09-12 | 0.64 | 4.090 | 10.060 | 11.9 | 7.50 | 16:11 | 16:52 | 0:41 | 0.18 | PS | 2.920 | 6.74 | 3.5 | 3744.0 | 12.50 | peristaltic | 2 |
| Second Impoundment | P03-01-2 | 2014-09-16 | NR | 5.800 | 39.325 | 4.2 | 6.00 | 11:10 | 12:01 | 0:51 | 0.12 | PS | NR* | 6.94 | 5.0 | 416.0 | 2.65 | peristaltic | 0.5 |
|  | P03-01-8 | 2014-09-16 | NR | 5.723 | 10.060 | 0.5 | 3.00 | 10:22 | 10:54 | 0:32 | 0.09 | 3WV | NR* | 5.92 | 6.0 | >4000 | 6.94 | peristaltic | 0.5 |
|  | P03-03-2 | 2014-09-15 | 0.67 | 7.089 | 34.292 | 3.4 | 9.00 | 13:51 | 14:17 | 0:25 | 0.36 | PS | NR* | 4.80 | 5.6 | 218.4 | 1.48 | peristaltic | 0.5 |
|  | P03-03-4 | 2014-09-15 | 0.83 | 8.650 | 24.545 | 2.0 | 6.50 | 11:18 | 11:39 | 0:21 | 0.31 | PS | NR* | 6.20 | 5.2 | 1155.0 | 4.72 | micro waterra | 0.5 |
|  | P03-03-9 | 2014-09-15 | 0.83 | 6.870 | 18.179 | 0.4 | 1.50 | 12:49 | 13:04 | 0:05 | 0.30 | 3WV | NR* | 5.68 | 8.8 | 336.4 | 613 AU | peristaltic | 0.5 |
|  | P03-05-4 | 2014-09-15 | 0.73 | 7.130 | 23.425 | 2.1 | 6.00 | 13:25 | 13:43 | 0:18 | 0.33 | 3WV | NR* | 5.50 | 5.2 | 202.7 | 2.32 | peristaltic | 0.5 |
|  | P03-06-1 | 2014-09-15 | 0.85 | 14.920 | 26.581 | 1.5 | 6.00 | 10:08 | 10:30 | 0:22 | 0.27 | 3WV | NR* | 4.68 | 8.2 | 3186.0 | 9.17 | micro waterra | 0.5 |
|  | P03-06-2 | 2014-09-15 | 0.88 | 12.260 | 23.699 | 1.4 | 6.00 | 9:35 | 9:47 | 0:12 | 0.50 | PS | NR* | 4.90 | 5.4 | 3099.0 | 939 AU | micro waterra | 0.5 |
|  | P03-06-6 | 2014-09-15 | 0.94 | 12.349 | 13.627 | 0.2 | 1.50 | 8:45 | 9:13 | 0:28 | 0.05 | 3WV | NR* | NR** | NR** | NR** | out of range3 | micro waterra | 0.5 |
|  | P03-06-7 | 2014-09-15 | 0.95 | dry | 11.816 | - | - | - | - | - | - | - | - | - | - | - | - | - | 0.5 |
| S-Wells Area S-Wells Area | P09-SIS1 | 2014-09-09 | 0.99 | 4.600 | 6.555 | 3.9 | 4.00 | 12:24 | 13:01 | 0:37 | 0.11 | PS | 0.740 | 6.51 | 6.4 | >4000 | 20.00 | peristaltic | 2 |
|  | P09-SIS2 | 2014-09-09 | 1.16 | 3.920 | 6.325 | 4.8 | 5.00 | 14:06 | 14:37 | 0:31 | 0.16 | PS | 0.040 | 5.60 | 6.6 | 1214.0 | 2.15 | peristaltic | 2 |
|  | P09-SIS3 | 2014-09-09 | 1.04 | 3.910 | 4.605 | 1.4 | 3.50 | 14:55 | 15:16 | 0:21 | 0.17 | PS | 0.050 | 7.05 | 5.9 | 1323.0 | 0.95 | peristaltic | 2 |
|  | P09-SIS4 | 2014-09-09 | 0.95 | 4.040 | 4.440 | 0.8 | 1.50 | 15:34 | 15:47 | 0:13 | 0.12 | PS | 0.300 | 7.05 | 6.5 | >4000 | 12.00 | peristaltic | 2 |
|  | P09-SIS5 | 2014-09-09 | 1.09 | 3.596 | 4.550 | 1.9 | 2.00 | 16:04 | 16:22 | 0:18 | 0.11 | PS | 0.624 | 7.05 | 6.9 | >4000 | 17.40 | peristaltic | 2 |
|  | P96-7 | 2014-09-10 | 0.85 | 5.138 | 9.895 | 9.5 | 9.00 | 8:03 | 8:42 | 0:39 | 0.04 | PS | 0.112 | 7.12 | 1.4 | 3102.0 | 1.75 | peristaltic | 2 |
|  | S1A | 2014-09-10 | 1.34 | 4.793 | 12.890 | 16.4 | 22.00 | 14:00 | 14:30 | 0:30 | 0.73 | PS | NR | 5.80 | 1.8 | 3005.0 | 61.40 | waterra | 2 |
|  | S1B | 2014-09-11 | 1.17 | 4.530 | 5.175 | 1.5 | 2.50 | 12:50 | 18:00 | 5:10 | -5 | 3WV | Dry | 6.47 | 4.1 | 809.0 | 1.93 | peristaltic | 2 |
|  | S2A | 2014-09-10 | 1.22 | 5.232 | 12.670 | 15.0 | 12.00 | 11:36 | 12:27 | 0:31 | 0.39 | PS | 0.018 | 5.96 | 2.6 | 2125.0 | 18.50 | peristaltic | 2 |
|  | S2B | 2014-09-10 | 0.54 | 4.457 | 7.065 | 5.2 | 7.00 | 10:52 | 11:28 | 0:36 | 0.19 | PS | 1.043 | 5.85 | 2.5 | >4000 | 20.30 | peristaltic | 2 |
|  | SRK05-SP-1A | 2014-09-10 | 0.75 | 7.235 | 20.170 | 25.9 | 35.00 | 15:37 | 16:21 | 0:44 | 0.80 | PS | 0.025 | 5.64 | 1.1 | 2320.0 | 4.18 | hydrolift | 2 |
|  | SRK05-SP-1B | 2014-09-11 | 0.52 | 7.115 | 13.110 | 12.0 | 7.00 | 11:41 | 12:18 | 0:37 | 0.19 | PS | 0.005 | 6.05 | 1.9 | 1067.0 | 4.03 | peristaltic | 2 |
|  | SRK05-SP-3A | 2014-09-10 | 0.86 | 4.896 | 23.810 | 37.8 | 58.00 | 15:13 | 15:34 | 0:21 | 2.76 | PS | 0.364 | 6.03 | 1.3 | 1560.0 | 3.88 | hydrolift | 2 |
|  | SRK05-SP-3B | 2014-09-10 | 0.90 | 4.064 | 13.305 | 18.5 | 10.00 | 14:36 | 15:06 | 0:30 | 0.33 | PS | 0.036 | 5.68 | 1.8 | 1145.0 | 6.58 | peristaltic | 2 |
|  | SRK05-SP-4A | 2014-09-09 | 0.59 | 4.522 | 22.400 | 7.878 | 35.00 | 17:53 | 18:20 | 0:27 | 1.30 | PS | -6 | 5.98 | 2.0 | 1392.0 | 4.00 | hydrolift | 2 |
|  | SRK05-SP-4B | 2014-09-09 | 0.84 | 4.025 | 4.765 | 1.5 | 3.00 | 17:15 | 17:39 | 0:24 | 0.13 | PS | -6 | 5.97 | 4.9 | >4000 | 5.54 | peristaltic | 2 |
|  | SRK05-SP-5 | 2014-09-09 | 0.99 | 6.136 | 14.730 | 17.2 | 55.00 | 13:01 | 13:35 | 0:34 | 1.62 | 3WV | 1.874 | 5.88 | 2.8 | 151 | 8.54 | hydrolift | 2 |
|  | SRK05-SP-6 | 2014-09-11 | - | Dry | 11.780 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | SRK08-SBR1 | 2014-09-11 | - | Broken | Broken | - | - | - | - | - | - | - | - | - | - | - | - | - | 2 |
|  | SRK08-SBR2 | 2014-09-11 | 0.54 | 6.577 | 0.540 | 24.0 | 52.00 | 14:15 | 14:34 | 0:19 | 2.74 | PS | NR | 7.06 | 2.3 | 3475.0 | 43.00 | waterra | 2 |
|  | SRK08-SBR3 | 2014-09-11 | 0.96 | 11.631 | 13.173 | 3.1 | 9.00 | 13:02 | 13:17 | 0:15 | 0.60 | 3WV | 0.009 | 6.90 | 2.1 | 3362.0 | 16.60 | waterra | 2 |
|  | SRK08-SBR4 | 2014-09-10 | 0.80 | 7.621 | 21.500 | 27.6 | 39.00 | 12:52 | 13:21 | 0:29 | 1.34 | PS | NR | 5.88 | 1.6 | >4000 | 2.40 | hydrolift | 2 |
|  | SRK08-SP-7A | 2014-09-09 | 1.00 | 2.635 | 17.750 | 30.2 | 45.00 | 11:30 | 11:52 | 0:22 | 2.05 | PS | 0.475 | 5.95 | 1.9 | 830.0 | 2.80 | hydrolift | 2 |
|  | SRK08-SP-7B | 2014-09-09 | 1.06 | 2.718 | 8.690 | 11.9 | 10.50 | 10:30 | 11:10 | 0:40 | 0.26 | PS | 0.004 | 5.76 | 2.8 | 273.0 | 1.99 | peristalic | 2 |
|  | SRK08-SP-8A | 2014-09-10 | 1.20 | 1.800 | 8.520 | 13.4 | 10.50 | 9.26 | 9.52 | 0:26 | 0.40 | PS | 0.020 | 5.98 | 0.9 | 2781.0 | 1.07 | peristaltic | 2 |
|  | SRK08-SP-8B | 2014-09-10 | 1.04 | 1.900 | 7.030 | 10.3 | 9.50 | 9:58 | 10:23 | 0:25 | 0.38 | PS | 0.010 | 6.04 | 1.0 | 2709.0 | 0.98 | peristaltic | 2 |
| NFRC Near S-Wells Area | MW14-08 | 2014-09-10 | 0.96 | 1.687 | 3.270 | 3.2 | 5.50 | 16:38 | 17:00 | 0:38 | 0.14 | PS | 0.253 | 6.70 | 6.6 | 505.0 | 5.00 | peristaltic | 2 |
|  | MW14-09 | 2014-09-11 | 0.83 | 2.678 | 6.710 | 8.1 | 9.50 | 9:02 | 9:35 | 0:33 | 0.29 | PS | 1.547 | 6.31 | 1.5 | 757.0 | 11.40 | peristalitic | 2 |
|  | MW14-10 | 2014-09-11 | 0.94 | 3.124 | 5.540 | 4.8 | 7.50 | 9:47 | 10:12 | 0:19 | 0.39 | PS | 0.456 | 7.22 | 3.7 | 1035.0 | 23.00 | peristaltic | 2 |
|  | MW14-11 | 2014-09-10 | 0.79 | 1.845 | 4.455 | 5.4 | 5.00 | 17:16 | 17:40 | 0:24 | 0.21 | PS | 0.735 | 6.96 | 2.8 | 670.0 | 9.34 | peristaltic | 2 |
|  | SRK05-SP-2 | 2014-09-11 | 0.82 | 2.276 | 11.820 | 19.1 | 10.00 | 8:25 | 8:52 | 0:27 | 0.37 | PS | 0.014 | 5.78 | 1.5 | 511.0 | 2.34 | peristaltic | 2 |
| Upstream of Tailings | TH86-2 | 2014-09-16 | 0.43 | 1.755 | 11.410 | 19.3 | 14.00 | 14:05 | 14:50 | 0:45 | 0.31 | PS | 0.015 | 7.01 | 5.4 | 245.0 | 9.40 | peristaltic | 2 |
|  | TH86-5 | 2014-09-16 | 1.22 | 8.747 | 27.500 | 37.5 | 130.00 | 12:59 | 13:47 | 0:48 | 2.71 | 3WV | 0.073 | 6.54 | 3.6 | 855.0 | 8.79 | hydrolift | 2 |
| Vangorda/ Grum | BH05-9B-R | 2014-09-12 | 0.93 | 1.011 | 19.852 | 37.7 | 2.30 | 14:03 | 14:23 | 0:20 | 0.12 | PS | 0.488 | 7.75 | 6.8 | 160.0 | 8.16 | peristaltic | 2 |
|  | P09-GS1A | 2014-09-14 | 1.24 | 2.339 | 7.373 | 10.0 | 18.00 | 8:38 | 9:08 | 0:30 | 0.60 | PS | 0.026 | 6.76 | 8.1 | 850.0 | 1.35 | peristaltic | 2 |
|  | P09-GS1B | 2014-09-14 | 0.92 | 2.170 | 29.710 | 55.0 | 18.00 | 9:30 | 10:20 | 0:50 | 0.36 | PS | 2.310 | 6.76 | 6.1 | 129.5 | 3.24 | peristaltic | 2 |
|  | P09-LCD1 | 2014-09-13 | 0.95 | 3.741 | 7.350 | 7.2 | 7.70 | 9:00 | 9:25 | 0:25 | 0.31 | PS | 0.079 | 7.17 | 3.1 | 563.0 | 1.93 | peristaltic | 2 |
|  | P09-LCD4 | 2014-09-14 | 0.95 | 7.417 | 12.210 | 9.6 | 9.35 | 9:50 | 10:00 | 0:10 | 0.94 | 3WV | 4.800 | 7.23 | 3.7 | 531.0 | 45.40 | peristaltic | 2 |
|  | P09-LCD6 | 2014-09-13 | 0.79 | 5.769 | 7.930 | 4.5 | 4.75 | 10:13 | 10:31 | 0:18 | 0.26 | PS | 0.076 | 7.18 | 3.4 | 617.0 | 5.96 | peristaltic | 2 |
|  | P09-VC1 | 2014-09-12 | 0.99 | 3.786 | 68.635 | 130.0 | 27.50 | 10:05 | 10:35 | 0:30 | 0.92 | PS | 1.799 | 8.04 | 3.9 | 213.0 | 5.22 | hydrolift | 2 |
|  | P09-VC2 | 2014-09-12 | 0.94 | 1.555 | 19.870 | 37.0 | 25.00 | 8:45 | 9:25 | 0:40 | 0.63 | PS | 0.350 | 7.39 | 3.9 | 241.9 | 1.82 | hydrolift | 2 |
|  | P2001-02A | 2014-09-13 | 0.53 | 3.997 | 6.595 | 5.3 | 1.70 | 12:37 | 13:07 | 0:30 | 0.06 | 3WV | 2.600 | 6.79 | 6.8 | 211.5 | 13.90 | peristaltic | 2 |
|  | P2001-02B | 2014-09-11 | 0.37 | 3.830 | 27.670 | 47.8 | 19.50 | 13:40 | 14:20 | 0:40 | 0.49 | PS | 4.670 | 6.95 | 5.5 | 1918.0 | 10.58 | hydrolift | 2 |
|  | P2001-3 | 2014-09-11 | 0.70 | 37.004 | 62.42 | ~50 | 120.00 | 11:15 | 12:05 | 0:50 | 2.40 | PS | NR | 7.44 | 2.8 | 563.0 | 47.10 | hydrolift | 2 |
|  | P96-9A | 2014-09-12 | 0.92 | 5.808 | 9.393 | 7.2 | 3.00 | 13:28 | 13:48 | 0:20 | 0.15 | PS | 0.057 | 6.62 | 6.2 | 238.1 | 1.71 | peristaltic | 2 |
|  | SRK05-07 | 2014-09-12 | 0.69 | 5.730 | 6.445 | 1.4 | 1.45 | 11:25 | 11:44 | 0:19 | 0.08 | PS | 0.042 | 6.82 | 5.0 | 2113.0 | 2.69 | peristaltic | 2 |
|  | SRK05-08 | 2014-09-12 | 0.74 | 5.876 | 8.489 | 5.2 | 2.90 | 12:19 | 12:39 | 0:20 | 0.15 | PS | 0.175 | 6.86 | 5.4 | 1720.0 | 1.49 | peristaltic | 2 |
|  | SRK05-5C | 2014-09-12 | 0.96 | 1.648 | 3.716 | 4.1 | 4.00 | 14:58 | 15:28 | 0:30 | 0.13 | PS | 0.140 | 7.55 | 6.2 | 410.5 | 7.15 | peristaltic | 1.5 |
|  | SRK05-9 | 2014-09-13 | 0.50 | 2.736 | 3.954 | 1.4 | 6.00 | 17:10 | 17:42 | 0:32 | 0.19 | 3WV | 1.218 | 7.35 | 6.0 | 1456.0 | 19.90 | micro-waterra | 1.5 |
|  | SRK08-P14 | 2014-09-14 | 0.73 | 6.710 | 9.875 | 6.4 | 6.25 | 14:36 | 15:00 | 0:24 | 0.26 | PS | 0.046 | 6.85 | 3.3 | 187.4 | 4.62 | peristaltic | 2 |
|  | SRK08-P15 | 2014-09-14 | 0.37 | 3.178 | 8.275 | 10.2 | 10.00 | 13:08 | 13:52 | 0:44 | 0.23 | PS | 0.045 | 6.97 | 2.8 | 122.7 | 6.40 | peristaltic | 2 |
|  | SRK08-P16 | 2014-10-15 | 0.73 | 7.297 | 7.316 | 0.38 (Dry) | - | - | - | - | - | - | - | - | - | - | - | peristaltic | 2 |
|  | V34 | 2014-09-11 | 0.55 | 5.713 | 12.307 | 13.3 | 2.75 | 8:42 | 9:09 | 0:27 | 0.10 | PS | 0.837 | 6.83 | 3.3 | 108.1 | 12.80 | peristaltic | 2 |
|  | V35 | 2014-09-11 | 0.45 | 6.362 | 16.010 | 19.0 | 1.75 | 10:00 | 10:25 | 0:25 | 0.07 | PS | 0.974 | 6.93 | 3.6 | 1645.0 | 2.10 | peristaltic | 2 |
|  | V36 | 2014-09-11 | 0.995 | 8.706 | 11.855 | 6.3 | 8 | 15:20 | 15:33 | 0.13 | 0.62 | PS | 0.28 | 6.94 | 4.1 | 3192.0 | 6.27 | waterra | 2 |
|  | V37 | 2014-09-13 | 0.49 | 8.500 | 14.497 | 12.0 | 35.00 | 16:07 | 16:33 | 0:26 | 1.35 | 3WV | 5.997 | 7.36 | 3.2 | 735.0 | 22.00 | waterra | 2 |
| Zone 2 Outwash/Pit | BH10A | 2014-09-12 | 1.65 | 6.080 | 6.920 | 1.7 | 6.00 | 10:08 | 10:49 | 0:41 | 0.15 | 3WV | 0.030 | 6.24 | 3.2 | 451.0 | 3.65 | peristaltic | 2 |
|  | BH10B | 2014-09-12 | 0.87 | 5.311 | 9.190 | 7.8 | 9.00 | 9:16 | 9:59 | 0:43 | 0.21 | PS | 0.029 | 6.15 | 2.1 | 404.0 | 28.90 | peristaltic | 2 |
|  | BH5 | 2014-09-12 | 0.69 | 2.199 | 7.350 | 10.3 | 9.00 | 12:42 | 13:20 | 0:38 | 0.24 | PS | 0.001 | 5.48 | 3.3 | 669.0 | 7.85 | peristaltic | 2 |
|  | BH6 | 2014-09-12 | 0.74 | 3.357 | 6.646 | 6.6 | 5.50 | 7:58 | 8:21 | 0:23 | 0.24 | PS | 0.003 | 5.85 | 2.2 | 522.0 | 5.49 | peristaltic | 2 |
|  | BH8 | 2014-09-13 | 0.79 | 17.119 | 20.780 | 7.4 | 8.00 | 14:03 | 18:31 | 4:28 | -5 | PS | 3.661 | 7.05 | 5.1 | 3088.0 | 31.60 | hydrolift | 2 |
|  | P05-04 | 2014-09-12 | 0.67 | 3.094 | 7.025 | 7.9 | 6.50 | 8:35 | 8:55 | 0:20 | 0.33 | PS | 0.006 | 5.94 | 1.9 | 555.0 | 0.81 | peristaltic | 2 |
|  | SRK08-P12A | 2014-09-12 | 0.72 | 2.105 | 12.735 | 21.3 | 7.50 | 11:46 | 12:18 | 0:32 | 0.23 | PS | 0.035 | 5.60 | 2.3 | 1255.0 | 18.10 | peristaltic | 2 |
|  | SRK08-P12B | 2014-09-12 | 0.73 | 2.091 | 8.490 | 12.3 | 7.50 | 11:08 | 11:40 | 0:32 | 0.23 | PS | 0.001 | 5.61 | 1.8 | 851.0 | 3.98 | peristaltic | 2 |

Notes:

NR = Not recorded in the field, NR* = Not Recorded due to limiting diameter of well casing, NR** = Not Recorded due to turbid conditions.

‘-‘ = Not Applicable.

1 Sample sites where conductivity is measured using a Hanna 991300 has a range of 0.0 to 3999.0 µS/cm.

2 Turbidity values are reported in NTU unless otherwise indicated.

3 Purging criteria for well MW14-04S was not met due to insufficient volumes.

4 Out-of-range indicates turbidity is >4000 NTU

5Due to low volume and slow recharge, wells S1B and BH8 were drawn down over a longer time period, and purge rate could not be calculated.

6 Drawdown/recharge could not be measured at wells SRK05-SP-4A and SRK05-SP-4B due to a broken well casing (damage restricted access to the top of well casing during the purging process).

## Analytical Results

Analytical results, including a brief summary of CCME FAL guideline exceedances and factors which may have influenced data precision, are provided below. In some instances the laboratory detection limits (DL) exceeded applicable CCME FAL standards (values shaded in light grey in Table 3-2). This occurs when samples with high levels of some elements or compounds require dilution in order for the lab to properly analyse the sample. Accordingly, the laboratory detection limit must then be increased. In the case of mercury, low-level analytical methods are required to achieve detection limits relevant to the guidelines. Because mercury was not specified in the SOW, specific sampling for mercury was not included as part of the laboratory program during this sample event, and therefore the low-level analytical methods were not used. For the purpose of this report, samples where the reported DL is higher than the applicable guideline have not been reported as CCME FAL exceedances.

### Cross Valley Dam

Groundwater wells located in the Cross Valley Dam (CVD) area were sampled on September 10, 2014. Samples were obtained from seven (7) wells within this area identified for the sampling event.

Concentrations of dissolved aluminum, arsenic, cadmium, iron and silver in groundwater exceeded the CCME FAL guidelines in all samples collected from the CVD area. Concentrations of total aluminum, arsenic, cadmium, chromium, copper, iron, lead, silver, and zinc in groundwater also exceeded the CCME FAL guidelines in at least one (1) of the seven (7) wells.

Hemmera/ELR noted that P03-09-9, P03-09-6, P05-01-05, P05-01-03, and P09-C2 did not have PVC caps or J-plugs on the wells, which can allow for potential contamination. At two of the sample stations (P05-01 and P03-09), the well diameter was too narrow to allow for continuous monitoring of drawdown/recharge during the purging process. Drawdown measurements are therefore not provided for either of these sample site locations (P05-01 and P03-09). Hemmera/ELR also noted that the stickup of well P09-C2 was damaged and loose, leaning at an angle of ~20 degrees from vertical, and eroding at the concrete base.

Groundwater turbidity of all CVD samples was less than 50 NTU.

### Down Gradient of Cross Valley Dam

Groundwater wells located down gradient of the CVD area were sampled on September 9, 2014. Samples were obtained from both wells (2) within this area identified for the sampling event.

Concentrations of dissolved cadmium and iron in groundwater exceeded the CCME FAL guidelines in samples collected from one (1) well each down gradient of the CVD area. Concentrations of total cadmium and iron in groundwater also exceeded the CCME FAL guidelines in one (1) well.

Hemmera/ELR staff noted that wells located down gradient of the CVD area did not have PVC caps or J-plugs covering the wells, which can allow for potential contamination. Groundwater turbidity in all samples within this area was less than 50 NTU.

### ETA Area

Groundwater wells located in the ETA Area were sampled between September 11 and 13, 2014. Samples were obtained from seven (7) wells in this area identified for the sampling event.

Groundwater pH in the ETA area was less than CCME FAL guidelines in five (5) of the seven (7) samples (ranging from 3.61 to 6.05 based on laboratory pH values). Concentrations of either dissolved aluminum, arsenic, cadmium, copper, iron, lead, nickel, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in at least one sample. Concentrations of total aluminum, arsenic, cadmium, copper, iron, lead, mercury, nickel, silver, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

Hemmera/ELR staff noted that well SRK05-ETA-BR1 had no PVC cap or J-plug covering the well, which can allow for potential contamination. It was also noted that well SRK05-ETA-BR2 had no metal casing and that the concrete base was eroding and in risk of collapse. Groundwater turbidity in all samples collected within the ETA area was less than 50 NTU.

### Haul Road/Near NFRC

Groundwater wells located on the haul road near the NFRC were sampled between September 14 and 17, 2014. Samples were obtained from nine (9) wells in this area identified for the sampling event.

Groundwater pH in the Haul Road area was not in compliance with CCME FAL guidelines in three (3) of the nine (9) samples (ranging from 4.47 to 6.45 based on laboratory pH values). Concentrations of dissolved aluminum, arsenic, cadmium, iron, lead, nickel, thallium, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in the three samples. Concentrations of total aluminum, arsenic, cadmium, chromium copper, iron, lead, nickel, thallium, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

Wells located in the Haul Road area were sampled using a Redi-Flo2 submersible pump and have a DTB ranging from 46 to 69 meters. Due to energy release by the pump, in-situ groundwater temperatures may have been altered during both the purging and sampling process. Purging criteria for well MW14-04S were not met due to insufficient volumes. Groundwater turbidity in all samples collected within the ETA area was less than 50 NTU.

### Intermediate Dam

Groundwater wells located within the intermediate dam area were sampled on September 9, 2014. Samples were obtained from six (6) wells within this area identified for the sampling event.

Concentrations of dissolved aluminum, cadmium, iron, nickel, silver, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected within the intermediate dam area. Concentrations of total aluminum, cadmium, copper, iron, nickel, silver, and zinc in groundwater also exceeded the CCME FAL guidelines.

Hemmera/ELR staff noted that well P01-04B had no PVC cap or J-plug covering the well, which can allow for potential contamination. No additional concerns were identified in the field that may affect data quality. Groundwater turbidity in all samples collected within the intermediate dam area was less than 50 NTU.

### Intermediate Dump

The one (1) groundwater well located within the intermediate dump area included in the fall sampling event (well P96-6) was sampled on September 12, 2014.

Concentrations of dissolved selenium, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples from this well. Concentrations of total aluminum, selenium, uranium, and zinc in groundwater also exceeded the CCME FAL standards.

No additional concerns were identified in the field that may affect data quality. The groundwater turbidity of samples collected from station P96-6 was less than 50 NTU.

### Main Dump

One (1) groundwater well located within the main dump area was included in the fall sampling event (SRK08-P9), which was sampled on September 11, 2014.

Concentrations of dissolved iron in groundwater exceeded the CCME FAL guidelines in samples from this well. Concentrations of total aluminum, copper, and iron in groundwater also exceeded the CCME FAL standards.

No additional concerns were identified in the field that may affect data quality. Groundwater turbidity in all samples collected within the intermediate dam area was less than 50 NTU.

### Mill Area

Groundwater wells located in the mill area were sampled on September 10, 2014. Samples were obtained from three (3) wells in this area identified for the sampling event.

Concentrations of dissolved aluminum, cadmium, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected within the mill area. Concentrations of total aluminum, cadmium, chromium, copper, iron, lead, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

No additional concerns were identified in the field that may affect data quality. Groundwater turbidity in all samples collected within the intermediate dam area was less than 50 NTU.

### NFRC Rock Drain Pond

Groundwater wells located in the NFRC Rock Drain Pond Area were sampled between September 13 and 16, 2014. Samples were obtained from six (6) wells in this area identified for the sampling event.

Groundwater pH in the NFRC Rock Drain Pond Area was not in compliance with CCME FAL guidelines in one (1) of the six (6) samples (6.48, based on laboratory pH values). Concentrations of dissolved aluminum, cadmium, selenium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected within the NFRC Rock Drain Pond Area. Concentrations of total aluminum, cadmium, chromium, copper, iron, selenium, and zinc in groundwater also exceeded the CCME FAL guidelines.

Samples collected from well MW14-15 were very turbid (71 NTU) due to low purge volumes and rate of recharge. No additional concerns were identified in the field that may affect data quality.

### Northeast Dumps

Groundwater wells located in the northeast dumps area were sampled on September 12, 2014. Samples were obtained from three (3) wells in this area identified for the sampling event.

Concentrations of dissolved cadmium, copper, lead, nickel, selenium, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected within the northeast dumps area. Concentrations of total aluminum, cadmium, copper, iron, lead, mercury, nickel, selenium, silver, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

No additional concerns were identified in the field that may affect data quality. Groundwater turbidity of all samples within the northeast dumps area was less than 50 NTU.

### Second Impoundment

Groundwater wells located in the Second Impoundment Area were sampled between September 15 and 16, 2014. Samples were obtained from nine (9) of the ten (10) wells in this area identified for the sampling event. Well P03-06-07 was dry during the time of sampling.

Groundwater pH in the Second Impoundment Area was not in compliance with CCME FAL guidelines in seven (7) of the ten (9) samples (ranging from 3.16 to 5.14, based on laboratory pH values). Concentrations of dissolved aluminum, arsenic, cadmium, copper, iron, lead, nickel, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected from the Second Impoundment Area. Concentrations of total aluminum, arsenic, cadmium, chromium, copper, iron, lead, mercury, nickel, silver, thallium, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

Hemmera/ELR staff could not record drawdown at stations P03-03-2, P03-03-4, P03-03-9, P03-06-1, P03-06-2, and P03-06-6 due well diameter. Field parameters could not be measured at P03-06-6 due to low purge volumes.

Samples collected from wells P03-03-9, P03-06-2, and P03-06-6 were extremely turbid (613 AU, 939 AU, and “out-of-range”; >4000 NTU), likely due to the placement of the wells in the tailings. Groundwater turbidity of all other collected samples within the Second Impoundment Area was less than 50 NTU.

### S-Wells Area

Groundwater wells located in the S-Wells area were sampled between September 9 and 11, 2014. Samples were obtained from twenty four (24) of the twenty six (26) wells in this area identified for the sampling event. Well SRK05-SP-6 was found dry and well SRK08-SBR1 was found broken during the time of sampling.

Groundwater pH in the S-Wells area was not in compliance with CCME FAL guidelines in three (3) of the twenty four (24) samples (ranging from 6.02 to 6.48, based on laboratory pH values). Concentrations of dissolved aluminum, arsenic, cadmium, chromium, copper, iron, lead, nickel, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected from the S-Wells area. Concentrations of total aluminum, arsenic, cadmium, copper, iron, lead, mercury, nickel, silver, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

Hemmera/ELR noted that wells SRK05-SP-4B, SRK08-SBR1, and SRK08-SBR2 were either damaged or broken. Drawdown/recharge could not be measured at well SRK05-SP-4B due to a broken well casing (damage restricted access to the top of well casing during the purging process), SRK08-SBR1 had broken PVC at the surface and was plugged with bentonite, and the PVC at site SRK08-SBR2 was reported damaged. The casing of well SRK05-SP-5 was repaired by Hemmera/ELR in the field, and this well was redeveloped before the purging and sampling process.

Hemmera/ELR noted that SRK08-SBR4, SRK08-SP-7A, and SRK08-SP-7B had no PVC caps or J-plugs covering the wells, which can allow for potential contamination. Samples collected from station S1A were turbid (61.4 NTU) due to low purge volumes and rate of recharge. No additional concerns were identified in the field that may affect data quality.

### NFRC near S-Wells Area

Groundwater wells located at the NFRC near the S-Wells area were sampled between September 10 and 11, 2014. Samples were obtained from five (5) of the wells in this area identified for the sampling event.

Concentrations of dissolved aluminum, arsenic, iron, selenium, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected from the S-Wells area near NFRC. Concentrations of total aluminum, arsenic, chromium, copper, iron, lead, selenium, silver, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

No additional concerns were identified in the field that may affect data quality. Groundwater turbidity of all samples within the northeast dumps area was less than 50 NTU.

### Upstream of Tailings

Groundwater wells located Upstream of the Tailing were sampled on September 16, 2014. Samples were obtained from both of the two (2) wells in this area identified for the sampling event.

Concentrations of dissolved arsenic and iron in groundwater exceeded the CCME FAL guidelines in samples collected from Upstream of the Tailings. Concentrations of total aluminum, arsenic, chromium, copper, iron, molybdenum, and silver in groundwater also exceeded the CCME FAL guidelines.

No additional concerns were identified in the field that may affect data quality. Groundwater turbidity of all samples within the northeast dumps area was less than 50 NTU.

### Groundwater Vangorda/Grum

Groundwater wells located in the Vangorda/Grum area were sampled between September 11 and September 15, and on October 15, 2014. Samples were obtained from thirty (30) of the thirty one (31) wells in this area identified for the sampling event. Well SRK08-P14 was found to have insufficient volume (38 mL of groundwater) at the time of sampling.

Groundwater pH in the Vangorda/Grum area was not in compliance with CCME FAL guidelines in two (2) of the thirty (30) samples (5.13 and 6.31, based on laboratory pH values).Concentrations of dissolved aluminum, arsenic, cadmium, copper, iron, lead, nickel, selenium, thallium, uranium, and zinc in groundwater exceeded the CCME FAL guidelines in samples collected from the Vangorda/Grum area. Concentrations of total aluminum, arsenic, cadmium, chromium, copper, iron, lead, mercury, nickel, selenium, silver, thallium, uranium, and zinc in groundwater also exceeded the CCME FAL guidelines.

Hemmera/ELR staff noted that wells BH05-9B-R, P09-VC1, SRK05-08, SRK08-P15, V34, V35, V36 and V37 had no PVC caps or J-plugs covering the wells, which can allow for potential contamination. The PVC of P09-GS1A was also noted to be above the metal protective casing which did not allow for a proper seal.

No additional concerns were identified in the field that may affect data quality. Groundwater turbidity of all samples within the northeast dumps area was less than 50 NTU.

## Quality Assurance and Quality Control Results

Twelve (12) duplicate groundwater samples were collected during the fall sampling event. Two (2) travel blanks were provided by the laboratory and accompanied the samples through the sampling program. Two (2) field blanks were prepared during the sampling program on September 9 and September 11, 2014. The detailed results of the QA/QC sampling program are provided in Table 3-3, including RPD values for all duplicate and sample pairs collected.

### Field and Travel Blanks

Field blank and travel blank analytical results were reported less than detection limits for all analysed parameters, indicating that there was no evidence of contamination during the sampling, transportation, or laboratory analysis process.

### Field Duplicates

#### P01-01B / DUP1

The RPD value for dissolved arsenic (21.6%), and total and dissolved lithium (21.6% and 21.1%, respectively) between P01-01B and DUP1 slightly exceeded the desired limit. The RPD values for other parameters within this duplicate/sample pair were less than 20%, therefore, RPD exceedances were not considered representative of a systemic bias.

#### SRK08-SP-7B / DUP2

The RPD values for all corresponding pairs of results between Site SRK08-SP-7B and DUP2 were less than the 20% QA/QC threshold, indicating that sampling error or laboratory error did not occur.

#### P01-11 / DUP3

The RDP value for dissolved potassium (25.2%) between P01-11 and DUP3 slightly exceeded the desired limit. The RPDs for other parameters within this duplicate/sample pair were less than 20%. Slight groundwater sample variation is considered to be the likely cause of the single variable result for potassium, and therefor is not considered representative of a systematic bias.

#### P96-7 / DUP4

The RDP value for dissolved potassium (26.1%) between P96-7 and DUP4 slightly exceeded the desired limit. The RPDs for other parameters within this duplicate/sample pair were less than 20%. Slight groundwater sample variation is considered to be the likely cause of the single variable result for potassium and therefor is not considered representative of a systematic bias.

#### V34 / DUP5

The RDP value for dissolved zinc (34.1%) between V34 and DUP5 exceeded the desired limit, suggesting a potential error affecting data precision. Although greater than five times the detection limit, both dissolved zinc values were relatively low (0.0051 and 0.0072 mg/L), suggesting that a subtle groundwater sample variation could be the cause of the single variable result for zinc.

#### SRK05-SP-1B / DUP6

The RDP value for total suspended solids between SRK05-SP-1B and DUP6 and slightly exceeded the desired limit (23.2%). As the sediment retrieved from groundwater wells during sampling can be variable, this level of variation between a sample pair is not considered to indicate a sampling or laboratory error. Additionally, as the variation between all dissolved and total metals were within an acceptable range, this variation is not considered to have affected sample integrity.

#### SRK05-ETA-BR2 / DUP7

The RPD value for total suspended solids (31.4%), dissolved cadmium (79.7%), dissolved phosphorus (20.7%), and total cadmium (76.7%) between SRK05-ETA-BR2 and DUP7 all exceeded the desired limit. Differences in TSS concentrations between the sample and duplicate pair (56 mg/l and 40.8 mg/l) show variation between total particulates within these samples, and suggest that there was some variation in the groundwater between samples, although field data suggests that purging was completed according to program specifications. The observed variation in TSS may provide some explanation for variation between dissolved and total metals results. Increase dissolved metals concentration would not necessarily be an expected result of increased turbidity.

#### BH6 / DUP8

The RPD values for all corresponding pairs of results between Site BH6 and DUP8 were less than the 20% QA/QC threshold, indicating that sampling error or laboratory error did not occur

#### SRK08-P14 / DUP9

The RPD value for total copper between SRK08-P14 and DUP9 exceeded the desired limit (30.5%). Slight groundwater sample variation is considered to be the likely cause of the single variable result for total copper.

#### MW14-13 / DUP10

The RPD values for all corresponding pairs of results between Site MW14-13 and DUP10 were less than the 20% QA/QC threshold, indicating that sampling error or laboratory error did not occur.

#### MW14-05 / DUP11

The RPD values for all corresponding pairs of results between Site MW14-05 and DUP11 were less than the 20% QA/QC threshold, indicating that sampling error or laboratory error did not occur.

#### MW13-03 / DUP12

The RDP value for laboratory conductivity between MW14-03 and DUP12 show an exceeded the desired limit (85.0%). The cause of such a variation in a laboratory general parameter is not known, but may represent an error in laboratory practices or equipment, as all other parameters tested fell within the 20% RPD limit.

# Recommendations

Hemmera/ELR prepared the following recommendations based on the observations and results of the fall 2014 groundwater sampling program.

- Groundwater wells should be properly sealed with PVC caps or J-plugs. Wells without caps have risk of becoming contaminated from external sources which may affect data precision or quality. Wells without caps include; P03-09-9, P03-09-6, P05-01-05, P05-01-03, P09-C2, P01-01A, P01-01B, SRK05-ETA-BR1, P01-04B, SRK08-SBR4, SRK08-SP-7A, SRK08-SP-7B, BH05-9B-R, P09-VC1, SRK05-08, SRK08-P15, V34, V35, V36 and V37.

- Damaged or degraded wells should be repaired. This includes wells where the stick-up height is above the height of the well casing. Wells of this type which are unable to close properly are at risk of external source contamination. Damaged or degraded wells include the following; SRK05-ETA-BR2, P09-C2, SRK05-SP-4B, SRK08-SBR1, SRK08-SBR2, SRK05-SP-4B, and P09-GS1A.

- Low-flow sampling techniques were used to collect samples at wells that had likely been sampled previously by another method (as noted in section 2.2, pre-existing tubing found within wells indicated a prior sampling method by Waterra inertial footvalve).  To avoid creation of turbulent conditions, inclusion of particles not normally mobile in groundwater, and a positive bias to unfiltered results, it is recommended that low-flow sampling be used for wells where a significant drawdown would occur by the use of other methods.

- To avoid inclusion of acid or alkaline-generating solids that are not representative of an equilibrium condition with groundwater, it is recommended that samples for analysis of acidity, alkalinity, and hardness be field-filtered.

- To avoid degassing of carbon dioxide, precipitation of calcium carbonate in sample bottles, and exclusion of the representative precipitate component from analysis, it is recommended that samples for analysis of alkalinity be collected in a separate bottle with zero headspace and that the laboratory be instructed to analyze the contents of the entire bottle.

- In order to examine chromium exceedances of CCME-FAL guidelines it is recommended to analyse for trivalent and hexavalent forms of chromium.  CCME does not provide groundwater quality guidelines for total chromium but does offer a guideline for Cr(VI), 0.001 mg/l, and an interim guideline for Cr(III), 0.0089 mg/l.

# Closure

We have appreciated the opportunity of working with you on this project and trust that this report is satisfactory to your requirements. Please feel free to contact the undersigned regarding any questions or further information that you may require.

Report prepared by:

ELR

Aaron Nicholson, B.Sc.

Environmental Scientist

aaron@elr.ca

Report peer reviewed by:

Hemmera Envirochem Inc.

Natasha Sandys, B.Sc., EP

Environmental Scientist

nsandys@hemmera.com

Report senior reviewed by:

ELR

Chris Jastrebski, M.Sc., R.P.Bio.

Project Manager

chris@elr.ca

Report senior reviewed by:

Hemmera Envirochem Inc.

Jason Wilkins, P.Ag., EP, CSAP

Director, Land Development and Projects

jwilkins@hemmera.com

# References

ASTM Standard D4448-01. 2013. Standard Guide for Sampling Groundwater Monitoring Wells. ASTM International, West Conshohocken, PA, 2013, www.astm.org.

ASTM Standard D6452-99 2012 Guide for Purging Methods for Wells used for Groundwater Quality investigations. ASTM International, West Conshohocken, PA, 2012, www.astm.org.

Canadian Council of Ministers of the Environment (CCME). 2014. Canadian Water Quality Guidelines for the Protection of Aquatic Life. Accessed online at http://st-ts.ccme.ca/, November 2014.

Rice, E.W., Baird, R.B., Eaton, A.D., & Clesceri, L.S. 2012. Standard Methods for the Examination of Water and Wastewater, 22nd Edition. American Public Health Association. ISBN: 9780875530130

Yukon Government. 2002. Environment Act O.I.C. 2002/171 Contaminated Sites Regulation.

# Statement of Limitations

This report was prepared by Hemmera Envirochem Inc. (“Hemmera”), based on fieldwork conducted by Hemmera, for the sole benefit and exclusive use of Government of Yukon. The material in it reflects Hemmera’s best judgment in light of the information available to it at the time of preparing this Report. Any use that a third party makes of this Report, or any reliance on or decision made based on it, is the responsibility of such third parties. Hemmera accepts no responsibility for damages, if any, suffered by any third party as a result of decisions made or actions taken based on this Report.

Hemmera has performed the work as described above and made the findings and conclusions set out in this Report in a manner consistent with the level of care and skill normally exercised by members of the environmental science profession practicing under similar conditions at the time the work was performed.

This Report represents a reasonable review of the information available to Hemmera within the established Scope, work schedule and budgetary constraints. It is possible that the levels of contamination or hazardous materials may vary across the Site, and hence currently unrecognised contamination or potentially hazardous materials may exist at the Site. No warranty, expressed or implied, is given concerning the presence or level of contamination on the Site, except as specifically noted in this Report. The conclusions and recommendations contained in this Report are based upon applicable legislation existing at the time the Report was drafted. Any changes in the legislation may alter the conclusions and/or recommendations contained in the Report. Regulatory implications discussed in this Report were based on the applicable legislation existing at the time this Report was written.

In preparing this Report, Hemmera has relied in good faith on information provided by others as noted in this Report, and has assumed that the information provided by those individuals is both factual and accurate. Hemmera accepts no responsibility for any deficiency, misstatement or inaccuracy in this Report resulting from the information provided by those individuals.

The liability of Hemmera to Government of Yukon shall be limited to injury or loss caused by the negligent acts of Hemmera. The total aggregate liability of Hemmera related to this agreement shall not exceed the lesser of the actual damages incurred, or the total fee of Hemmera for services rendered on this project.

TABLES

Appendix A

Site Photos

Appendix B

Laboratory Analytical Reports

Appendix C

Field Forms