## Introduction

This appendix includes tabular summaries of field investigation conducted by CH2M from fiscal year (FY) 2012 to FY2016. The tables (C-1 through C-5) show the applicable geographic site area, program objectives, summary of completed fieldwork scope, and applicable data report document citations for each program.

| Table C-1. Summary of Field and Analytical Programs for FY2012-2013 Faro Mine Remediation Project |  |  |  |  |
| --- | --- | --- | --- | --- |
| Geographic Areas | Remediation Design Purpose | Characterization Objectives | Completed Fieldwork Scope | Data Report Reference |
| Site-wide | Wildlife Baseline - Winter Furbearer and Wildlife Tracking Survey | Characterize presence, relative abundance, and habitat use by furbearers and other wildlife species during winter | Completed 12 transects (each composed of a triangle with 1-km side length) Identified and count wildlife tracks | Ecological Logistics & Research Ltd. (ELR). November 2012. Faro Mine Site Winter Track Study Memo – Activity R05.09.02B. Memo. Prepared for CH2M HILL Canada Limited (CH2M), Chuck Shewen, B.Sc., P.Biol. November 9. |
| RCTA, CVD | Construct Cross Valley Dam Interception System | Characterize hydrostratigraphy Evaluate GW quality Investigate GW/SW interaction in Rose Creek | Drilled seven borings to bedrock at toe of CVD and completed as monitoring wells Drilled four shallow paired borings downstream adjacent to Rose Creek and completed as monitoring wells Collected GW samples from all new wells and 56 existing locations Completed aquifer tests at all installed wells, including the four-day aquifer test using multiple pumping wells Installed and monitor 16 streambed piezometers | CH2M HILL Canada Limited (CH2M). March 2013m. Cross Valley Dam Interception System Investigation Data Report Faro Mine Remediation Project. Technical memorandum. Draft. Prepared for Government of Yukon. March 28. |
| Site-wide Waste Dumps, Site-wide Pits | Seep and Surface Water Monitoring | Characterize seepage water quality Characterize ARD evolution Characterize WTP influent quality | Sampled 109 seep locations and 10 SW locations in spring and fall | CH2M HILL Canada Limited (CH2M). October 2013a. 2012 Seepage, Surface Water, and Water Treatment Plant Influent Field Sampling Report, Faro Mine Remediation Project. Final. Prepared for Government of Yukon. October 29. |
| Rose Creek, NFRC, SFRC | Fish Telemetry Survey | Investigate seasonal movements, habitat use, critical habitat of Arctic Grayling | Sampled, tagged, and completed telemetry monitoring of arctic grayling | Environmental Dynamics Inc. (EDI). November 2012. Faro Mine Fish Telemetry Project – Interim Field Report No. 1. Prepared for CH2M HILL Canada Limited (CH2M), Dawn Kelly, Project Manager. November 6. |
|  |  |  |  | Environmental Dynamics Inc. (EDI). December 2012. Faro Mine Remediation Project Fish Telemetry Program – Interim Field Report No. 2. Prepared for CH2M HILL Canada Limited (CH2M), Charles Shewen. December 13. |
| Main Dump, Zone II Waste Dump, Intermediate Dump, Grum Waste Dump, Vangorda Waste Dump | Waste Rock Dump Monitoring | Characterize hydrostratigraphy Characterize GW quality Characterize ARD evolution Characterize geotechnical properties of waste Rock and underlying native material | Drilled six borings through waste dumps (NE, Zone II, Main, Intermediate, Grum, Vangorda), completed as monitoring wells, thermistor ports, gas sampling ports Sampled groundwater wells, and monitored temperatures and pore gas in fall and winter | CH2M HILL Canada Limited (CH2M). October 2013b. Waste Rock Dump Monitoring Report. Technical memorandum. Final. Prepared for Government of Yukon. October 29. |
| Mill Building Area | Construct new Faro and Vangorda Water Treatment Plant and Piping | Investigate geotechnical foundation conditions Characterize WTP influent water quality | Drilled 10 borings into bedrock. Completed borings as piezometers and developed Monitored water levels | CH2M HILL Canada Limited (CH2M). December 2012. Geotechnical Design Report, Water Treatment Plant, Faro Mine Remediation Project. Technical memorandum. Draft. Prepared for Government of Yukon. December 7. |
| Zone II Pit | Replace Zone II Pit Pumping Well | Characterize hydrostratigraphy in backfilled Zone II Pit | Drilled, installed, and developed replacement Zone II pit pumping well | CH2M HILL Canada Limited (CH2M). November 2012g. Task Authorization 007 Summary of Zone II Drilling Activities, Faro Mine Remediation Project. Technical memorandum. Draft. Prepared for Karen Furlong/Government of Yukon (YG). November 1. |
| Site-wide | Develop and Decommission Faro and Vangorda Grum Borrow Sources | Assess overall areal limit of potentially viable, currently undisturbed borrow source materials | Performed geological reconnaissance Hand-dug 20 test pits | CH2M HILL Canada Limited (CH2M). June 2013f. Limited Borrow Material Reconnaissance, Faro Mine Remediation Project. Technical memorandum. Final. Prepared for Government of Yukon. June 10. |
| Grum Overburden Dump | Develop Grum Overburden Dump Borrow Source | Quantify available borrow material Characterize geochemistry, fertility, and geotechnical properties of borrow material | Drilled six borings Excavated eight test pits | CH2M HILL Canada Limited (CH2M). June 2013i. Grum Overburden Dump Fatal Flaw Evaluation Data Report, Faro Mine Remediation Project. Technical memorandum. (Final). June 10 |
| Site-wide Surface Water | Construct New Faro and Vangorda Water Treatment Plant and Piping | Characterize water quality from pits and water collection systems Characterize WTP influent water quality | Collected SW samples from eight locations three times (spring, fall, winter) | CH2M HILL Canada Limited (CH2M). October 2013a. 2012 Seepage, Surface Water, and Water Treatment Plant Influent Field Sampling Report, Faro Mine Remediation Project. Final. Prepared for Government of Yukon. October 29. |
|  |  |  |  | CH2M HILL Canada Limited (CH2M). November 2012c. Water Treatment Plant Design – Influent Characterization/Plant Capacity Phasing, Faro Mine Remediation Project. Technical memorandum No. 1. Final. November 23. |
| NFRC, SFRC, Rose Creek | Aquatic Community and Habitat Baseline Field Study | Estimate fish density Characterize fish habitat | Deployed and track radio tags Collected and analyze tissue samples Collected habitat data | Environmental Dynamics Inc. (EDI). March 2015a. Arctic Grayling Telemetry & Fish Habitat Investigations, Faro Mine, 2012-2014. (Final). Prepared for CH2M HILL Canada Limited (CH2M). March 25. |
| WTP | Construct New Faro and Vangorda Water Treatment Plant and Piping | Evaluate sludge and effluent qualities produced using high-density sludge treatment | Operated three high-density sludge pilot WTP systems Tested effluent and sludge qualities under varying influent chemistries and system configurations | CH2M HILL Canada Limited (CH2M). November 2012a. Water Treatment Pilot Study Report, Faro Mine Remediation Project, TA 013. Technical Memorandum No. 4. Prepared for Government of Yukon and Aboriginal Affairs and Northern Development Canada (AANDC). November 27. |
| Notes: AANDC = Aboriginal Affairs and Northern Development Canada ARD = acid rock drainage CH2M = CH2M HILL Canada Limited CVD = Cross Valley Dam FSP = field sampling plan FY = fiscal year GW = groundwater km = kilometre NE = northeast NFRC = North Fork Rose Creek RCTA = Rose Creek Tailings Area SFRC = South Fork Rose Creek SW = surface water WRD = waste rock dump WTP = water treatment plant |  |  |  |  |

| Table C-2. Summary of Field and Analytical Programs for FY2013-2014 Faro Mine Remediation Project |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Geographic Areas | Remediation Design Purpose | Characterization Objectives | Completed Fieldwork Scope | Data Report Reference |  |
| RCD, CVD, Intermediate Impoundment, NFRC | Construct Cross Valley Dam Interception System | Characterize hydrostratigraphy Characterize GW Quality Characterize GW/SW interaction in RCD and NFRC | Completed surface geophysical transect along potential CVD SIS alignment (GPR, resistivity, seismic refraction) Drilled one boring downstream from CVD to investigate potential fractured bedrock zone Drilled, installed and developed six well clusters along the RCD - each consisting of paired wells and a stream stage Drilled, installed, and developed a well cluster of three wells in the north portion of the Intermediate Impoundment Collected GW samples Conducted four aquifer tests at RCD well clusters Installed and monitored 19 streambed piezometers in the NFRC upstream of the Rock Drain Conducted a specific conductivity survey in the RCD adjacent to the Secondary Impoundment | CH2M HILL Canada Limited (CH2M). February 2014q. 2013 Cross Valley Dam Seepage Interception System Field Investigation Data and Analysis Report, Faro Mine Remediation Project. Final. Prepared for Government of Canada as represented by Aboriginal Affairs and Northern Development Canada and the Government of Yukon. February 7. |  |
| Vangorda Creek, Granitic Rock Outcrop, Grum Waste Rock Dump Quarry | Construct Replacement Vangorda Creek Diversion | Characterize Hydrostratigraphy Evaluate channel hydraulics and morphology Characterize geotechnical foundation conditions Characterize rip rap borrow material geotechnical properties | Drilled three borings along proposed VCD alignment (SRK, November 2010), completed as standpipe piezometers Completed resistivity, GPR, seismic reflection and seismic refraction geophysical surveys of SRK’s proposed VCD alignment, granitic rock outcrop, and Grum Waste Rock Dump Quarry Collected and logged surface samples from Granitic Rock Outcrop and Grum Waste Rock Dump Quarry Drilled two diamond core drill holes in the Granitic Rock Outcrop Completed topographic survey of SRK's proposed VCD alignment Completed fluvial geomorphic assessment of Vangorda Creek Inspected meteorological stations Completed reconnaissance of SRK's proposed VCD alignment | CH2M HILL Canada Limited (CH2M). March 2014d. 2013 Vangorda Creek Diversion Field Investigation Data and Analysis Report, Faro Mine Remediation Project. Final. Prepared for Government of Yukon and Government of Canada as represented by Aboriginal Affairs and Northern Development Canada. March 28. |  |
| Ridge North of Grum Pit, Grum Overburden Dump, Haul Road Borrow, Rose Creek Borrow | Develop and Decommission Faro-Grum-Vangorda Borrow Sources | Characterize geotechnical properties, geochemistry, and fertility of potential borrow material Quantify available borrow material | Logged and sampled surface bedrock exposures Collected surface soil samples Drilled 14 pionjar borings Drilled six sonic borings Excavated 40 test pits | CH2M HILL Canada Limited (CH2M). March 2014n. Phase II Borrow Reconnaissance, Faro Mine Remediation Project. Final. Prepared for Government of Yukon and Government of Canada as represented by Aboriginal Affairs and Northern Development Canada. March 28. |  |
|  |  |  |  | CH2M HILL Canada Limited (CH2M). March 2014b. Faro Water Treatment Plant Geotechnical Design Report Addendum, 2013 Borrow Source and Quarry Investigation, Faro Mine Remediation Project. Final. Prepared for Government of Yukon and Government of Canada as represented by Aboriginal Affairs and Northern Development Canada. March 31. |  |
| Faro Pit, Vangorda Pit, Grum Pit | Evaluate pit wall stability | Investigate slope stability of pit walls | Completed field reconnaissance/mapping Installed survey monitoring points at Grum Pit | CH2M HILL Canada Limited (CH2M). March 2014q. Faro Mine Complex 2013 Pit Wall Stability Inspection, Faro Mine Remediation Project. Final. Prepared for Government of Yukon and Government of Canada as represented by Aboriginal Affairs and Northern Development Canada. March 26. |  |
| NFRD, RCD, ID, NFRC, SFRC, Upper Parking Lot Waste Dump, Lower Northwest Waste Dump | Construct Down Valley Interim Hydraulic Upgrades | Characterize hydrostratigraphy Characterize geotechnical properties at base of Rock Drain, RCD, and ID Characterize geochemical and geotechnical properties of potential calc silicate borrow material Characterize flows in NFRC and RCD | Performed geological reconnaissance of RCD Excavated seven test pits along crest of ID Excavated three test pits in ID spillway Completed five geophysical transects across NFRC upstream and downstream of the NFRD Drilled two borings at downstream toe of Rock Drain Established continuous stream stage monitoring at SW stations R10 and X2 Performed dye test through the NFRD Surveyed weekly water elevations at NF1 and NF2 Excavated 15 test pits in the Upper Parking Lot WRD and Lower Northwest WRD | CH2M HILL Canada Limited (CH2M). March 2014m. 2013 Down Valley Interim Hydraulic Upgrades Field Investigation Data and Analysis Report, Faro Mine Remediation Project. Final. Prepared for Government of Yukon and Government of Canada as represented by Aboriginal Affairs and Northern Development Canada. March 28. |  |
| Site-wide Waste Dumps, Site-wide Pits | Seep and Surface Water Monitoring | Characterize seepage water quality Characterize ARD evolution Characterize WTP influent quality | Sampled 109 seep locations and 10 surface water locations in spring and fall | CH2M HILL Canada Limited (CH2M). March 2014c. 2013 Seep Sampling and Waste Rock Dump Monitoring Data and Analysis Report, Faro Mine Remediation Project. Final. Prepared for Government of Yukon and Government of Canada as represented by Aboriginal Affairs and Northern Development Canada. March 31. |  |
| Main Dump, Zone II Waste Dump, Grum Waste Dump, Intermediate Dump, Vangorda Waste Dump | Waste Rock Dump Monitoring | Characterize GW quality Characterize ARD evolution | Monitored thermistors, pore gas and groundwater in spring, fall and winter | CH2M HILL Canada Limited (CH2M). March 2013n. Waste Rock Dump Monitoring Data Report Faro Mine Remediation Project. Technical memorandum. Prepared for Government of Yukon. March 28. |  |
| Site-wide Surface Water Bodies | Waterfowl Study | Characterize quality of potential waterfowl habitat Evaluate potential for contaminant ingestion | Performed visual survey of potential waterfowl habitat with spotting scope and binoculars Performed waterfowl habitat surveys Sampled vegetation and sediment | Ecological Logistics & Research Ltd (ELR). December 2013. Faro Mine Complex 2013 Waterfowl Studies. Prepared for CH2M HILL Canada Limited (CH2M), Chuck Shewen. December 12. |  |
| Notes: ARD = acid rock drainage CVD = Cross Valley Dam FY = fiscal year GPR = ground-penetrating radar GW = groundwater ID = Intermediate Dam NFRC = North Fork Rose Creek RCD = Rose Creek Diversion SRK = SRK Consulting Engineers and Scientists SW = surface water VCD = Vangorda Creek Diversion WRD = waste rock dump |  |  |  |  |

| Table C-3. Summary of Field and Analytical Programs for FY2014-2015 Faro Mine Remediation Project |  |  |  |  |
| --- | --- | --- | --- | --- |
| Geographic Areas | Remediation Design Purpose | Characterization Objectives | Completed Fieldwork Scope | Data Report Reference |
| Faro Pit | Construct Faro Pit Safety Berm | Observe surface conditions and hydrologic | Performed geological reconnaissance of pit perimeter berm alignment provided in SRK Draft 4a Closure Plan (SRK, November 2010) | CH2M HILL Canada Limited (CH2M). March 2015oo. Summary of 2014 Field Investigation – Faro Pit Safety Berm; FSP Name: Activity 101.2 – Field Reconnaissance for the Faro Pit Safety Berm. Technical memorandum. Draft. Prepared for Government of Yukon. March 12. |
| FCD | Upgrade Faro Creek Diversion | Characterize surface conditions at existing diversions Characterize surface conditions along SRK proposed replacement Faro Creek Diversion alignment (SRK, November 2010) | Performed geological reconnaissance Performed Bed and Bank Material Characterization Completed 20 channel cross section surveys | CH2M Canada Limited (CH2M). March 2015i. Summary of 2014 Field Investigation – Faro Creek Diversion East Valley and Faro Creek Diversion West Valley Interceptor Ditch FSP Name: 102.1 – Faro Creek Diversion (East Valley); 102.2 – Faro Creek Diversion (West Valley Interceptor Ditch). Technical memorandum. Prepared for Government of Yukon. March 18. |
| NWID | Upgrade North Wall Interceptor Ditch | Characterize channel shape and surface conditions Characterize geotechnical properties of channel bank material | Excavated four test pits Performed geological reconnaissance Completed eight channel cross section surveys | CH2M HILL Canada Limited (CH2M). March 2015j. Summary of 2014 Field Investigation – North Wall Interceptor Ditch and Upper Guardhouse Creek FSP Name: 102.3 – Upgrade North Wall Interceptor Ditch. Technical memorandum. Draft. Prepared for Government of Yukon. March 19. |
| Lower Guardhouse Creek | Install Lower Guardhouse Creek Interception System | Characterize channel shape and general conditions | Performed geological reconnaissance Performed specific conductivity survey | CH2M HILL Canada Limited (CH2M). March 2015k. Summary of 2014 Field Investigation – Lower Guardhouse Creek Interception FSP Name: 102.4 – Lower Guardhouse Creek Interception. Technical memorandum. Prepared for Government of Yukon. March 19. |
| Oxide Fines Stockpile, Low Grade Ore Stockpile C | Relocate Oxide Fines to Low-grade Stockpile “C” | Characterize geotechnical properties of oxide fines stockpile and LGSPC Characterize geochemical properties of the oxide fines stockpile and LGSPC Delineate extent of oxide fines stockpile | Drilled one boring in Oxide Fines Stockpile and installed a GW monitoring well, thermistor port, and pore gas ports Excavated four test pits Collected two shallow hand-dug test pit samples | CH2M HILL Canada Limited (CH2M). March 2015l. Summary of 2014 Field Investigation – Relocate Oxide Fines to Low-grade Stockpile “C” FSP Name: 104.1 – Relocate Oxide Fines to LGSPC. Technical memorandum. Prepared for Government of Yukon. March 13. |
| ETA | Remove ETA Tailings to Rose Creek Tailings Facility | Investigate tailings thickness and geotechnical properties Characterize extent of contamination in tailings and underlying soils Characterize geotechnical properties of road fill in mine access road adjacent to ETA | Drilled three borings Advanced six seismic CPT soundings | CH2M Canada Limited (CH2M). March 2015m. Summary of 2014 Field Investigation – Relocate ETA Tailings to Rose Creek Tails Area FSP Name: 105.1 – Remove Tailings to Rose Creek Tailings Facility. Technical memorandum. Prepared for Government of Yukon. March 9. |
| ETA | Design of a Surface Water Collection Structure for the Emergency Tailings Area | Characterize bedrock conditions Characterize overburden and bedrock conditions along the proposed alignment of the mine Access Road Quantify surface water flow at FCS-4 and X-23 | Completed two seismic refraction surface geophysical survey lines Excavated one test pit Installed stilling wells and water level transducers at two SW locations Performed geological reconnaissance | CH2M Canada Limited (CH2M). March 2015n. Summary of 2014 Field Investigation – Design of a Surface Water Collection Structure for the Emergency Tailings Area FSP Name: 105.2 – Install Emergency Tailings Area Surface Water Collection System. Technical memorandum. Prepared for Government of Yukon. March 9. |
| Main Dump, Intermediate Dump | Regrade and Reslope Main Dump and Intermediate Dump Sulphide Cells | Delineate geographic extent of the Main Dump and Intermediate Dump sulphide cells Characterize geotechnical properties of the sulphide material Determine foundation preparation requirements for a cover Characterize infiltration properties of the sulphide cell material | Excavated 12 test pits Drilled one boring in the Main WRD and installed a thermistor port and pore gas ports | CH2M HILL Canada Limited (CH2M). March 2015o. Summary of 2014 Field Investigation – Delineate Main and Intermediate Sulphide Cells for Regrading and Capping Design FSP Name: 106.1 – Regrade and Reslope Main and Intermediate Sulphide Cells. Technical memorandum. Prepared for Government of Yukon. March 13. |
| Faro Waste Dumps | Regrade and Reslope Faro Waste Rock Dumps | Characterize geotechnical properties of waste rock material Characterize geochemical kinetics and leachability of waste rock material | Excavated seven test pits. | CH2M HILL Canada Limited (CH2M). March 2015p. Summary of 2014 Field Investigation – Regrade and Reslope Faro Waste Rock Dumps (Upper Surface) FSP Name: 106.2 – Regrade and Reslope Faro Waste Rock Dumps (Upper Surfaces). Technical memorandum. Prepared for Government of Yukon. March 13. |
| Faro Waste Dumps | Regrade and Reslope Faro Waste Rock Dumps | Characterize geotechnical foundation conditions along toe of Faro WRDs Characterize hydrostratigraphy in areas of potential WRD footprint expansion | Performed geological reconnaissance Excavated five test pits. | CH2M HILL Canada Limited (CH2M). March 2015q. Summary of 2014 Field Investigation – Regrade and Reslope Faro Waste Rock Dumps (Toe of Slope) FSP Name: 106.3 – Regrade and Reslope Faro Waste Rock Dumps (Toe of Slope). Technical memorandum. Prepared for Government of Yukon. March 13. |
| NFRC, SFRC | Upgrade North Fork Rose Creek | Document general conditions along NFRC and SFRC | Performed geological reconnaissance Characterized bed and bank material Completed 14 channel cross section surveys | CH2M Canada Limited (CH2M). March 2015i. Summary of 2014 Field Investigation – Upgrade North Fork Rose Creek FSP Name: 107.1 – Upgrade North Fork Rose Creek. Technical memorandum. Prepared for Government of Yukon. March 19. |
| Zone II Outwash | Relocate Zone II Outwash Material to Intermediate Sulphide Cell | Delineate extent of contamination in Zone II outwash sediments | Excavated four test pits. | CH2M Canada Limited (CH2M). March 2015j. Summary of 2014 Field Investigation – Relocate Zone II Outwash Material to Intermediate Sulphide Cell FSP Name: 107.3 – Relocate Zone II Outwash Material to Intermediate Sulphide Cell). Technical memorandum. Prepared for Government of Yukon. March 19. |
| Zone II Outwash | Install Zone II Outwash Collection System | Characterize hydrostratigraphy in Zone II Outwash | Drilled, installed, developed and sampled eight monitoring wells Performed packer testing in bedrock Completed stream stage monitoring Completed one seismic refraction geophysics line | CH2M HILL Canada Limited (CH2M). March 2015t. Summary of 2014 Field Investigation – Install Zone II Outwash Collection System FSP Name: 107.4 – Install Zone II Outwash Collection System. Technical memorandum. Prepared for Government of Yukon. March 2. |
| S-Wells | Install S-Wells Groundwater Collection System | Characterize hydrostratigraphy in S-Wells area Investigate GW/SW interaction in the NFRC downstream of the Rock Drain | Drilled, installed, developed and sampled six monitoring wells Installed and monitored 16 streambed piezometers Performed packer testing in bedrock Completed one seismic refraction geophysics line Completed stream stage monitoring | CH2M HILL Canada Limited (CH2M). March 2015u. Summary of 2014 Field Investigation – Install S-Wells Groundwater Collection System FSP Name: 107.5 – Install S-Wells Groundwater Collection System. Technical memorandum. Prepared for Government of Yukon. March 19. |
| Former Faro Lube Shack and Tank Farm | Relocate Contaminated Soil from Faro Lube Shack and Tank Farm Area and Cover | Delineate extent of hydrocarbon contamination | Excavated two test pits | CH2M HILL Canada Limited (CH2M). March 2015v. Summary of 2014 Field Investigation – Relocate Contaminated Soil from Faro Lube Shack and Tank Farm Area and Cover FSP Name: 108.5 – Relocate Contaminated Soil from Faro Lube Shack and Tank Farm Area and Cover. Technical memorandum. Prepared for Government of Yukon. March 12. |
| Intermediate Tailings Impoundment | Dewater and Cover Intermediate Tailings Area | Delineate thickness and geotechnical properties of tailings. | Drilled one boring and completed with thermistor port casing Advanced three CPT soundings | CH2M Canada Limited (CH2M). March 2015m. Summary of 2014 Field Investigation – Dewater and Cover Intermediate Tailings Area FSP Name: 201.1 – Dewater and Cover Intermediate Tailings Area. Technical memorandum. Prepared for Government of Yukon. March 13. |
| Intermediate Tailings Impoundment | Construct New Side Channel and Side Channel Dike | Delineate thickness and geotechnical properties of tailings Evaluate seasonal frost penetration. | Drilled 4 borings and completed with thermistor port casings Advanced 9 CPT soundings Monitored thermistor ports in fall and winter. | CH2M Canada Limited (CH2M). March 2015o. Summary of 2014 Field Investigation – Construct New Side Channel and Side Channel Dike FSP Name: 201.5 – Construct New Side Channel and Side Channel Dike. Technical memorandum. Prepared for Government of Yukon. March 13. |
| ID Spillway | Construct Intermediate Dam Spillway | Characterize geotechnical foundation conditions along proposed ID Spillway alignment | Drilled two borings into bedrock Performed packer testing | CH2M Canada Limited (CH2M). March 2015p. Summary of 2014 Field Investigation – Construct Intermediate Dam Spillway FSP Name: 201.6 – Construct Intermediate Dam Spillway. Technical memorandum. Prepared for Government of Yukon. March 13. |
| RCD | Upgrade Rose Creek Diversion | Characterize channel morphology | Completed two channel cross section surveys | CH2M Canada Limited (CH2M). March 2015n. Summary of 2014 Field Investigation – Upgrade Rose Creek Diversion FSP Name: 201.3 – Upgrade Rose Creek Diversion. Technical memorandum. Prepared for Government of Yukon. March 13. |
| Secondary Tailings Impoundment near Fuse Plug | Construct Erosion Dissipation Structure | Characterize local setting and foundation conditions at location of proposed erosion dissipation structure | Performed geological reconnaissance | CH2M Canada Limited (CH2M). 2015q. Summary of 2014 Field Investigation – Erosion Dissipation Structure FSP Name: 203.2 – Erosion Dissipation Structure. Technical memorandum. Prepared for Government of Yukon. March 13. |
| RCD near Fuse Plug | Construct New Overflow Weir | Investigate geotechnical foundation conditions at location of proposed overflow weir | Advanced two borings into bedrock | CH2M HILL Canada Limited (CH2M). March 2015cc. Summary of 2014 Field Investigation – Construct New Overflow Weir FSP Name: 203.4 – Construct New Overflow Weir. Technical memorandum. Prepared for Government of Yukon. March 13. |
| Pump House Pond | Develop and Decommission Faro Borrow Sources – Granular Soil, Pump House Borrow | Characterize geotechnical and geochemical properties of borrow material Delineate extent of potential borrow source | Excavated five test pits | CH2M Canada Limited (CH2M). March 2015f. Summary of 2014 Field Investigations – Pump House Borrow Source, Faro Mine Remediation Project. Technical memorandum. Prepared for Government of Yukon. March 27. |
| Granitic Rock Outcrop | Develop and Decommission Grum/Vangorda Rip Rap Sources – Granitic Rock Outcrop | Characterize geotechnical properties of borrow material Delineate extent of potential borrow source | Drilled two borings Completed two seismic refraction geophysics lines | CH2M Canada Limited (CH2M). March 2015g. Summary of 2014 Field Investigations – Granitic Rock Outcrop, Faro Mine Remediation Project. Technical memorandum. Prepared for Government of Yukon. March 27. |
| RCD, NFRC | Site-wide Surface Water Monitoring | Characterize surface water flows | Installed stilling wells and water level transducers at five surface water monitoring locations | CH2M HILL Canada Limited (CH2M). March 2015dd. Summary of 2014 Field Investigation – Site-wide Surface Water Monitoring FSP Name: SW-005 – Site-wide Surface Water Monitoring. Technical memorandum. Prepared for Government of Yukon. March 31. |
| Site-wide Waste Dumps, RCTA | Site-wide Geochemical Monitoring | Characterize seepage water quality Characterize ARD evolution Characterize GW quality in RCAA Characterize pit Lake water quality Characterize WTP influent quality Characterize waste rock geotechnical and geochemical characteristics and extents Characterize contaminant loading to Rose Creek Alluvial aquifer from ETA. | Drilled 5 borings in Original Tailings Impoundment, Northwest Dump and Northeast Dump Installed, and develop four monitoring wells in WRDs and Original tailings impoundment Installed gas ports and thermistor ports in two WRD borings Monitored WRD GW, thermistor, soil gas (spring, fall, winter) Sampled 14 streambed piezometers upstream of Rock Drain Sampled 85 wells in RCTA (fall) Monitor 188 seeps (spring, fall). Complete Pit Lake Profiling. | CH2M HILL Canada Limited (CH2M). March 2015b. Summary of 2014 Field Investigation – Site-wide Geochemical Monitoring FSP Name: SW-015 – Site-wide Geochemical Monitoring. Technical memorandum. Prepared for Government of Yukon. March 31. |
| Zone II Outwash | Reclaim Zone II Outwash | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples | CH2M HILL Canada Limited (CH2M). March 2015kk. Vegetation and Soil – 2014 Field Summary Report, Faro Mine Remediation Project Proposal. (Final). Prepared for Government of Canada as represented by Aboriginal Affairs and Northern Development Canada and the Government of Yukon. March. |
| Site-wide Mine Disturbance Areas | Baseline Mine Vegetation Regrowth Study | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples | CH2M HILL Canada Limited (CH2M). February 2015c. Site-wide Soil Characterization Report, Faro Mine Remediation Project. Draft. Prepared for Government of Canada as represented by Aboriginal Affairs and Northern Development Canada and the Government of Yukon. February 9. |
| Undisturbed Portions of Local Study Area | Site-wide Soils and Vegetation Baseline Study | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples |  |
| LGSP A | Cover and Revegetate LGSP A | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples |  |
| ETA | Cover and Revegetate ETA | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples |  |
| Northeast WRD | Cover and Revegetate Northeast Sulphide Cell | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples |  |
| Main Dump and Intermediate Dump | Cover and Revegetate Main Dump and Intermediate Dump Sulphide Cells | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples |  |
| Mt. Mungly Waste Dump and Southwest Pit Wall Dump | Cover and Revegetate Mt Mungly (East and West) and Southwest Pit Wall Dumps | Determine terrain Classification Characterize vegetation community Characterize soil series Investigate toxicology | Conducted field survey of soil and vegetation Collected vegetation tissue and soil samples |  |
| Pipeline Alignment from CVD to WTP | Construct Cross Valley Dam SIS Pipeline | Characterize geotechnical conditions along proposed CVD SIS pipeline alignment | Excavated 10 test pits | CH2M HILL Canada Limited (CH2M). January 2015b. Cross Valley Dam Seepage Interception System Test Pit Data Summary, Faro Mine Remediation Project. Technical memorandum. Draft. Prepared for Government of Yukon. January 30. |
| Notes: CVD = Cross Valley Dam ETA = Emergency Tailings Area FCD = Faro Creek Diversion FY = fiscal year GW = groundwater ID = Intermediate Dam LGSPC = low-grade ore stock pile “C” NFRC = North Fork Rose Creek RCD = Rose Creek Diversion NWID = North Wall Interceptor Ditch SFRC = South Fork Rose Creek SIS = seepage interception system SRK = SRK Consulting Engineers and Scientists WRD = waste rock dump WTP = water treatment plant YG = Government of Yukon |  |  |  |  |

| Table C-4. Summary of Field and Analytical Programs for FY2015-2016 Faro Mine Remediation Project |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Geographic Areas | Remediation Design Purpose | Characterization Objectives | Completed Fieldwork Scope |  | Data Report Reference |  |
| Site-wide Waste Dumps and Pits, RCTA | Site-wide Geochemical Monitoring | Characterize seepage water quality Characterize ARD evolution Characterize GW quality in RCAA Characterize pit lake water quality | Monitored WRD GW, thermistor, soil gas (spring, fall, winter) Sampled 85 wells in RCTA (fall) Monitored 188 seeps (spring, fall) Completed pit lake profiling (fall, winter) |  | CH2M Canada Limited (CH2M). April 2016d. Summary of 2015 Field Investigation – Site-wide Geochemical Monitoring, FSP Name: SW-015 – Site-wide Geochemical Monitoring. Technical memorandum. Draft. Prepared for Government of Yukon. April 6. |  |
| FCD and Faro Valley | Upgrade Faro Creek Diversion | Characterize hydrostratigraphy in Faro Valley Evaluate proportion of leakage from FCD vs. quantity of GW flow beneath channel | Drilled, installed, and developed three well clusters (three wells per cluster) |  | CH2M Canada Limited (CH2M). April 2016g. Summary of 2015 Field Investigation - Faro Valley Seepage Investigation, Faro Mine Remediation Project, Field Sampling Plan Name: 106.1 – Faro Valley Seepage Investigation. Technical memorandum. Draft. Prepared for Government of Yukon. April 1. |  |
| Faro Pit and FCD | Upgrade Faro Creek Diversion | Characterize geotechnical conditions and stability of east wall of Faro Pit | Drilled two borings into bedrock, Performed downhole acoustic and optical televiewer surveys Installed two slope inclinometers nested with vibrating wire piezometers Drilled two shallow borings and completed with standpipe piezometers Established routine monitoring schedule for piezometer and inclinometer installations |  | BGC Engineering Inc. (BGC). March 2016. Faro Open Pit Stability Assessment, Site-wide Design Basis Report. Draft. Prepared for CH2M HILL Canada Limited (CH2M HILL). March 18 |  |
| FCD, Upper Guardhouse Creek, NWID | Upgrade Faro Creek Diversion, Upper Guardhouse Creek and NWID | Characterize channel shape and morphology | Complete rapid geomorphic assessment Survey channel cross sections at nine locations |  | CH2M Canada Limited (CH2M). April 2016h. Summary of 2015 Field Investigation – West Side and East Side Diversions, Faro Mine Remediation Project. Technical memorandum. Draft. Prepared for Government of Yukon. April 1. |  |
| FCD | Upgrade Faro Creek Diversion | Characterize geotechnical conditions along proposed replacement FCD alignment Characterize hydrostratigraphy and permafrost conditions in Faro Creek Valley Confirm bedrock characterization at east wall of Faro Pit North Instability | Completed 21 surface geophysical survey transects Drilled six borings into bedrock Installed monitoring wells and thermistor ports in five borings |  | CH2M Canada Limited (CH2M). August 2016a. Summary of 2015-2016 Field Investigation – Faro Creek Diversion Proposed Realignment, Field Sampling Plan Name: Faro Creek Diversion New Alignment Drilling. Technical memorandum. Draft. Prepared for Government of Yukon. August 18. |  |
| CVD Pond, ID Pond | Remediate CVD and ID Pond | Prepare higher resolution CVD Pond stage - storage curve Characterize thickness and volume of deposited sediment and tailings Characterize geochemical and geotechnical properties of sediments in CVD and ID ponds | Bathymetry and sub-bottom profiling of CVD Pond Advanced eight CPT soundings in the ID Pond Advanced three CPT soundings in CVD Pond Drilled eight sonic borings in CVD Pond |  | CH2M Canada Limited (CH2M). August 2016c. Summary of FY 2015 – 2016 Field Investigations – CVD and ID Pond Field Data Report, Field Sampling Plan Name: CVP and ID Pond CPT and Sonic Sampling. Technical Memorandum. Prepared for Government of Yukon. August 25. |  |
| ID Spillway | Upgrade Intermediate Dam Spillway | Characterize depth and condition of bedrock surface in the area of the proposed PMF spillway Characterize geotechnical conditions in ID Spillway footprint Characterize permeability of overburden at upstream and downstream end of spillway | Complete one surface geophysical survey lines with seismic reflection and resistivity Complete six surface geophysical survey lines with multichannel analysis of surface waves and seismic refraction Drill five borings into bedrock, and install and develop two pumping wells and three piezometers Complete packer testing in bedrock |  | CH2M Canada Limited (CH2M). April 2016f. Summary of 2015 Field Investigation – Construct Intermediate Dam Spillway, Faro Mine Remediation Project, Field Sampling Plan Name: 201.6 Construct Intermediate Dam Spillway. Technical memorandum. Draft. Prepared for Government of Yukon. March 31. |  |
| Site-wide | Water Management Inventory | Document locations and specifications of water conveyance infrastructure | Complete survey and inventory of water conveyance structures |  | CH2M HILL Canada Limited (CH2M). February 2016b. Water Conveyance System Mapping and Inventory, Faro Mine Remediation Project. Technical memorandum. Prepared for Government of Yukon. February 1. |  |
| Notes: |  |  |  |  |  |
| ARD = acid rock drainage BGC = BCG Engineering CPT = cone penetration test CVD Pond = Cross Valley Pond FCD = Faro Creek Diversion |  |  |  | GW = groundwater ID = Intermediate Dam PMF = probable maximum flood RCAA = Rose Creek Alluvial Aquifer |  |

| Table C-5. Summary of Field and Analytical Programs for FY2016-2017 Faro Mine Remediation Project |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Geographic Areas | Remediation Design Purpose | Characterization Objectives | Completed Fieldwork Scope |  | Data Report Reference |  |
| Site-wide Waste Dumps and Pits, RCTA | Site-wide Geochemical Monitoring | Characterize seepage water quality Characterize ARD evolution | Monitored WRD GW, thermistor, soil gas (spring) Monitored 188 seeps (spring) |  | CH2M Canada Limited (CH2M). August 2016j. Summary of 2016 Field Investigation – Site-wide Geochemical Monitoring. Technical memorandum. Draft. Prepared for Government of Yukon. August 18. |  |
| FCD and Faro Valley | Upgrade Faro Creek Diversion | Characterize hydrostratigraphy in Faro Valley Evaluate proportion of leakage from FCD vs. quantity of GW flow beneath channel Delineate spatial extent of permafrost | Completed aquifer testing at three well clusters Completed surveying of 21 surface geophysical transects FCD alignment Collected volumetric discharge measurements at four locations Collected GW samples from one location at each well cluster |  | CH2M Canada Limited (CH2M). August 2016h. Summary of 2016 Field Investigation – Faro Creek Diversion Aquifer Testing, Faro Mine Remediation Project. Technical Memorandum. Draft. Prepared for Government of Yukon. August 24. |  |
| ID Pond | Remediate ID Pond | Prepare higher resolution ID Pond stage - storage curve Characterize thickness and volume of deposited sediment and tailings | Bathymetry and sub-bottom profiling of ID Pond Completed surveying of 7 surface geophysical transects for ID PMF Spillway |  | CH2M Canada Limited (CH2M). August 2016c. Summary of FY 2015 – 2016 Field Investigations – CVD and ID Pond Field Data Report, Field Sampling Plan Name: CVP and ID Pond CPT and Sonic Sampling. Technical Memorandum. Prepared for Government of Yukon. August 25. |  |
| Notes: |  |  |  |  |  |
| ARD = acid rock drainage CPT = cone penetration test CVD Pond = Cross Valley Pond FCD = Faro Creek Diversion |  |  |  | GW = groundwater ID = Intermediate Dam RCTA – Rose Creek Tailings Area WRD – Waste Rock Dump |  |