Mount Nansen June 2014

Groundwater Monitoring and Sampling

Prepared for:

Yukon Government

Assessment and Abandoned Mines Branch

P.O. Box 2703

Whitehorse, Yukon  Y1A 2C6

Prepared by:

Hemmera Envirochem Inc.

230 – 2237 2nd Avenue

Whitehorse, YT  Y1A 0K7

File: 1343-005.03

November 2014

TABLE OF CONTENTS

1.0	Introduction	1

1.1	Site Location	1

1.2	Scope of Work	1

1.3	Sample Sites	5

2.0	Methodology	8

2.1	Protocols	8

2.2	Well Measurements and Purging	8

2.3	Field Parameters	9

2.4	Groundwater Sampling	10

2.5	Data Analysis	10

2.6	Quality Assurance and Quality Control	10

2.6.1	Field QA/QC	10

2.6.2	QA/QC	11

3.0	Results	12

3.1	Groundwater Sampling Summary	12

3.2	Analytical Results	16

3.2.1	Dome Creek	16

3.2.2	Mill Complex	16

3.2.3	Brown McDade Pit	17

3.2.4	Pony Creek	18

3.2.5	Seepage Dam	18

3.2.6	Tailings Facility	18

3.3	Quality Assurance and Quality Control Results	19

4.0	Recommendations	20

5.0	Closure	21

6.0	References	22

7.0	Statement of Limitations	23

List of Tables (within text)

Table 1-1	Summary of Samples Collected at each Well Location	3

Table 2-1	Groundwater Sampling – Field Parameter Purging Criteria	9

Table 2-2	Groundwater Sampling – Preservation and Intended Analysis	10

Table 3-1	Groundwater Field Parameters and Well Measurements for 2014 Spring Sampling Program	13

List of Figures (within text)

Figure 1-1	Site Location – Mount Nansen Mine Complex	2

Figure 1-2	Groundwater Sampling Locations – Dome Creek and Tailings Facility	6

Figure 1-3	Groundwater Sampling Locations – Mill Complex and Brown McDade Pit	7

List of Tables (following text)

Table A	Groundwater Sampling Analytical Results and CCME Guideline Exceedances for 2014 Spring Sampling Program

Table B	QA/AC Analytical Data

List of Appendices

Appendix A 	Site Photos

Appendix B 	Field Forms

Appendix C	Laboratory Reports

# Introduction

Hemmera Envirochem Inc. (“Hemmera”) and Ecological Logistics & Research Ltd. (Hemmera / ELR) were retained by the Government of Yukon (GY), Assessment and Abandoned Mines (AAM) to conduct a groundwater monitoring and sampling program at the Mount Nansen Site in June of 2014. This report summarizes the activities conducted, the field conditions encountered, and the in-situ and laboratory analytical results for the program.

## Site Location

The Mount Nansen Site (the Site) is located approximately 45 km west of the Town of Carmacks (70 km by road). This Type II abandoned mine site consists of three (3) primary areas of existing infrastructure: the Brown McDade Pit, a Mill Complex, and a Tailings Facility (Figure 1-1). Previously installed groundwater sampling stations exist throughout much of the site, a subset of which were sampled during the June 2014 groundwater monitoring and sampling program. The groundwater monitoring locations included in this program are described in Sections 1.2 and 1.3.

## Scope of Work

The scope of work included the coordination and execution of the spring groundwater monitoring and sampling program and the preparation of this report. The report provides a summary of the monitoring and sampling activities, methodologies (including and deviations), laboratory analytical results, comparison to the applicable guidelines, and recommendations relating to sample procedures and monitoring well condition. This report does not provide an interpretation of the analytical results or provide recommendations relating to contaminated groundwater. Groundwater sampling at the Mount Nansen site was conducted over a four (4) day period, between June 26 and 29, 2014. Sampling was conducted by a team of four (4) field staff from Hemmera/ELR (Aaron Nicholson, Rusto Martinka, Andrew Brown, and Michelle McKay). A total of 65 groundwater wells were included in the June sampling event (Table 1-1).

At each well (sampling station) headspace gas concentrations were measured, basic well and water level parameters were measured (Depth-to-Water, Depth-to-Bottom, well diameter, and well stick-up height), the well was purged, then field water quality parameters were measured. Finally, water quality samples were collected for laboratory analysis. A detailed description of the sampling methods and water quality parameters monitored and sampled for is provided in Section 2, below.

Figure 1-1	Site Location – Mount Nansen Mine Complex

Table 1-1	Summary of Samples Collected at each Well Location

| Area | Well Name | UTM (Zone 08N) |  | Status1 | Sample Collected | QA/QC Sample Collected |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Easting | Northing |  |  |  |
| Dome Creek | GSI-DC-01B | 387675 | 6881124 | Insufficient Volume | - | - |
|  | GSI-DC-02B | 387839 | 6881129 | Insufficient Volume | - | - |
|  | GSI-DC-03B | 388107 | 6881079 | Insufficient Volume | - | - |
|  | GSI-DC-05B | 388725 | 6880836 | Insufficient Volume | - | - |
|  | GSI-DC-06B | 389788 | 6880567 | Good |  | - |
|  | GSI-DC-07B | 390065 | 6880641 | Good |  | - |
|  | GSI-DC-08-B | 390311 | 6880583 | Frozen | - | - |
|  | GSI-DC-09-B | 390614 | 6880494 | Good |  | - |
|  | GSI-DC-10-B | 390859 | 6880452 | Good |  | - |
| Mill Complex | GSI-HA-01A | 387842 | 6881132 | Insufficient Volume | - | - |
|  | GSI-HA-02A | 387861 | 6881129 | Insufficient Volume | - | - |
|  | GSI-HA-03A | 387878 | 6881131 | Insufficient Volume | - | - |
|  | GSI-HA-04A | 387916 | 65881130 | Insufficient Volume | - | - |
|  | GSI-HA-05A | 387898 | 6881125 | Insufficient Volume | - | - |
|  | MW09-16 | 387992 | 6881094 | Good |  | Duplicate |
|  | MW09-17 | 388075 | 6880970 | Good |  | - |
|  | MW09-18 | 388054 | 6880986 | Good |  | - |
|  | MW09-19 | 388051 | 6881016 | Good |  | - |
| Brown McDade Pit | CH-P-13-01/10 | 388657 | 6881116 | Frozen | - | - |
|  | CH-P-13-03/10 | 389145 | 6881105 | Damaged2 | - | - |
|  | CH-P-13-03/50 | 389143 | 6881105 | Good |  | - |
|  | CH-P-13-04/10 | 389138 | 6881472 | Insufficient Volume | - | - |
|  | CH-P-13-04/35 | 389138 | 6881472 | Obstruction2 | - | - |
|  | CH-P-13-05/50 | 388954 | 6881466 | Good |  | Duplicate |
|  | GLL07-01 | 388851 | 6881777 | Frozen | - | - |
|  | GLL07-02 | 389069 | 6881703 | Dry | - | - |
|  | GLL07-03 | 388959 | 6881477 | Good |  | - |
|  | MP14-01 | N/A | N/A | Not installed | - | - |
|  | MW09-13 | 389006 | 6881665 | Frozen | - | - |
|  | MW09-14 | 389006 | 6881663 | Frozen | - | - |
|  | MW09-15 | 388915 | 6881723 | Frozen | - | - |
| Pony Creek | GSI-PC-01-B | 388720 | 6881918 | Destroyed2 | - | - |
|  | GSI-PC-02-B | 388907 | 6881786 | Dry | - | - |
|  | GSI-PC-03-B | 389256 | 6881706 | Insufficient Volume | - | - |
| Pony Creek | GSI-PC-04-B | 389586 | 6881660 | Insufficient Volume | - | - |
|  | GSI-PC-05-B | 389713 | 6881661 | Dry | - | - |
|  | MP09-01 | N/A | N/A | Unable to Locate | - | - |
|  | MP09-02 | 388867 | 6881816 | Good |  | - |
|  | MP09-03 | 388956 | 6881739 | Insufficient Volume | - | - |
|  | MP09-08 | 389160 | 6881718 | Good |  | - |
| Seepage Dam | W14103083BH01 | 389522 | 6880669 | Frozen | - | - |
|  | W14103083BH02 | 389561 | 6880665 | Insufficient Volume | - | - |
|  | W14103083BH04 | 389544 | 6880666 | Frozen | - | - |
| Tailings Facility | MP09-04 | 389575 | 6880609 | Good |  | - |
|  | MP09-05 | 389548 | 6880590 | Good |  | - |
|  | MP09-09 | 389240 | 6880681 | Good |  | - |
|  | MP09-10 | 389241 | 6880684 | Good |  | - |
|  | MP09-11 | 389220 | 6880614 | Good |  | - |
|  | MP09-12 | 389220 | 6880614 | Good |  | - |
|  | MP09-14 | 389138 | 6880722 | Insufficient Volume | - | - |
|  | MW09-01 | 389391 | 6880557 | Obstruction2 | - | - |
|  | MW09-02 | 389393 | 6880557 | Good |  | Duplicate |
|  | MW09-03 | 389421 | 6880555 | Good |  | - |
|  | MW09-04 | 389420 | 6880557 | Good |  | - |
|  | MW09-05 | 389413 | 6880656 | Unable to Access | - | - |
|  | MW09-06 | N/A | N/A | Unable to Locate3 | - | - |
|  | MW09-07 | 389322 | 6880699 | Good |  | - |
|  | MW09-08 | 389620 | 6880576 | Good |  | Duplicate |
|  | MW09-11 | 389037 | 6880711 | Dry | - | - |
|  | MW09-20 | 389592 | 6880586 | Dry | - | - |
|  | MW09-21 | 389536 | 6880577 | Good |  | - |
|  | MW09-22 | 389495 | 6880549 | Good |  | - |
|  | MW09-23 | 389459 | 6880553 | Good |  | - |
|  | MW09-24 | 389561 | 6880624 | Good |  | - |
|  | W14103083BH03 | 389132 | 6880730 | Insufficient Volume | - | - |

Notes:	1 	Insufficient Volume as defined by AAM where less than two litres of water could be purged from the well, and where less than two litres of water was present after allowing the well to recharge.

2 	Further details concerning damaged, degraded, or obstructed wells are provided in Section 3.2.

3 	Well MW09-06 was noted as ‘submerged’ during 2013 (as described in the Scope of Work). Based on field observations, this is the likely reason for that well not being located during the spring 2014 program.

## Sample Sites

Groundwater wells monitored during the spring event was conducted across six (6) areas of the Mount Nansen site (Table 1-1). The majority of spring well sites were located around previous infrastructure areas including the tailings facility (25 wells; including tailings facility and seepage pond/dam), the Brown McDade Pit (13 wells) and the Mill Complex (9 wells). Spring groundwater sampling was also completed via targeted drive-point piezometer installations in the vicinity of Dome and Pony Creeks (9 sample sites in each creek). Table 1-1 describes the location, status of wells and sample recovery for groundwater wells included in the spring sampling program. The well locations are also illustrated in Figures 1-2 and 1-3. Photographs of each sample site are included in Appendix A.

Figure 1-2	Groundwater Sampling Locations – Dome Creek and Tailings Facility

Figure 1-3	Groundwater Sampling Locations – Mill Complex and Brown McDade Pit

# Methodology

## Protocols

Groundwater purging, monitoring and sampling conducted by Hemmera/ELR was in accordance with the Groundwater Sampling Standard Operating Procedures included in the document Scope of Work: Groundwater Sampling Program – Mount Nansen Site June 2014. At no time during the spring program did field methodologies deviate from the prescribed sampling procedures. These procedures were consistent with Yukon Environment’s Protocol for the Contaminated Sites Regulation #7 - Sampling and Decommissioning (Yukon Environment, March 2011). Methods used were also consistent with the ASTM D4448-01 Standard Guide for Sampling Groundwater Monitoring Wells (ASTM, 2013), and the D6452-99 Guide for Purging Methods for Wells used for Groundwater Quality Investigations (ASTM, 2012).

## Well Measurements and Purging

Upon arriving at each well, headspace gases were measured prior to any other well measurements. Oxygen (%), carbon dioxide (ppm), and methane (%LEL) were measured using a RAE Systems MultiRAE Four-Gas Monitor with photoionization detector (PID).

The well structure and casing of each well were inspected for damage, closure, and general conditions. Several measurements were then recorded from each well, including Depth-to-Water (DTW; m), Depth-to-Bottom (DTB; m), well diameter (cm), and well stick-up height (m).

DTB and DTW were measured using either a Solinst - Model 102 Water Level Meter (for 2.54 cm diameter wells) or a Heron Water Tape (for wells with diameter greater than 2.54 cm). DTB and DTW were measured from (in hierarchical order): 1) a black mark drawn on the top of the well; 2) the bottom of the most significant notch found on the top of the PVC if a mark was not present; or 3) a line that was drawn on the highest point of the well if no distinguishable point of measure was present. Stick-up height was measured from the lowest point on the bottom of the well casing to the highest point (or distinguishing mark) on the well. Water level meters were rinsed between each sample site with de-ionized water.

Following initial inspection and monitoring, groundwater wells were purged and sampled using dedicated equipment including high density polyethylene (HDPE) tubing and footvalves. In some cases existing tubing found within wells was not considered to be suitable for sampling. In such cases, the existing equipment was removed and new tubing installed. New dedicated tubing was also installed where no dedicated tubing was present. Groundwater wells were purged and sampled using one of three (3) techniques: 1) Hydrolift electric pump using Waterra tubing and footvalve, 2) manual purging using Waterra tubing and footvalve, or 3) GeoPump peristaltic pump. The purging technique chosen for each well was that which would produce the most representative groundwater sample.

Groundwater wells were determined to be sufficiently purged when either three successive field parameter measurements were recorded to be within an allowable tolerance level (as summarized in Table 2-1, below) or when a volume of water equivalent to three standing well volumes of water had been purged. Numerous groundwater wells were found to have a limited standing volume or recharge at the time of sampling, which was communicated to AAM during the field program. Instances where less than two litres of water could be purged from the well initially or where less than two litres of water was present after allowing the well to recharge, the well was determined to have an insufficient volume of water for sampling purposes .

Groundwater turbidity (NTU) was also measured prior to sampling (described below in Section 2.3) and was used as an indication of sample quality. Where possible samples were not collected until turbidity levels were less than 50 NTU. Purge volumes and purge rates were measured using a graduated container and stop watch.

All well measurements, purging details, and additional field notes were recorded on customized field forms in order to minimize the potential for field errors; this information is presented in Table 3-1.

Table 2-1	Groundwater Sampling – Field Parameter Purging Criteria

| Field Parameter | Allowable Variance |
| --- | --- |
| Temperature (°C) | 3% |
| pH | +0.1 |
| Conductivity (µS/cm) | 3% |
| Specific Conductivity (µS/cm) | 3% |

## Field Parameters

Hemmera/ELR measured in-situ water quality parameters using YSI Professional Plus and field meters, Lamotte 2020e or Hach 2100Q turbidity meters, and Hach DR 2800 Portable Spectrophotometers. Flow-through cells were used in conjunction with the YSI Professional Plus meters to minimize field parameter variability. The in-situ water quality parameters recorded at each sample site included; water temperature (oC), specific conductivity (μs/cm), conductivity (μs/cm), ORP (mv), and pH (pH units), sulphide (µg/l), and turbidity (NTU).

During purging, field parameters were monitored at 5 minute intervals, or at volume related intervals (e.g., every 500 mL) in the case of wells with slow recharge. A final set of measurements was recorded at the conclusion of purging.

## Groundwater Sampling

Groundwater quality samples were collected and preserved in accordance with laboratory directions, and using techniques consistent with Standard Methods for the Examination of Water and Wastewater (Rice et al., 2012). ALS Global was the analytical subcontractor chosen for this project, and an example summary of the sample set collected at each sample site, including parameters analysed and preservation techniques, is provided in Table 2-2.

Table 2-2	Groundwater Sampling – Preservation and Intended Analysis

| Bottle Type | Parameters Analyzed | Sample Treatment | Preservative Added |
| --- | --- | --- | --- |
| 1 L (plastic) | General Chemistry | - | - |
| 120 ml (plastic) | Dissolved Metals | Field Filtered and Preserved | HNO3 |
| 120 Ml (glass vial) | Dissolved Mercury | Field Filtered and Preserved | HCl |
| 250 ml (glass amber) | NH3 | Preserved | H2SO4 |
| 250 ml (glass amber) | TIC – total inorganic carbon | - | - |
| 120 ml (plastic) | Sulphide | Preserved | Zinc Acetate, capped and mixed, then NaOH |
| 120 ml (plastic) | Thyocyanate | Preserved | HNO3 |
| 145 ml (plastic red cap) | Cyanide (total, free, and WAD) | Preserved | NaOH |
| 145 ml (plastic red cap) | Cyanide (total, free, and WAD) | Preserved | NaOH |

## Data Analysis

Groundwater analytical results were compared to the Canadian Council of Ministers of the Environment (CCME) Water Quality Guidelines for the Protection of Freshwater Aquatic Life (FAL; CCME, 1999). All relevant CCME FAL guidelines are presented in Table A.

## Quality Assurance and Quality Control

### Field QA/QC

Several controls were used by Hemmera/ELR staff while in the field to help ensure that sample integrity was maintained and that data were recorded completely and accurately. All equipment used during the sampling process was dedicated to individual wells, including tubing and Waterra footvalves. The project laboratory provided pre-cleaned sample containers, disposable filters, and disposable syringes. Field staff wore dedicated disposable nitrile gloves for all measurements, purging, and sampling. Water level meters were cleaned using de-ionized water between well locations, and field instruments (YSI field meters and turbidity meters) were checked and/or calibrated before each site visit to ensure the parameters recorded were as accurate as possible.

Project-specific field data sheets were created for the sampling event to help ensure that all required measurements were taken, and that information was recorded correctly. Field data sheets have been included as Appendix B of this report.

### QA/QC

Analytical QA/QC measures were included in the spring sampling program as outlined in the scope of work and as per standard industry practice. This included the collection of travel blanks, duplicates, and field blanks. Duplicate samples were collected at a minimum rate of 10% of the regular sample collection rate (4 duplicates were collected in relation to 28 sample sites), and a field blank was collected for each day field sampling was conducted (a total of 4 field blanks were collected). Two travel blank accompanied the analytical supplies and samples from the lab to the field and back to the lab again (1 for each shipment).

The variation in sample and sample duplicate values is represented as relative percent difference (RPD). RPD provides a measure of the relative difference between two values in comparison to their mean value, and is calculated as the difference between a sample and its field duplicate over the average of two values. RPD values greater than 20% indicate a potential error that has affected the data precision. RPD was calculated according to the following formula:

%RPD=x1-x2x1+x22 X 100

RPD is not calculated if either the sample or the field duplicate concentration is less than five times the detection limit.

The analytical results for field and travel blanks were reviewed to determine whether any of the parameters tested were detected (i.e., result exceeding the detection limit). In such cases, the parameter or element in question and its concentration were reviewed to determine potential sources of contamination or error.

# Results

Summary tables of the laboratory analytical results were prepared and are presented in Table A of this report. This table includes a comparison of results to CCME FAL guidelines. A summary of the QA/QC sampling results is also attached as Table B, including analytical data for duplicates, field blanks, and travel blanks, and RPD values. Copies of original laboratory analytical reports are provided as Appendix C.

## Groundwater Sampling Summary

Groundwater sampling was completed between June 26 and June 29, 2014. Weather conditions varied throughout the time of sampling with ambient air temperature ranging from 8 to 24°C. Weather conditions were pre-dominantly clear and sunny with periods of overcast conditions. Of the 65 groundwater wells included in the spring sampling event, 60 were located and monitored during the sampling event. Of the five wells listed in the scope of work that were not assessed, two were not located (MP09-01 and MW09-06), one (GSI-PC-01-B) was found destroyed, one (MW09-05) was not accessible (as it was in the tailings pond), and one (MP14-01) had not yet been installed. Of the 60 monitored wells, groundwater samples were collected from 28 for laboratory analysis of select parameters.

There were several reasons for a low sampling rate during the program: eight (8) wells were frozen, five (5) wells were dry, sixteen (16) wells had insufficient groundwater volume to sample (minimal volume and recharge rate, according to criterial provided to Hemmera/ELR by AAM),and four (4) wells were either damaged or had an obstruction in the well, three (3) wells were either inaccessible or not located, and one (1) well listed in the SOW was not installed at the time of sampling. A summary of the condition (status) of groundwater wells is provided in Table 1-1, which also indicates where samples were successfully collected. A summary of all well measurements, purge details, and field parameter results is provided in Table 3-1.

| Table 3-1 Groundwater Field Parameters and Well Measurements for 2014 Spring Sampling Program |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Area | Location ID | Sample Date (yyyy-mm-dd) | Stick up Height (m) | Depth To Water (m)1 | Depth to Bottom (m) | Standing Water Volume (L) | Volume Purged (L) | Purge Start Time | Purge End Time | Elapsed Purge Time | Purge Rate (l/min) | Criteria1 (3WV/PS) | Draw Down (m) | pH | Temperature (ºC) | Specific Conductance (µS/cm) | Conductivity, Uncorrected (µS/cm) | Redox, Uncorrected (mV) | Dissolved Oxygen (mg/L) | Dissolved Sulphide (µg/L) | Methane (%LEL) | Oxygen (%) | Carbon Dioxide (ppm) | Field Turbidity (NTU) | Method Used | Well Diameter (inches) |
| Dome Creek | GSI-DC-01A | 2014-06-26 | 0.930 | 0.901 | 1.447 | 3.330 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 1 |
|  | GSI-DC-01B | 2014-06-26 | 0.950 | 1.527 | 1.574 | 0.290 | <0.5 | 18:15 | 18:16 | 0:01 |  | - | - | - | - | - | - | - | - | - | - | - | - | - | Peristaltic | 1 |
|  | GSI-DC-02A | 2014-06-27 | 0.925 | 1.596 | 1.950 | - | - | - | - | - |  | - | - | - | - | - | - | - | - | - | 0 | 20.8 | 610 | - | - | 1 |
|  | GSI-DC-02B | 2014-06-27 | 0.880 | 2.487 | 3.840 | 0.690 | <0.5 | 7:21 | 7:26 | 0:05 |  | - | - | 7.54 | 2.6 | 320 | 291.4 | -61.1 | 3.02 | - | 0 | 20.6 | 600 | - | Peristaltic | 1 |
|  | GSI-DC-03A | 2014-06-27 | 0.910 | 1.208 | 1.325 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 610 | - | - | 1 |
|  | GSI-DC-03B | 2014-06-27 | 0.910 | 1.533 | 2.405 | 0.440 | <0.5 | 9:29 | 9:33 | 0:04 |  | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 610 | - | - | 1 |
|  | GSI-DC-05A | 2014-06-27 | 1.040 | 1.135 | 1.937 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.4 | 690 | - | - | 1 |
|  | GSI-DC-05B | 2014-06-27 | 0.550 | 1.120 | 2.805 | 0.860 | <0.5 | 10:40 | 10:46 | 0:06 |  | - | - | - | - | - | - | - | - | - | 0 | 20.4 | 690 | - | - | 1 |
|  | GSI-DC-06A | 2014-06-29 | 0.870 | 0.977 | 1.755 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 1 |
|  | GSI-DC-06B | 2014-06-27 | 0.510 | 0.855 | 1.394 | 0.270 | 1.2 | 14:35 | 14:39 | 0:04 | 0.30 | 3WV | - | 7.10 | 8.1 | 402 | 250 | -78.5 | 0.66 | 32 | 0 | 21 | 520 | 12.81 | Peristaltic | 1 |
|  | GSI-DC-07A | 2014-06-29 | 0.940 | 1.325 | 1.985 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 0 | - | - | 1 |
|  | GSI-DC-07B | 2014-06-29 | 0.930 | 1.323 | 1.925 | 0.310 | 3.0 | 15:45 | 16:12 | 0:27 | 0.11 | 3WV | - | 7.03 | 2.8 | 1088 | 628 | -84.4 | 0.42 | 1179 | 0 | 20.6 | 0 | 11.79 | Peristaltic | 1 |
|  | GSI-DC-08-A | 2014-06-29 | 0.910 | 1.121 | 1.534 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 550 | - | - | 0.5 |
|  | GSI-DC-08-B | 2014-06-29 | 0.270 | Frozen | 0.759 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 640 | - | - | 0.5 |
|  | GSI-DC-09-A | 2014-06-29 | 0.910 | 1.094 | 1.359 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 470 | - | - | 0.5 |
|  | GSI-DC-09-B | 2014-06-29 | - | 1.262 | 3.858 | 0.625 | 1.1 | 12:18 | 12:27 | 0:09 | 0.12 | 3WV | 2.74 | 6.60 | 3.9 | 1940 | 1151 | 63.5 | 0.23 | 32 | 0 | 20.5 | 530 | 27.6 | Peristaltic | 0.5 |
|  | GSI-DC-10-A | 2014-06-29 | 1.040 | - | 1.809 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 520 | - | - | 0.5 |
|  | GSI-DC-10-B | 2014-06-29 | 1.030 | 0.981 | 3.763 | 0.900 | 2.8 | 14:20 | 14:35 | 0:15 | 0.19 | 3WV | 1.05 | 6.60 | 3.2 | 1212 | 709 | 70.5 | 0.1 | 40 | 0 | 20.5 | 440 | 13.9 | Peristaltic | 0.5 |
| Mill Complex | GSI-HA-01A | 2014-06-27 | 1.220 | 2.380 | 3.120 | 0.380 | <0.5 | 7:41 | 7:45 | 0:04 |  | - | - | 7.38 | 3 | 921 | 531 | -54.2 | 4.9 | - | 0 | 20.9 | 630 | - | - | 1 |
|  | GSI-HA-02A | 2014-06-27 | 1.490 | 2.414 | 2.480 | 0.034 | - | - | - | - |  | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 600 | - | - | 1 |
|  | GSI-HA-03A | 2014-06-27 | 0.930 | 1.512 | 2.170 | 0.330 | <0.5 | 8:05 | 8:11 | 0:06 |  | 3WV | - | 6.84 | 3.5 | 990 | 586 | -60.6 | 1.83 | - | 0 | 20.9 | 610 | - | Peristaltic | 1 |
|  | GSI-HA-04A | 2014-06-27 | 0.595 | 2.050 | 2.133 | 0.042 | - | - | - | - |  | - | - | - | - | - | - | - | - | - | 0 | 19.3 | 3100 | - | - | 1 |
|  | GSI-HA-05A | 2014-06-27 | 0.960 | 1.375 | 1.770 | 0.200 | <0.5 | 8:36 | 8:37 | 0:02 |  | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 620 | - | Peristaltic | 2 |
|  | MW09-16 | 2014-06-26 | 1.220 | 1.686 | 2.680 | 2.020 | 7.5 | 13:15 | 13:40 | 0:25 | 0.30 | 3WV | 1.69 | 6.67 | 4.8 | 1695 | 1040 | 136.6 | 3.62 | 3 | 0 | 20.6 | 4.38 | 3.86 | Peristaltic | 2 |
|  | MW09-17 | 2014-06-29 | 0.970 | 4.778 | 5.610 | 1.690 | 3.0 | 10:55 | 11:06 | 0:11 | 0.27 | 3WV | - | 6.80 | 2.6 | 2788 | 1594 | 124 | 0.08 | 14 | - | - | - | 3.47 | Peristaltic | 2 |
|  | MW09-18 | 2014-06-26 | 0.900 | 4.555 | 7.770 | 6.530 | 20.0 | 15:57 | 16:56 | 1:01 | 0.33 | 3WV | 4.57 | 7.01 | 1.5 | 1434 | 2590 | 31.3 | 0.8 | 42 | 0 | 20.6 | 2.76 | 6.49 | Peristaltic | 2 |
|  | MW09-19 | 2014-06-26 | 0.990 | 2.527 | 5.870 | 6.793 | 20.0 | 14:45 | 15:33 | 0:48 | 0.42 | 3WV | 3.25 | 6.76 | 1.6 | 2327 | 1285 | -86.7 | 2.19 | 125 | 0 | 20.6 | 2.92 | 2.46 | Peristaltic | 2 |
| Brown-McDade Pit | CH-P-13-01 | 2014-06-27 | 0.500 | Frozen | 2.630 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 640 | - | - | 2 |
|  | CH-P-13-03/10 | 2014-06-27 | 0.685 | Damaged | 5.136 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 490 | - | - | 2 |
|  | CH-P-13-03/50 | 2014-06-27 | 0.606 | 48.454 | 50.762 | 1.150 | 1.0 | 16:40 | 16:50 | 0:10 | 0.10 | 3WV | - | - | - | - | - | - | - | - | 0 | 20.4 | 1390 | 70.1 | 1" bailer | 1 |
|  | CH-P-13-04/10 | 2014-06-28 | 0.638 | 2.928 | 2.976 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 440 | - | - | 2 |
|  | CH-P-13-04/35 | 2014-06-28 | 0.608 | Obstruction | 6.505 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 690 | - | - | 1 |
|  | CH-P-13-05/50 | 2014-06-27 | 0.880 | 25.595 | 50.470 | 12.640 | 60.0 | 16:25 | 17:12 | 0:47 | 1.28 | 3WV | 26.85 | 6.27 | 2.6 | 2864 | 1640 | 122.4 | 2.53 | 434 | 0 | 20.9 | 1.42 | 24.1 | Hydrolift | 1 |
|  | GLL07-01 | 2014-06-26 | 0.810 | Frozen | 12.876 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 1.4 | - | - | 2 |
|  | GLL07-02 | 2014-06-28 | 1.370 | Dry | 7.120 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 19.8 | 3840 | - | - | 6 |
|  | GLL07-03 | 2014-06-27 | 1.150 | 10.031 | 11.745 | 3.483 | 5.0 | 15:15 | 15:39 | 0:24 | 0.21 | 3WV | 10.89 | 6.19 | 4.5 | 1659 | 1014 | 103.7 | 5.32 | 95 | 0 | 20.9 | 1.42 | 22 | Manual | 2 |
|  | MW09-13 | 2014-06-27 | 0.760 | Frozen | 8.995 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.1 | 2540 | - | - | 2 |
|  | MW09-14 | 2014-06-27 | 0.750 | Frozen | 5.098 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.4 | 550 | - | - | 2 |
|  | MW09-15 | 2014-06-26 | 0.910 | Frozen | 14.004 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 1.41 | - | - | 2 |
| Pony Creek | GSI-PC-01-A | 2014-06-27 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-01-B | 2014-06-27 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-02-A | 2014-06-27 | - | 0.919 | 1.297 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 460 | - | - | 0.5 |
|  | GSI-PC-02-B | 2014-06-27 | - | Dry | 1.285 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 490 | - | - | 0.5 |
|  | GSI-PC-03-A | 2014-06-28 | 0.920 | 1.095 | 2.006 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 470 | - | - | 0.5 |
|  | GSI-PC-03-B | 2014-06-28 | 0.900 | 1.398 | 2.825 | 0.181 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.4 | 890 | - | - | 0.5 |
|  | GSI-PC-04-A | 2014-06-28 | 0.890 | - | 1.262 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 490 | - | - | 0.5 |
|  | GSI-PC-04-B | 2014-06-28 | 0.920 | 1.888 | 2.586 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 560 | - | - | 0.5 |
|  | GSI-PC-05-A | 2014-06-28 | 0.920 | - | 1.306 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 470 | - | - | 0.5 |
|  | GSI-PC-05-B | 2014-06-28 | 0.910 | Dry | 3.751 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.2 | 730 | - | - | 0.5 |
|  | MP09-01 | 2014-06-28 | - | Unable to locate | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | MP09-02 | 2014-06-27 | 1.360 | 1.228 | 1.970 | 0.100 | 2.5 | 9:45 | 10:38 | 0:53 | 0.05 | 3WV | - | 7.22 | 3.3 | 522 | 304.9 | 75.8 | 5.4 | 24 | 0 | 20.5 | 450 | 1.96 | Peristaltic | 0.5 |
|  | MP09-03 | 2014-06-27 | 1.300 | 1.519 | 1.617 | 0.012 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 430 | - | - | 0.5 |
|  | MP09-08 | 2014-06-27 | 1.500 | 0.892 | 1.975 | 0.125 | 2.2 | 14:10 | 14:40 | 0:30 | 0.07 | 3WV | - | 7.12 | 2.9 | 720.3 | 416.6 | -95.5 | 1.64 | 124 | 0 | 20.6 | 490 | 1.02 | Peristaltic | 0.5 |
| Seepage Dam | W14103083BH01 | 2014-06-28 | 0.640 | Frozen | 6.646 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 21 | 521 | - | - | 2 |
|  | W14103083BH02 | 2014-06-28 | 0.800 | 6.897 | 7.920 | 2.070 | 0.5 | 8:49 | 8:54 | 0:05 | 0.10 | - | - | 7.46 | 3.4 | 2436 | 1429 | -46.2 | 1.6 | - | 0 | 21.3 | 602 | - | Peristaltic | 2 |
|  | W14103083BH04 | 2014-06-28 | 0.800 | Frozen | 6.730 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 520 | - | - | 2 |
| Tailing Facility | MP09-04 | 2014-06-28 | 1.200 | 2.034 | 3.070 | 2.110 | 7.0 | 11:04 | 11:39 | 0:35 | 0.20 | 3WV | 2.15 | 7.04 | 3.6 | 1610 | 959 | 58.2 | 4.09 | 12 | 0 | 20.5 | 630 | 1.87 | Peristaltic | 2 |
|  | MP09-05 | 2014-06-28 | 1.200 | 1.414 | 1.820 | 0.450 | 1.5 | 16:47 | 16:59 | 0:12 | 0.13 | 3WV | 1.58 | 6.75 | 8.5 | 2380 | 1723 | -41.4 | 4.1 | 16 | 0 | 21.1 | 550 | 5.05 | Peristaltic | 1.5 |
|  | MP09-09 | 2014-06-29 | 2.240 | 2.924 | 5.630 | 2.980 | 3.5 | 9:15 | 9:30 | 0:15 | 0.23 | 3WV | 5.3 | 9.66 | 5.8 | 275.4 | 174.3 | -23.1 | 0.25 | 651 | 0 | 20.9 | 520 | 61 | 1" bailer | 1.5 |
|  | MP09-10 | 2014-06-29 | 1.980 | 2.730 | 4.540 | 1.991 | 4.0 | 9:02 | 9:13 | 0:11 | 0.36 | 3WV | - | 9.21 | 8.2 | 415.4 | 283 | -46.7 | 7.04 | - | 0 | 20.9 | 520 | - | 1" bailer | 1.5 |
|  | MP09-11 | 2014-06-29 | 1.740 | 2.211 | 4.950 | 3.010 | 3.5 | 8:09 | 8:29 | 0:20 | 0.18 | 3WV | 4.2 | 7.54 | 5.1 | 804.7 | 498.7 | -146.2 | 0.37 | 411 | 31 | 19.7 | 830 | 34 | 1" bailer | 1.5 |
|  | MP09-12 | 2014-06-29 | 1.700 | 2.229 | 4.175 | 2.140 | 2.5 | 7:55 | 8:05 | 0:10 | 0.25 | 3WV | 3.6 | 7.46 | 6.5 | 518 | 333.9 | -91.7 | 5.93 | 279 | 0 | 20.9 | 580 | 82.7 | 1" bailer | 1.5 |
|  | MP09-14 | 2014-06-27 | 1.070 | 1.342 | 1.971 | 0.080 | <0.1 | 17:36 | 17:38 | 0:02 |  | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 470 | - | - | 0.5 |
|  | MW09-01 | 2014-06-27 | 0.820 | 2.946 | 8.680 | 11.650 | - | - | - | - |  | - | - | - | - | - | - | - | - | - | 0 | 20.6 | 650 | - | - | 2 |
|  | MW09-02 | 2014-06-27 | 0.750 | 2.021 | 4.705 | 5.440 | 17.0 | 12:55 | 13:33 | 0:27 | 0.63 | 3WV | 3.24 | 7.13 | 4.7 | 2670 | 1670 | -92.1 | 0.14 | 23 | 0 | 20.6 | 650 | 16.9 | Peristaltic | 2 |
|  | MW09-03 | 2014-06-27 | 0.560 | 4.787 | 9.930 | 10.450 | 32.0 | 15:15 | 16:07 | 0:52 | 0.62 | 3WV | 5.15 | 7.22 | 3.5 | 2526 | 1492 | -24.1 | 0.76 | 17 | 0 | 20.9 | 601.3 | 1.58 | Peristaltic | 2 |
|  | MW09-04 | 2014-06-27 | 0.500 | 2.697 | 7.670 | 10.110 | 30.0 | 14:06 | 15:08 | 1:02 | 0.48 | 3WV | 5.01 | 8.64 | 3.6 | 2666 | 154.3 | -147.3 | 0.28 | 27 | 0 | 20.6 | 602 | 5.68 | Peristaltic | 2 |
|  | MW09-05 | 2014-06-28 | - | Unable to access | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | MW09-06 | 2014-06-28 | - | Unable to locate | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | MW09-07 | 2014-06-29 | 1.350 | 2.461 | 3.397 | 1.900 | 1.5 | 16:35 | 16:45 | 0:10 | 0.15 | 3WV | 3.222 | 6.80 | 5.7 | 2411 | 1542 | 102.4 | 3.1 | 294 | 0 | 20.9 | 460 | 32.2 | Peristaltic | 2 |
|  | MW09-08 | 2014-06-28 | 1.080 | 1.340 | 3.905 | 5.210 | 18.0 | 11:55 | 12:40 | 0:45 | 0.40 | 3WV | 1.45 | 6.82 | 3.3 | 381.2 | 216.7 | -96.7 | 1.41 | 92 | 0 | 20.0 | 520 | 2.02 | Peristaltic | 2 |
|  | MW09-11 | 2014-06-29 | 0.820 | Dry | 4.909 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.5 | 440 | - | - | 2 |
|  | MW09-20 | 2014-06-28 | 0.910 | Dry | 3.670 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.4 | 1100 | - | - | 2 |
|  | MW09-21 | 2014-06-28 | 0.720 | 1.568 | 3.576 | 4.080 | 14.0 | 17:17 | 17:55 | 0:38 | 0.37 | 3WV | 2.02 | 6.82 | 2 | 2544 | 1430 | -64.8 | 1.91 | 63 | 0 | 20.6 | 550 | 11.1 | Peristaltic | 2 |
|  | MW09-22 | 2014-06-27 | 0.880 | 4.194 | 5.235 | 2.120 | 6.5 | 16:19 | 16:44 | 0:45 | 0.14 | 3WV | 5.05 | 5.82 | 3.7 | 2073 | 1684 | 58.3 | 2.58 | 26 | 4 | 2.05 | 950 | 10.54 | Peristaltic | 2 |
|  | MW09-23 | 2014-06-27 | 0.940 | 11.913 | 15.805 | 7.900 | 30.0 | 11:49 | 12:23 | 0:34 | 0.88 | 3WV | 12.16 | 7.12 | 2.1 | 2612 | 1468 | -64.7 | 4.32 | 90 | 0 | 20.6 | 620 | 30.1 | Waterra | 2 |
|  | MW09-24 | 2014-06-28 | 0.680 | 9.389 | 11.170 | 3.620 | 12 | 9:3 | 10:00 | 0:25 | 0.48 | 3WV | 10.04 | 7.34 | 2.6 | 1198 | 761 | 3.3 | 5.77 | 84 | 0 | 20.5 | 610 | 11.3 | Manual Waterra | 2 |
|  | W14103083BH03 | 2014-06-27 | 0.760 | 1.842 | 1.942 | 0.200 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 440 | - | - | 2 |
|  | CH-P-13-02 | 2014-06-27 | - | 8.125 | 8.200 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

Notes:	1 	Depth to Water (DTW) values for frozen or damaged wells indicates depth to ice or blockage.

## Analytical Results

The following section provides a summary and brief discussion of the program analytical results, including a brief summary of CCME FAL guideline exceedances and a description of factors that may have influenced results. Details regarding well status, including a description of damaged, destroyed, or underperforming wells, are also discussed. In numerous instances, the reported laboratory method detection limits (MDL) for parameters exceeded applicable CCME FAL standards (values italicized and shaded light grey in Table A). In these cases, samples having high levels of certain materials required laboratory dilution in order to perform the required analyses, and thereby resulting in an elevated MDL. For the purpose of this report, samples where the reported MDL is higher than the applicable guideline have not been reported as CCME FAL exceedances.

### Dome Creek

Groundwater along Dome Creek was sampled between June 27 and June 29, 2014. Samples were obtained from four (4) of the nine (9) drive-point piezometers located within this area identified for the sampling program. Sample sites GSI-DC-01B, GSI-DC-02B, GSI-DC-03B, GSI-DC-05B produced insufficient groundwater volume to obtain a sample. The remaining site, drive-point GSI-DC-08-B, was frozen at the time of the sampling program.

Concentrations of dissolved aluminum, arsenic, chromium, and iron exceeded the CCME FAL guidelines at one or more sample location in the Dome Creek area (Table A). Concentrations of ammonia also exceeded the CCME FAL guidelines in various sample locations.

The measurement of in-situ headspace vapours was made difficult at the Dome Creek sample sites due to dedicated sampling tubing being present in these small diameter wells. There was no space in the well head to sample vapours until dedicated sampling equipment was removed, after which time well head gases may have dispersed.

The turbidity of all samples collected within the Dome Creek area was less than 50 NTU (Table 3-1).

### Mill Complex

Groundwater in the Mill Complex Area was sampled between June 26 and June 29, 2014. Samples were obtained from four (4) of the nine (9) wells identified in this area. Drive-points GSI-HA-01A, GSI-HA-02A, GSI-HA-03A, GSI-HA-04A, and GSI-HA-05A produced insufficient groundwater volumes to be sampled.

Concentrations of dissolved arsenic, cadmium, copper, iron and zinc exceeded the CCME FAL guidelines at one or more sample location in Mill Complex area.

Monitoring well MW09-17 was damaged at the surface during the sampling program. This was communicated to AAM, who approved an on-site repair during the program. Accordingly, the well was repaired by attaching a coupling and new well pipe, and installing a new bentonite seal (see photos 15-21, in Appendix A). The steel monument was also re-installed and secured with concrete. The well was then re-developed (purged until almost dry) the following morning (~25L purged before parameters or sampling took place). Finally, sampling was completed once the well had time to settle. Well head gases were not measured due to the repair of this well.

Monitoring well MW09-18 had vents installed on the side of the PVC stand pipe, which is likely to have influenced in-situ gas concentrations.

The turbidity of all samples collected within this area was less than 50 NTU (Table 3-1).

### Brown McDade Pit

Groundwater wells in the Brown McDade Pit Area were sampled between June 27 and June 28, 2014. Samples were obtained from three (3) of the 13 sample sites located within this area. Five (5) wells were frozen during the time of sampling (CH-P-13-01, GLL07-01, MW09-13, MW09-14 and MW09-15), one well was damaged (CH-P-13-03/10; broken at the surface), one well had an obstruction affecting the ability to sample (CH-P-13-04/35), one (1) well had insufficient well volume to sample (CH-P-13-04/10), and one (1) well was dry during the time of sampling (GLL07-02). Well MP14-01 was listed as a sample site in the scope of work but had not been installed at the time of sampling.

CH-P-13-03/10 was found to be broken at the top coupler of the PVC pipe. The wells casing material (sand) was missing and presumed to have fallen into the well, as evidenced by the DTB measurement of 5.136 m being less than that previously documented (10 m). CH-P-13-04/35 was found to have a blockage at 6.505 m below the surface. Based on the sound and feel of vibration on the water level meter, it appeared to the crew as though it may be a plastic obstruction like a bailer, but this could not be confirmed.

Concentrations of dissolved cadmium, copper, iron and zinc exceeded the CCME FAL guidelines at one or more sample location in the Brown McDade Pit area.

The crew had some difficulty confirming the location of well CH-P-13-03/50. Based on UTM coordinates and by the marking CH-P-13-03 on the well cap, the correct well was assumed, although the outer casing of the well was labeled as CH-P-13-02. Field parameters were not measured at site CH-P-13-03/50 due to inability to purge, sample collected with 1" disposable bailer the following day.

Wells CH-P-13-05/50 and GLL07-03 were not properly sealed (no PVC caps or J-plugs were observed during initial inspection). These caps could not be replaced during this sampling event, but the replacement of caps has been included in the recommendations of this report. Also, groundwater turbidity in CH-P-13-03/50 was measured 70.1 NTU at the time of sample collection which is considered to be higher than optimal. The turbidity of CH-P-13-05/50 and GLL07-03 were within an acceptable range during the start of sample collection but became extremely cloudy during the collection of the general chemistry and dissolved metals samples.

### Pony Creek

Groundwater wells along Pony Creek were sampled on June 27, 2014. Samples were obtained from two (2) of the nine (9) sample sites in this area during the sampling event. Two (2) wells were dry during the time of sampling (GSI-PC-02-B and GSI-PC-05-B), three (3) drive-points did not produce sufficient volume (GSI-PC-03-B, GSI-PC-04-B, and MP09-03), one (1) well was not located (MP09-01), and one (1) was destroyed by placer mining operations (GSI-PC-01-B).

Concentrations of dissolved arsenic and iron exceeded the CCME FAL guidelines at one or more sample location in this area.

It should be noted that UTM coordinates provided in the scope of work show sample sites MP09-01 and MP09-08 at the same location. Only a single drive-point was found by Hemmera/ELR at this location (MP09-08) and no site marked MP09-01 was identified either upstream or downstream of this site. Sample site GSI-PC-01-A/B was found removed from the stream bed in a location currently being excavated by placer mining operations (as expected based on information provided by AAM).

The turbidity of all samples collected within this area was less than 50 NTU (Table 3-1).

### Seepage Dam

Groundwater wells in the Seepage Dam area were not sampled during the 2014 spring sampling event. Sample sites W14103083BH01 and W14103083BH04 were frozen at the time of sampling, and site W14103083BH02 did not produce sufficient sample volume.

### Tailings Facility

Groundwater wells in the Tailings Facility area were sampled between June 27 and June 29, 2014. Samples were obtained from 15 of the 22 sample sites located in this area.

Two (2) wells were dry during the time of sampling (MW09-11 and MW09-20), two (2) wells produced insufficient sample volume (MP09-14 and W14103083BH03), one (1) well was not located (MW09-06), one (1) well was not accessible as it was within the wetted tailings pond (MW09-05), and one (1) well had an obstruction that limited the ability to sample (MW09-01). Well MW09-06 had been listed in the SOW as not sampled in the fall of 2013 because it was submerged in the tailings pond, and this was assumed to be the reason that this well could not be located during the June 2013 program. At well MW09-01, an obstruction was noted at approximately 3m below the ground surface, and it was not possible to lower either 3/8” or 3/16” diameter tubing to the level of the water. The tubing lowered into the well had sediment on its end, suggesting that there is sediment/soil built up at this obstruction.

Concentrations of dissolved arsenic, cadmium, chromium, copper, iron, mercury, selenium, silver and zinc exceeded the CCME FAL guidelines at one or more sample location in this area. Groundwater pH in the tailings facility area was also outside of CCME FAL guidelines at sample site MP09-10. Concentrations of Ammonia in groundwater also exceeded the CCME FAL guidelines at several locations.

Wells MP09-09, MP09-10, MP09-12, and MW09-07 had vents installed on the side of the PVC well, which likely influenced in-situ gas measurements. Groundwater turbidity of samples collected from MP09-09 and MP09-12 were greater than the target limit of 50 NTU for sampling (61 and 82.7 NTU), indicating that suspended solids could potentially affect the sample quality. Field turbidity and in-situ sulphide was not measured in samples collected from MP09-10 due extremely high observed turbidity at the time of sampling. The turbidity of all other samples collected within this area was less than 50 NTU (Table 3-1).

## Quality Assurance and Quality Control Results

A total of four (4) duplicate groundwater samples were collected during the spring sampling event. Two travel blanks were provided by the laboratory and accompanied the samples throughout the sampling program. One field blank was prepared on site for each consecutive day of sampling (4 field blanks in total). Detailed results of QA/QC sampling is provided in Table B, including RPD values for all duplicate and sample pairs collected.

Field blank and travel blank analytical results were reported as less than the MDL for all analysed parameters with the exception of a single detection of Molybdenum and Manganese in one sample (FB1). Both reported values were very close to detection limits (< two times MDL) and are not considered to represent sampling or laboratory error.

The RDP value for total cyanide between DUP-3 and MW09-02 was 51.6%, which exceeded the desired limit and suggests a potential error affecting data precision. In the field it was recorded that the PVC cap at site MW09-02 did not fit properly on the well, but no other condition or occurrence that would affect data precision was recorded in the field. The RPD for other parameters including other cyanide analysis was less than 20%; thus there does not seem to be a systemic bias. Sample variation is considered to be the likely cause of the single variable result for cyanide.

All other RDP values were within an acceptable range of variability (less than 20%).

# Recommendations

Hemmera/ELR has prepared the following recommendations based on the observations and results of the spring 2014 groundwater sampling program.

- All groundwater wells should be properly sealed with PVC caps or J-plugs. Wells without caps have risk of becoming contaminated which may affect data precision or quality.

- Damaged or degraded wells should be repaired. This includes wells where an obstruction is restricting ability to sample the well. Damaged or degraded wells include the following; CH-P-13-03/10, CH-P-13-04/35, and MW09-17. As stated in Section 3.2.2, well MW09-17 was found broken at the surface during the time of sampling. The well was repaired and re-developed (purged until almost dry) during the spring 2014 field event, although it should be re-developed the following season to ensure the well screen is free of fine particles.

- Many of the drive-point piezometers included in the spring sampling event did not produce sufficient volumes necessary for sample collection. Issues with ice build-up were also observed at the drive-point sample locations. These sites should be re-developed and potentially re-installed if purge volumes do not improve. Alternatively, drive-point sites could be sampled later in the season (potentially early July) in order to ensure drive-points are free of ice.

- Monitoring wells should be fitted for the measurement of in-situ headspace vapour. This would include installing PVC caps or J-plugs on each well, and addressing vents currently installed on the side of some of the PVC wells.

- To avoid inclusion of acid or alkaline-generating solids that are not representative of an equilibrium condition with groundwater, it is recommended that samples for analysis of acidity, alkalinity, and hardness be field-filtered.

- To avoid degassing of carbon dioxide, precipitation of calcium carbonate in sample bottles, and exclusion of the representative precipitate component from analysis, it is recommended that samples for analysis of alkalinity be collected in a separate bottle with zero headspace and that the laboratory be instructed to analyze the contents of the entire bottle.

# Closure

We have appreciated the opportunity of working with you on this project and trust that this report is satisfactory to your requirements. Please feel free to contact the undersigned regarding any questions or further information that you may require.

Report prepared by:

ELR

Aaron Nicholson, B.Sc.

Environmental Scientist

aaron@elr.ca

Report senior reviewed by:

ELR

Chris Jastrebaski, M.Sc., R.P.Bio., P.Biol.

Project Manager

chris@elr.ca

Report senior reviewed by:

Hemmera Envirochem Inc.

Jason Wilkins, P.Ag., EP, CSAP

Director, Land Development and Projects

jwilkins@hemmera.com

# References

ASTM Standard D4448-01. 2013. Standard Guide for Sampling Groundwater Monitoring Wells. ASTM International, West Conshohocken, PA, 2013, www.astm.org.

ASTM Standard D6452-99 2012 Guide for Purging Methods for Wells used for Groundwater Quality Investigations. ASTM International, West Conshohocken, PA, 2012, www.astm.org.

Canadian Council of Ministers of the Environment (CCME). 2014. Canadian Water Quality Guidelines for the Protection of Aquatic Life. Accessed online at http://st-ts.ccme.ca/, July 2014.

Rice, E.W., Baird, R.B., Eaton, A.D., and Clesceri, L.S. 2006. Standard Methods for the Examination of Water and Wastewater. 22nd Edition. American Water Works Association.

Yukon Government. 2002. Environment Act O.I.C. 2002/171 Contaminated Sites Regulation.

# Statement of Limitations

This report was prepared by Hemmera Envirochem Inc (“Hemmera”), based on fieldwork conducted by Hemmera, for the sole benefit and exclusive use of the Yukon Government. The material in it reflects Hemmera’s best judgment in light of the information available to it at the time of preparing this Report. Any use that a third party makes of this Report, or any reliance on or decision made based on it, is the responsibility of such third parties. Hemmera accepts no responsibility for damages, if any, suffered by any third party as a result of decisions made or actions taken based on this Report.

Hemmera has performed the work as described above and made the findings and conclusions set out in this Report in a manner consistent with the level of care and skill normally exercised by members of the environmental science profession practicing under similar conditions at the time the work was performed.

This Report represents a reasonable review of the information available to Hemmera within the established Scope, work schedule and budgetary constraints. It is possible that the levels of contamination or hazardous materials may vary across the Site, and hence currently unrecognised contamination or potentially hazardous materials may exist at the Site. No warranty, expressed or implied, is given concerning the presence or level of contamination on the Site, except as specifically noted in this Report. The conclusions and recommendations contained in this Report are based upon applicable legislation existing at the time the Report was drafted. Any changes in the legislation may alter the conclusions and/or recommendations contained in the Report. Regulatory implications discussed in this Report were based on the applicable legislation existing at the time this Report was written.

In preparing this Report, Hemmera has relied in good faith on information provided by others as noted in this Report, and has assumed that the information provided by those individuals is both factual and accurate. Hemmera accepts no responsibility for any deficiency, misstatement or inaccuracy in this Report resulting from the information provided by those individuals.

The liability of Hemmera to the Yukon Government shall be limited to injury or loss caused by the negligent acts of Hemmera. The total aggregate liability of Hemmera related to this agreement shall not exceed the lesser of the actual damages incurred, or the total fee of Hemmera for services rendered on this project.

tables

Appendix A

Site Photos

Appendix B

Field Forms

Appendix C

Laboratory Reports