Mount Nansen February 2016

Groundwater Monitoring and Sampling

Prepared for:

Government of Yukon

Assessment and Abandoned Mines Branch

P.O. Box 2703

Whitehorse, YT  Y1A 2C6

Prepared by:

Hemmera Envirochem Inc.

230 – 2237 2nd Avenue

Whitehorse, YT  Y1A 0K7

and

Ecological Logistics & Research Ltd.

204 – 105 Titanium Way

Whitehorse, YT  Y1A 0E7

File: 1343-005.14

March 2016

TABLE OF CONTENTS

1.0	Introduction	1

1.1	Site Location	1

1.2	Scope of Work	2

1.3	Sample Sites	2

2.0	Methodology	8

2.1	Protocols	8

2.2	Well Measurements and Purging	8

2.3	Direct Sampling	9

2.4	Field Parameters	10

2.5	Groundwater Sampling	10

2.6	Data Analysis	11

2.7	Quality Assurance and Quality Control	11

2.7.1	Field QA/QC	11

2.7.2	Analytical QA/QC	11

3.0	Results	13

3.1	Groundwater Sampling Summary	13

3.2	Analytical Results	18

3.2.1	Dome Creek	18

3.2.2	Mill Complex	20

3.2.3	Brown McDade Pit	20

3.2.4	Pony Creek	21

3.2.5	Seepage Dam	21

3.2.6	Tailings Facility	21

3.3	Quality Assurance and Quality Control Results	22

3.3.1	Field and Travel Blanks	22

3.3.2	Field Duplicates	22

3.3.2.1	MW09-18 and MW16-100	22

3.3.2.2	MW09-04 and MW16-200	22

3.3.3	Quality Assurance and Quality Control Summary	22

4.0	Recommendations	24

5.0	Closure	25

6.0	References	26

List of Tables (within text)

Table 1-1	Summary of Groundwater Well Locations and Samples Collected	4

Table 2-1	Groundwater Sampling – Field Parameter Purging Criteria	9

Table 2-2	Groundwater Sampling Parameter Priority, Preservation, and Intended Analysis	10

Table 3-1	Summary of Direct Samples Collected During February 2016 Sampling0 Program	14

Table 3-2 	Groundwater Field Parameters and Well Measurements for February 2016 Sampling Program	15

Table 3-3	Summary of CCME FAL Guideline Exceedances for February 2016 Sampling Program	19

List of Figures

Figure 1-1	Site Location – Mount Nansen Site	3

Figure 1-2	Groundwater Sampling Locations – Dome Creek and Tailings Facility	6

Figure 1-3	Groundwater Sampling Locations – Mill Complex and Brown McDade Pit	7

List of Tables (following text)

Table A	Groundwater Sampling Analytical Results and CCME Guideline Exceedances for 2015 September Sampling Program

Table B	QA/QC Analytical Data

List of Appendices

Appendix A 	Site Photographs

Appendix B 	Field Forms

Appendix C	Laboratory Reports

Appendix D	Response to Client Comments

# Introduction

This Work was performed in accordance with Contract C00028455 between Hemmera Envirochem Inc. (“Hemmera”) and Government of Yukon (“Client”), dated May 11, 2015 (“Contract”). In performing this Work, Hemmera has relied in good faith on information provided by others, and has assumed that the information provided by those individuals is both complete and accurate. This Work was performed to current industry standard practice for similar environmental work, within the relevant jurisdiction and same locale. The findings presented herein should be considered within the context of the scope of work and project terms of reference; further, the findings are time sensitive and are considered valid only at the time the Report was produced. The conclusions and recommendations contained in this Report are based upon the applicable guidelines, regulations, and legislation existing at the time the Report was produced; any changes in the regulatory regime may alter the conclusions and/or recommendations.

Hemmera Envirochem Inc. and Ecological Logistics & Research Ltd. (Hemmera/ELR) were retained by the Government of Yukon (GY), Assessment and Abandoned Mines (AAM) to conduct a groundwater monitoring and sampling program at the Mount Nansen Site (the Site) in February, 2016. Hemmera/ELR’s scope of work included the monitoring of groundwater wells and collection of groundwater samples from a series of existing groundwater wells at the Site. This report summarizes the monitoring and sampling activities, a description of methodologies and field conditions encountered, a summary of field in-situ and laboratory analytical results including a comparison to applicable guidelines, a description of any observations and/or occurrences that may have influenced program results, and recommendations relating to sample procedures and monitoring well conditions. This report does not provide an interpretation of the results, nor does it provide recommendations relating to groundwater quality at the Site.

## Site Location

The Mount Nansen site is located approximately 45 kilometres (km) west of the Town of Carmacks (70 km by road). This Type II abandoned mine site consists of three (3) primary areas of existing infrastructure: the Brown McDade Pit, a Mill Complex, and a Tailings Facility (Figure 1-1). Groundwater monitoring wells exist throughout the Site, a subset of which were sampled during the February 2016 groundwater monitoring and sampling program. The groundwater monitoring locations included in this program are described in Sections 1.2 and 1.3.

## Scope of Work

The scope of work for this program included the coordination and execution of the February 2016 groundwater monitoring and sampling, analysis of samples, and the presentation of results in a report.

Groundwater sampling at the Site was conducted over a four (4) day period, between February 1 and February 4, 2016. Sampling was conducted by a team of four (4) qualified field staff from Hemmera/ELR (Justin Hains, Aaron Nicholson, Glenn Rudman, and Dave Clegg). A total of 65 groundwater wells were included in the February 2016 sampling event (Table 1-1). It was not possible to sample four (4) of the groundwater wells listed in the scope of work as they had been destroyed; two (2) had been previously destroyed (MP09-01 and GSI-PC-01-B) and two (2) that appeared to have been destroyed during placer mining operations in the summer of 2015 (GSI-PC-02-B and MP09-02; Hemmera, 2015a). One (1) of the remaining 61 groundwater wells assessed was known to be damaged (CH-P-13-02/10), but sampling and monitoring was still attempted during the February 2016 program.

At each well (sampling station) headspace gas concentrations were measured, well and water level parameters were measured (depth to water, depth to bottom, well diameter, and well stick-up height), the well was purged, and then prescribed in-situ groundwater quality parameters were measured. Lastly, groundwater samples were collected for laboratory analysis. A detailed description of the sampling methods and measured groundwater quality parameters is provided in Section 2.

## Sample Sites

The groundwater wells included in the February 2016 monitoring and sampling event were grouped into six (6) main areas of the Mount Nansen Site (Table 1-1). The majority of groundwater wells were located around existing infrastructure including the tailings facility and seepage dam (25 wells), the Brown McDade Pit (13 wells) and the Mill Complex (9 wells). Additional wells (primarily drive-point piezometer installations) were sampled in the vicinity of Dome Creek (9 wells) and Pony Creek (9 wells). Table 1-1 provides the location, status, and sample recovery for groundwater wells included in the February 2016 sampling program. The well locations are also illustrated in Figures 1-2 and 1-3. Photographs of each sample site visited in February 2016 are included in Appendix A.

Figure 1-1	Site Location – Mount Nansen Site

Table 1-1	Summary of Groundwater Well Locations and Samples Collected

| Area | Well Name | UTM (Zone 08N) |  | Status1,2 | Sample Collected | QA/QC Sample Collected |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Easting | Northing |  |  |  |
| Dome Creek | GSI-DC-01B | 387675 | 6881124 | Frozen | - | - |
|  | GSI-DC-02B | 387879 | 6881129 | Direct Sampled 1 |  | - |
|  | GSI-DC-03B | 388107 | 6881079 | Frozen | - | - |
|  | GSI-DC-05B | 388725 | 6880836 | Frozen 2 | - | - |
|  | GSI-DC-06B | 389788 | 6880567 | Frozen | - | - |
|  | GSI-DC-07B | 390065 | 6880641 | Frozen 2 | - | - |
|  | GSI-DC-08-B | 390311 | 6880583 | Frozen 2 | - | - |
|  | GSI-DC-09-B | 390614 | 6880494 | Frozen | - | - |
|  | GSI-DC-10-B | 390859 | 6880447 | Frozen | - | - |
| Mill Complex | GSI-HA-01A | 387842 | 6881132 | Direct Sampled 1 |  | - |
|  | GSI-HA-02A | 387861 | 6881135 | Frozen | - | - |
|  | GSI-HA-03A | 387878 | 6881131 | Frozen | - | - |
|  | GSI-HA-04A | 387916 | 6881130 | Direct Sampled 1 |  | - |
|  | GSI-HA-05A | 387898 | 6881125 | Frozen | - | - |
|  | MW09-16 | 387992 | 6881094 | Frozen | - | - |
|  | MW09-17 | 388075 | 6880970 | Frozen | - | - |
|  | MW09-18 | 388054 | 6880986 | Good |  | Duplicate, Field Blank |
|  | MW09-19 | 388051 | 6881016 | Good |  | - |
| Brown McDade Pit | CH-P-13-01/10 | 388657 | 6881116 | Frozen | - | - |
|  | CH-P-13-03/10 | 389145 | 6881105 | Frozen | - | - |
|  | CH-P-13-03/50 | 389143 | 6881110 | Direct Sampled 1 |  | Field Blank |
|  | CH-P-13-04/10 | 389138 | 6881472 | Frozen | - | - |
|  | CH-P-13-04/35 | 389138 | 6881472 | Frozen | - | - |
|  | CH-P-13-05/50 | 388954 | 6881466 | Not Accessible 3 | - | - |
|  | GLL07-01 | 388851 | 6881783 | Frozen | - | - |
|  | GLL07-02 | 389069 | 6881703 | Dry | - | - |
|  | GLL07-03 | 388959 | 6881477 | Not Accessible 3 | - | - |
|  | MW09-13 | 389006 | 6881664 | Frozen | - | - |
|  | MW09-14 | 389008 | 6881669 | Direct Sampled 1 |  | - |
|  | MW09-15 | 388920 | 6881727 | Insufficient Volume | - | - |
|  | CH-P-13-02/10 | 388924 | 6881014 | Damaged | - | - |
| Pony Creek | GSI-PC-01-B | N/A | N/A | Destroyed 4 | - | - |
|  | GSI-PC-02-B | 388907 | 6881786 | Destroyed 4 | - | - |
|  | GSI-PC-03-B | 389256 | 6881706 | Frozen 2 | - | - |
|  | GSI-PC-04-B | 389586 | 6881656 | Frozen | - | - |
|  | GSI-PC-05-B | 389713 | 6881661 | Frozen | - | - |
|  | MP09-01 | N/A | N/A | Destroyed 4 | - | - |
|  | MP09-02 | 388867 | 6881816 | Destroyed 4 | - | - |
|  | MP09-03 | 388956 | 6881739 | Frozen | - | - |
|  | MP09-08 | 389160 | 6881718 | Frozen | - | - |
| Seepage Dam | W14103083BH01 | 389522 | 6880669 | Frozen | - | - |
|  | W14103083BH02 | 389561 | 6880665 | Frozen | - | - |
|  | W14103083BH04 | 389544 | 6880666 | Frozen | - | - |
| Tailings Facility | MP09-04 | 389575 | 6880609 | Frozen | - | - |
|  | MP09-05 | 389548 | 6880590 | Frozen | - | - |
|  | MP09-09 | 389240 | 6880681 | Direct Sampled 1 |  | - |
|  | MP09-10 | 389241 | 6880684 | Direct Sampled 1 |  | - |
|  | MP09-11 | 389220 | 6880619 | Frozen | - | - |
|  | MP09-12 | 389220 | 6880619 | Frozen | - | - |
|  | MP09-14 | 389138 | 6880722 | Frozen | - | - |
|  | MW09-01 | 389396 | 6880563 | Direct Sampled 1 |  | - |
|  | MW09-02 | 389393 | 6880562 | Direct Sampled 1 |  | - |
|  | MW09-03 | 389411 | 6880555 | Good |  | - |
|  | MW09-04 | 389420 | 6880557 | Good |  | Duplicate |
|  | MW09-05 | 389413 | 6880656 | Frozen | - | - |
|  | MW09-06 | 389411 | 6880653 | Good |  | - |
|  | MW09-07 | 389322 | 6880699 | Frozen | - | - |
|  | MW09-08 | 389620 | 6880576 | Frozen | - | - |
|  | MW09-11 | 389037 | 6880711 | Frozen | - | - |
|  | MW09-20 | 389592 | 6880586 | Frozen | - | - |
|  | MW09-21 | 389536 | 6880577 | Frozen | - | - |
|  | MW09-22 | 389495 | 6880549 | Direct Sampled 1 |  | - |
|  | MW09-23 | 389459 | 6880553 | Good |  | - |
|  | MW09-24 | 389561 | 6880624 | Good |  | - |
|  | W14103083BH03 | 389132 | 6880730 | Frozen | - | - |

Notes:	1Direct sampling was completed at sample stations where insufficient volume had been encountered during the event, which limited standard purging and sampling methodologies.

2 Groundwater well was found buried beneath ice and could therefore not be monitored.

3 Monitoring wells CH-P-13-05/50 and GLL07-03 were not visited during the February 2016 field event due to pit wall stability safety concerns.

4 Destroyed wells are included in the scope of work and are therefore listed above in the summary table.

Figure 1-2	Groundwater Sampling Locations – Dome Creek and Tailings Facility

Figure 1-3	Groundwater Sampling Locations – Mill Complex and Brown McDade Pit

# Methodology

## Protocols

Groundwater purging, monitoring and sampling conducted by Hemmera/ELR were completed in accordance with the Groundwater Sampling Standard Operating Procedures included in the document Scope of Work: Groundwater Sampling Program – Mount Nansen Site 2015. These procedures were consistent with Environment Yukon’s Protocol for the Contaminated Sites Regulation #7 - Sampling and Decommissioning (Environment Yukon, 2011). Methods used were also consistent with the ASTM D4448-01 Standard Guide for Sampling Groundwater Monitoring Wells (ASTM, 2013), and the D6452-99 Guide for Purging Methods for Wells used for Groundwater Quality Investigations (ASTM, 2012).

## Well Measurements and Purging

Upon arriving at each sample station, headspace gases were measured prior to any other well measurements. Oxygen (%), carbon dioxide (ppm), and methane (%LEL) were measured using an RKI Eagle 2 Gas Sample Drawing Monitor.

The well structure and casing were inspected for damage, closure, and general conditions. Depth to water (DTW; m), depth to bottom (DTB; m), well diameter (cm), and well stick-up height (m) were then recorded at each well.

DTB and DTW were measured using either a Solinst - Model 102 Water Level Meter (for 2.54 cm diameter wells) or a Solinst – Model 122 Interface Meter (for wells with diameter greater than 2.54 cm). DTB and DTW were measured from (in order of preference): 1) a black mark drawn on the top of the well; 2) the bottom of the most significant notch found on the top of the PVC if a mark was not present; or 3) a line that was drawn on the highest point of the well if no distinguishable point of measure was present. Stick-up height was measured from the lowest point on the bottom of the well casing to the highest point (or distinguishing mark) on the well. Water level meters were cleaned between each sample site using Alconox low-foaming phosphate-free detergent solution and deionized water.

Following initial inspection and measurements, groundwater wells were purged and sampled using dedicated equipment. Groundwater wells were purged and sampled using one of three (3) techniques: 1) manual purging using high density polyethylene (HDPE) Waterra tubing and a footvalve, 2) GeoPump peristaltic pump with HDPE tubing, or 3) manual purging using disposable polyethylene bailers. The purging technique chosen for each well was that which would produce the most representative groundwater sample.

Groundwater wells were determined to be sufficiently purged when either three (3) successive field parameter measurements were recorded to be within an allowable tolerance level (as summarized in Table 2-1, below) or when a volume of water equivalent to three (3) standing well volumes of water had been purged.

Groundwater turbidity measured in Nephelometric Turbidity Units (NTU) or Attenuation Units (AU) was also measured prior to sampling (described below in Section 2.4) and was used as an indication of sample quality. Where possible, samples were not collected until turbidity was less than 50 NTU. Purge volumes and purge rates were measured using a graduated container and stop watch. All well measurements, purging details, and additional field notes were recorded on customized field forms in order to minimize the potential for field errors.

Table 2-1	Groundwater Sampling – Field Parameter Purging Criteria

| Field Parameter | Allowable Variance |
| --- | --- |
| Temperature (°C) | ± 3% |
| pH | ± 0.1 |
| Conductivity (µS/cm) | ± 3% |
| Specific Conductivity (µS/cm) | ± 3% |

## Direct Sampling

During previous events a select number of groundwater wells were found to have an insufficient volume of groundwater to sample using conventional methods, limiting the number of wells that were sampled during the event. An alternate sampling strategy was established by AAM’s consultant (AMEC) in order to obtain samples from low producing wells, which was followed during the February 2016 sampling event. At wells identified as having insufficient volume of water, Hemmera/ELR direct sampled (analytical samples collected prior to purging or collecting field parameter measurements), after which time field parameter measurements were collected if possible. Additionally, a priority ranking order for analytical sample collection previously established by AAM’s consultant (AMEC) was used when collecting samples at direct sampled wells (as summarized in Table 2-2). This ranking system was used to ensure that samples for higher priority parameters were collected at each well if limited recharge or volume was encountered. Where sample collection was limited, Hemmera/ELR also re-visited wells, where feasible, in an attempt to collect a more thorough sample set.

In addition to the priority ranking order, Hemmera/ELR also considered the minimum sample volumes required for laboratory procedures (provided to Hemmera/ELR by ALS Laboratories). Where well volume was limited, minimum volumes were collected to maximize the number of program parameters collected.

## Field Parameters

Hemmera/ELR measured in-situ water quality parameters using YSI Professional Plus multi-parameter field meters, Lamotte 2020we turbidity meters, and Hach DR 890 Portable Colorimeters. Flow-through cells were used with the YSI meters to minimize field parameter variability; flow-through cells improve the precision of field measurements by limiting sample water contact with air, and by continuously moving sample water across the field meter sensors. In cases where ambient air temperature was too low to effectively use a flow-through cell without freeze-up, a graduated container was used. The in-situ groundwater quality parameters recorded at each sample station included water temperature (oC), specific conductivity (μs/cm), conductivity (μs/cm), oxidation/reduction potential (ORP; mv), pH (pH units), sulphide (mg/l), dissolved oxygen (mg/l and percent saturation), and turbidity (NTU).

During purging, field parameters were monitored at 5 minute intervals, or at volume related intervals (e.g., every 500 mL) in the case of wells with slow recharge. The final set of in situ measurements were recorded at the conclusion of purging.

## Groundwater Sampling

Groundwater quality samples were collected and preserved in accordance with laboratory directions, and using techniques consistent with Standard Methods for the Examination of Water and Wastewater (Rice et al., 2012). ALS Global was the analytical laboratory chosen for this project, and a summary of the sample bottle set (including parameters analysed and preservation techniques) is provided in Table 2-2.

Table 2-2	Groundwater Sampling Parameter Priority, Preservation, and Intended Analysis

| Priority | Bottle Type | Parameters Analyzed | Minimum Volume | Sample Treatment | Preservative Added |
| --- | --- | --- | --- | --- | --- |
| 1a | 120 ml (plastic) | Dissolved Metals | 100 ml | Field Filtered and Preserved | HNO3 |
| 1b | 40 ml (glass) | Dissolved Mercury | 15 mL | Field Filtered and Preserved | HCl |
| 2 | 1 L (plastic) | General Chemistry | 200 ml | - | - |
| 3 | 145 ml (plastic) | Cyanide (total, free, weak acid dissociable) | 120 ml | Preserved | NaOH |
| 4 | 250 ml (glass amber) | Ammonia (NH3) | 120 ml | Preserved | H2SO4 |
| 5 | 120 ml (plastic) | Thiocyanate (SCN) | 50 ml | Preserved | HNO3 |
| 6 | 120 ml (plastic) | Sulphide | 100 ml | Preserved | Zinc Acetate, and NaOH |
| 7 | 250 ml (glass amber) | Total Inorganic Carbon (TIC) | 100 ml | - | - |

## Data Analysis

Groundwater analytical field and laboratory results were tabulated and reviewed using Hemmera/ELR’s EQWin Data Manager water quality database. Data was tabulated for the report and compared to the Canadian Council of Ministers of the Environment (CCME) Water Quality Guidelines for the Protection of Freshwater Aquatic Life (FAL; CCME, 2014) standards using the database application. All relevant CCME FAL guidelines are presented in Table A.

## Quality Assurance and Quality Control

### Field QA/QC

Several controls were used by Hemmera/ELR staff while in the field to ensure that sample integrity was maintained and that data were recorded completely and accurately. All equipment used during the sampling process was dedicated to individual wells, including HDPE tubing and Waterra footvalves, laboratory provided pre-cleaned sample bottles, disposable filters, disposable syringes, and disposable polyethylene bailers. Field staff used dedicated disposable nitrile gloves for all measurements, purging, and sampling. Water level meters were cleaned between well locations using Alconox low-foaming phosphate-free detergent and deionized water, and field instruments (YSI field meters, turbidity meters, and portable colorimeters) were checked and calibrated before the site visit to ensure the parameters recorded were as accurate as possible.

Project-specific field data sheets were created for the sampling event to help ensure that all required measurements were taken, and that information was recorded correctly. Field data sheets have been included as Appendix B of this report.

### Analytical QA/QC

Analytical QA/QC measures were included in the February 2016 sampling program as outlined in the scope of work and as per standard industry practice. This included the collection of field duplicates and field blanks, and the use of travel blanks. Duplicate samples were collected at a ratio of 10% of the regular samples (1 duplicate was collected for every 10 samples), and one (1) field blank was prepared during the program. Two travel blanks accompanied the analytical supplies and samples from the laboratory to the field, and back to the laboratory again (1 for each shipment).

The variation between sample and duplicate values was calculated as relative percent difference (RPD). RPD provides a measure of the relative difference between two values in comparison to their mean value, and is calculated as the difference between a sample and its field duplicate over the average of two values. RPD values greater than 20% indicate a greater than expected variation in data that could potentially have affected the precision of sampling or analysis. RPD was calculated according to the following formula:

%RPD=χ1-χ2χ1+χ22 x 100

RPD is not calculated if either the sample or the field duplicate concentration is less than five times the detection limit.

The analytical results for field and travel blanks were reviewed to determine whether any of the parameters tested were detected (i.e., result exceeding the detection limit). In such cases, the parameter or element in question and its concentration were reviewed to determine potential sources of contamination or error.

# Results

A summary of laboratory analytical results is presented in Table A of this report, including a comparison of results to CCME FAL guidelines. A summary of the QA/QC sampling results is presented in Table B, including analytical data for duplicates, field blanks, and travel blanks. Laboratory analytical reports are appended to this report (Appendix C).

## Groundwater Sampling Summary

Groundwater sampling was completed between February 1 and February 4, 2016. Weather conditions varied throughout the time of sampling with ambient air temperature ranging from -10 to -19°C. Periods of light snow and heavy to light wind occurred throughout the sampling event.

Of the sixty-five (65) wells specified for the February 2016 sampling event, fifty-five (55) were located and assessed during the program. As noted in Section 1.2, four (4) groundwater wells listed in the scope of work had previously been reported as destroyed (GSI-PC-01-B, MP09-01, GSI-PC-02-B and MP09-02), two (2) were not accessible due to safety concerns (GLL07-03 and CH-P-13-05/50), and four (4) were covered in ice and could not be monitored (GSI-DC-05-B, GSI-DC-07-B, GSI-DC-08-B, and GSI-PC-03-B). Further details concerning these wells are provided in Section 3.2.

Of the fifty-five (55) wells located, seventeen (17) wells were sampled; seven (7) using purging and sample methods as per the program protocols, and ten (10) direct sampled without purging according to the sample priority ranking (Table 2-2). In four (4) of the ten (10) direct sampled wells, volumes were insufficient to collect a full sample set. Table 3-1 provides a summary of limited sample set collection.

Of the remaining thirty-eight (38) of the fifty-five (55) wells that were assessed but not sampled during the program, thirty-five (35) wells were frozen, one (1) well was dry, one (1) well had insufficient volume for sampling, and one (1) well was damaged and could not be sampled. Despite not collecting water quality samples, these wells were still assessed and water/ice depth, well depth, and headspace gas measurements were collected to the extent possible. A summary of the overall condition (status) and sampling result for groundwater wells is provided in Table 1-1, and a summary of all well measurements, purge details, and in-situ parameter results is provided in Table 3-2.

Table 3-1	Summary of Direct Samples Collected During February 2016 Sampling Program

| Well Name | Dissolved Metals | Dissolved Mercury | Physical Parameters | Anions/ Nutrients | Cyanide | Ammonia | Thiocyanate | Sulphide | Total Inorganic Carbon |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Priority | 1a | 1b | 2 | 2 | 3 | 4 | 5 | 6 | 7 |
| GSI-DC-02B |  |  |  |  |  |  |  |  | - |
| GSI-HA-01A 1 | - |  | - | - | - | - | - | - | - |
| GSI-HA-04A |  |  | - | - | - | - | - | - | - |
| CH-P-13-03/50 |  |  |  |  |  |  |  |  |  |
| MW09-14 |  |  |  |  |  |  |  |  |  |
| MP09-09 |  |  |  |  |  |  |  |  |  |
| MP09-10 |  |  |  | - | - | - | - | - | - |
| MW09-01 |  |  |  |  |  |  |  |  |  |
| MW09-02 |  |  |  |  |  |  |  |  |  |
| MW09-22 |  |  |  |  |  |  |  |  |  |

Notes:	Refer to Section 2.2 for details concerning direct sampling methodologies, including minimum volume collection. Samples were collected based on field priority ranking as specified in Table 2-2.

1 Insufficient volumes available to collect a full dissolved metals sample, only a dissolved mercury sample was collected.

Table 3-2 	Groundwater Field Parameters and Well Measurements for February 2016 Sampling Program

| Area | Location ID | Sample Date | Status | Stick up Height (m) | Depth To Water (m) | Depth to Bottom (m) | Standing Water Volume (L) | Volume Purged (L) | Purge Start Time | Purge End Time | Elapsed Purge Time | Purge Rate (l/min) | Criteria1 (3WV/PS/DS) | Draw Down (m) | pH | Temperature (ºC) | Conductivity (µS/cm) | Specific Conductivity (µS/cm) | ORP (mV) | Dissolved Oxygen (mg/L) | Field Sulphide (mg/L) | Methane (%LEL) | Oxygen (%) | Carbon Dioxide (ppm) | Field Turbidity (NTU) | Method Used | Well Diameter (cm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dome Creek | GSI-DC-01A | 01/02/2016 | Frozen | 0.64 | 0.795 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.9 | 350 | - | - | 2.54 |
|  | GSI-DC-01B | 01/02/2016 | Frozen | 0.67 | 1.456 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 19.1 | 325 | - | - | 2.54 |
|  | GSI-DC-02A | 01/02/2016 | Frozen | 0.94 | 1.815 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 17.7 | 720 | - | - | 2.54 |
|  | GSI-DC-02B 2 | 01/02/2016 | Direct Sample | 0.81 | 2.300 | 3.717 | 0.9 | - | - | - | - | - | DS | - | - | - | - | - | - | - | - | 0 | 17.7 | 1700 | - | peristaltic | 2.54 |
|  | GSI-DC-03A | 01/02/2016 | Frozen | 0.68 | 0.978 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 2.54 |
|  | GSI-DC-03B | 01/02/2016 | Frozen | 0.66 | 0.945 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 275 | - | - | 2.54 |
|  | GSI-DC-05A 3 | 02/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-DC-05B 3 | 02/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-DC-06A | 02/02/2016 | Frozen | 0.81 | 0.841 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 25 | - | - | 2.54 |
|  | GSI-DC-06B | 02/02/2016 | Frozen | 0.41 | 0.534 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 75 | - | - | 2.54 |
|  | GSI-DC-07A 3 | 02/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-DC-07B 3 | 02/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-DC-08A 3 | 02/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-DC-08B 3 | 02/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-DC-09A | 02/02/2016 | Frozen | 0.21 | 1.055 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 25 | - | - | 2.54 |
|  | GSI-DC-09B | 02/02/2016 | Frozen | 0.21 | 1.026 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 2.54 |
|  | GSI-DC-10A | 02/02/2016 | Frozen | 0.85 | 1.032 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 2.54 |
|  | GSI-DC-10B | 02/02/2016 | Frozen | 0.77 | 0.951 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 2.54 |
| Mill Complex | GSI-HA-01A 2 | 01/02/2016 | Direct Sample | 1.1 | 2.500 | 2.785 | 0.2 | - | - | - | - | - | DS | - | - | - | - | - | - | - | - | 0 | 18 | 320 | - | peristaltic | 2.54 |
|  | GSI-HA-02A | 01/02/2016 | Frozen | 1.46 | 2.114 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.0 | 340 | - | - | 2.54 |
|  | GSI-HA-03A | 01/02/2016 | Frozen | 0.94 | 0.985 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 200 | - | - | 2.54 |
|  | GSI-HA-04A 2 | 01/02/2016 | Direct Sample | 0.51 | 1.816 | 2.11 | 0.3 | - | - | - | - | - | DS | - | - | - | - | - | - | - | - | 0 | 20.9 | 160 | - | peristaltic | 2.54 |
|  | GSI-HA-05A | 01/02/2016 | Frozen | 0.68 | 0.400 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 2.54 |
|  | MW09-16 | 01/02/2016 | Frozen | 1.29 | 1.855 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 525 | - | - | 5.08 |
|  | MW09-17 | 01/02/2016 | Frozen | 0.97 | 5.609 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 9.3 | >10000* | - | - | 5.08 |
|  | MW09-18 | 01/02/2016 | Good | 0.86 | 5.104 | 7.696 | 5.2 | 5.0 | 13:00 | 13:07 | 0:07 | 0.7 | PS | 0.001 | 6.76 | -0.1 | 1356 | 2608 | 127.5 | 2.16 | 0.00 | 0 | 20.9 | 125 | 34.1 | disp. bailer | 5.08 |
|  | MW09-19 | 01/02/2016 | Good | 0.99 | 3.040 | 5.797 | 5.5 | 1.6 | 11:51 | 12:08 | 0:17 | 0.1 | PS | 0.240 | 6.68 | -0.3 | 1116 | 2157 | -77.8 | 0.52 | 0.12 | 0 | 20.9 | 50 | 2.9 | disp. bailer | 5.08 |
| Brown McDade Pit | CH-P-13-01/10 | 02/02/2016 | Frozen | 0.49 | 6.678 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 50 | - | - | 5.08 |
|  | CH-P-13-03/10 | 02/02/2016 | Frozen | 0.59 | 4.922 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 5125 | - | - | 3.81 |
|  | CH-P-13-03/50 2 | 02/02/2016 | Direct Sample | 0.54 | 48.985 | 50.555 | 0.8 | - | - | - | - | - | DS | - | - | - | - | - | - | - | 0.8 | 0 | 20.3 | 2750 | 1146 (AU)** | disp. bailer | 2.54 |
|  | CH-P-13-04/10 | 03/02/2016 | Frozen | 0.54 | 6.045 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 3.81 |
|  | CH-P-13-04/35 | 03/02/2016 | Frozen | 0.61 | 6.408 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 25 | - | - | 2.54 |
|  | CH-P-13-05/50 4 | - | Not Accessible | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GLL07-01 | 03/02/2016 | Frozen | 0.79 | 13.745 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 10.5 | >10000* | - | - | 5.08 |
|  | GLL07-02 | 03/02/2016 | Dry | 1.34 | 7.100 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.5 | 8350 | - | - | 15.24 |
|  | GLL07-03 4 | - | Not Accessible | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | MW09-13 | 03/02/2016 | Frozen | 0.73 | 8.920 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 1200 | - | - | 5.08 |
|  | MW09-14 2 | 03/02/2016 | Direct Sample | 0.74 | 5.114 | 7.618 | 5 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 525 | - | disp. bailer | 5.08 |
|  | MW09-15 | 03/02/2016 | Insufficient Volume | 0.89 | 13.989 | 14.025 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.7 | 650 | - | - | 5.08 |
|  | CH-P-13-02/10 | 02/02/2016 | Damaged | 0.84 | 8.238 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 19.1 | >10000* | - | - | 5.08 |
| Pony Creek | GSI-PC-01A 5 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-01B 5 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-02A 5 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-02B 5 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-03A 3 | 03/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-03B 3 | 03/02/2016 | Frozen | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | GSI-PC-04A | 03/02/2016 | Frozen | 0.68 | 0.817 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 25 | - | - | 2.54 |
|  | GSI-PC-04B | 03/02/2016 | Frozen | 0.77 | 1.419 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 53 | - | - | 2.54 |
|  | GSI-PC-05A | 03/02/2016 | Frozen | 0.73 | 0.838 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 9.0 | >10000* | - | - | 2.54 |
|  | GSI-PC-05B | 03/02/2016 | Frozen | 0.75 | 0.839 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 15.0 | >10000* | - | - | 2.54 |
|  | MP09-01 5 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | MP09-02 5 | - | Destroyed | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  | MP09-03 | 03/02/2016 | Frozen | 0.54 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 21.0 | 25 | - | - | 2.54 |
|  | MP09-08 | 03/02/2016 | Frozen | 0.82 | 0.360 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 2.54 |
| Seepage Dam | W14103083BH01 | 02/02/2016 | Frozen | 0.62 | 6.450 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.3 | 320 | - | - | 5.08 |
|  | W14103083BH02 | 02/02/2016 | Frozen | 0.77 | 6.690 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.4 | 300 | - | - | 5.08 |
|  | W14103083BH04 | 02/02/2016 | Frozen | 0.77 | 6.500 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.3 | 320 | - | - | 5.08 |
| Tailings Facility | MP09-04 | 02/02/2016 | Frozen | 1.23 | 1.814 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.4 | 700 | - | - | 3.81 |
|  | MP09-05 | 02/02/2016 | Frozen | 1.05 | 1.343 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.5 | 260 | - | - | 5.08 |
|  | MP09-09 2 | 04/02/2016 | Direct Sample | 2.46 | 3.890 | 5.56 | 1.8 | - | - | - | - | - | DS | - | - | - | - | - | - | - | - | 4 | 18 | 260 | - | peristaltic | 3.81 |
|  | MP09-10 2 | 04/02/2016 | Direct Sample | 2.2 | 3.655 | 4.39 | 1 | - | - | - | - | - | DS | - | - | - | - | - | - | - | - | 0 | 18.1 | 380 | - | disp. bailer | 3.81 |
|  | MP09-11 | 04/02/2016 | Frozen | 1.89 | 7.900 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 17.7 | 460 | - | - | 3.81 |
|  | MP09-12 | 04/02/2016 | Frozen | 1.84 | 2.000 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.0 | 340 | - | - | 3.81 |
|  | MP09-14 | 04/02/2016 | Frozen | 0.92 | 0.864 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 21.0 | 0 | - | - | 2.54 |
|  | MW09-01 2 | 03/02/2016 | Direct Sample | 0.73 | 7.170 | 9.18 | 4 | - | - | - | - | - | DS | - | - | - | - | - | - | - | - | 0 | 18.4 | 1700 | - | Waterra | 5.08 |
|  | MW09-02 | 02/02/2016 | Direct Sample | 0.74 | 3.612 | 4.72 | 2.2 | 1 | 16:52 | 16:55 | 0:03 | - | DS | 0.508 | 7.28 | 0.1 | 1463 | 2790 | -69.3 | 2.68 | 0.04 | 0 | 18.4 | 520 | 18.8 | peristaltic | 5.08 |
|  | MW09-03 | 03/02/2016 | Good | 0.23 | 6.754 | 9.953 | 6.4 | 5 | 12:17 | 12:54 | 0:37 | 0.1 | PS | 0.236 | 7.45 | 0.4 | 1431 | 2703 | 112 | 3.36 | - | 0 | 18 | 380 | 0.6 | peristaltic | 5.08 |
|  | MW09-04 | 03/02/2016 | Good | 0.08 | 4.738 | 7.68 | 5.8 | 4 | 10:00 | 10:40 | 0:40 | 0.1 | PS | 0.732 | 8.37 | 0.3 | 1467 | 2766 | 191.5 | 3.78 | 0.03 | 0 | 18 | 440 | 3.4 | peristaltic | 5.08 |
|  | MW09-05 | 03/02/2016 | Frozen | 1.11 | 7.480 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 375 | - | - | 5.08 |
|  | MW09-06 | 03/02/2016 | Good | 2.07 | 4.454 | 5.95 | 3 | 3.9 | 16:33 | 16:49 | 0:16 | 0.3 | PS | 1.306 | 7.29 | 1.6 | 678 | 1209 | 159.9 | 2.53 | 0.05 | 0 | 14.1 | 3350 | 16.6 | disp. bailer | 5.08 |
|  | MW09-07 | 04/02/2016 | Frozen | 1.37 | 3.316 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 0 | - | - | 5.08 |
|  | MW09-08 | 02/02/2016 | Frozen | 1.11 | 1.240 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.8 | 340 | - | - | 5.08 |
|  | MW09-11 | 04/02/2016 | Frozen | 0.82 | 4.845 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 20.9 | 1125 | - | - | 5.08 |
|  | MW09-20 | 02/02/2016 | Frozen | 0.92 | 3.700 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.7 | 1500 | - | - | 5.08 |
|  | MW09-21 | 02/02/2016 | Frozen | 0.40 | 1.030 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 18.6 | 260 | - | - | 5.08 |
|  | MW09-22 2 | 03/02/2016 | Direct Sample | 0.83 | 4.587 | 5.298 | 1.8 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 17.3 | 5000 | - | disp. bailer | 5.08 |
|  | MW09-23 | 03/02/2016 | Good | 0.11 | 12.600 | 15.85 | 6.5 | 25 | 15:20 | 15:32 | 0:12 | 2.1 | 3WV | 0.2 | 7.21 | 0 | 716 | 1372 | -79.7 | 3.3 | 0.29 | 0 | 18 | 320 | 49.1 | Waterra | 5.08 |
|  | MW09-24 | 02/02/2016 | Good | 0.67 | 9.440 | 11.24 | 3.6 | 15 | 13:39 | 14:50 | 1:11 | 0.2 | PS | 0.01 | 7.23 | 0.2 | 446 | 847 | 224.2 | 8.06 | 0.17 | 0 | 24.3 | 4500 | 37.6 | disp. bailer | 5.08 |
|  | W14103083BH03 | 04/02/2016 | Frozen | 0.74 | 1.425 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | 0 | 21.0 | 0 | - | - | 5.08 |

Notes:	To maximize the sample return for analytical analysis, field parameters were not collected at all direct sampled wells. Shaded rows indicate monitoring stations where analytical samples were collected.

1 3WV = Three well volumes purged prior to sample collection, PS = field parameters stabilized prior to sample collection, and DS = sample collected directly without purging.

2Due to low well volumes (direct sampling), field parameters were not measured.

3 Well found frozen beneath ice and therefore could not be monitored.

4 Well was not accessible during the sampling event due to health and safety concerns.

5 Well has been destroyed by placer mining activity.

* Field measurements over range of instrument.

**AU (Attenuation Unit)

## Analytical Results

Analytical results are provided below, including a brief summary of CCME FAL guideline exceedances and a description of factors that may have influenced the data. Details regarding well status, including a description of damaged or underperforming wells, are also provided.

In several instances, laboratory reportable detection limits (RDL) for parameters exceeded applicable CCME FAL standards (lightly shaded values in Table A). In these cases, samples having elevated levels of certain parameters required laboratory dilution in order to perform the required analyses, thereby resulting in an elevated RDL. For the purpose of this report, samples where the reported RDL is greater than the applicable guideline have not been reported as CCME FAL exceedances.

### Dome Creek

Groundwater wells along Dome Creek were monitored between February 1 and February 2, 2016. Samples were obtained from one (1) of the nine (9) drive-point piezometers located in this area. Eight (8) of the drive-point piezometers located in this area were found frozen during the time of sampling (GSI-DC-01B, GSI-DC-03B, GSI-DC-05B, GSI-DC-06B, GSI-DC-07B, GSI-DC-08B, GSI-DC-09B, GSI-DC-10B), the remaining well (GSI-DC-02B) was sampled directly without purging. Of the eight (8) wells reported to be frozen, three (3) were completely buried beneath ice and could not be inspected/monitored during the February 2016 monitoring program. A summary of field measurements, including headspace gases, is provided in Table 3-2.

No CCME FAL guideline exceedances were reported from the Dome Creek area during the February 2016 sampling event. Laboratory analytical results and provided in Appendix C.

Groundwater turbidity was not measured at site GSI-DC-02B due to insufficient well volumes (Table 3-2).

Table 3-3	Summary of CCME FAL Guideline Exceedances for February 2016 Sampling Program

| Area | Sample ID | Date Sampled | Parameter | Field Dissolved Oxygen | Ammonia, Total (as N) | Ammonia CCME-FAL2 | Fluoride (F) | Nitrite (as N) | Cyanide, Free | Dissolved Arsenic (As) | Dissolved Copper (Cu) | Copper CCME-FAL 2 | Dissolved Iron (Fe) | Dissolved Mercury (Hg) | Dissolved Selenium (Se) | Dissolved Silver (Ag) | Dissolved Uranium (U) | Dissolved Zinc (Zn) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Units | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L | mg/L |
|  |  |  | CCME-FAL1 | 9.5 | Varies 2 | - | 0.12 | 0.06 | 0.005 | 0.005 | Varies 10 | - | 0.3 | 0.000026 | 0.001 | 0.0001 | 0.015 | 0.03 |
| Mill Complex | GSI-HA-04A | 01/02/2016 | Direct Sample | - | - | - | - | - | - | 0.0421 | - | - | 8.26 | - | - | - | - | - |
|  | MW09-18 | 01/02/2016 | Good | 2.16 | - | - | <0.20 | - | - | 0.0519 | - | - | - | - | - | - | - | - |
|  | MW09-19 | 01/02/2016 | Good | 0.52 | - | - | - | - | - | 0.0224 | - | - | 3.00 | - | - | - | - | - |
| Brown McDade Pit | CH-P-13-03/50 | 02/02/2016 | Direct Sample | - | - | - | <0.20 | - | - | - | - | - | - | - | 0.00733 | - | - | - |
|  | MW09-14 | 03/02/2016 | Direct Sample | - | - | - | - | - | - | 0.0134 | - | - | 7.25 | - | - | - | 0.0193 | - |
| Tailings Facility | MP09-09 | 04/02/2016 | Direct Sample | - | - | - | 1.71 | - | 0.532 | 17.0 | 0.578 | 0.004 | - | 0.0000328 | 0.00103 | 0.0193 | - | - |
|  | MP09-10 | 04/02/2016 | Direct Sample | - | - | - | 0.924 | 0.805 | - | 6.26 | 0.0469 | 0.004 | - | 0.0000881 | 0.00156 | 0.00272 | - | - |
|  | MW09-013 | 03/02/2016 | Direct Sample | - | - | - | <0.20 | - | - | 0.0989 | 0.00870 | 0.004 | - | - | - | - | - | 0.968 |
|  | MW09-02 | 02/02/2016 | Direct Sample | 2.68 | - | - | 0.47 | - | - | 15.0 | - | - | 24.9 | - | - | - | - | 0.239 |
|  | MW09-03 | 03/02/2016 | Good | 3.36 | - | - | 0.22 | 0.071 | - | 1.66 | - | - | - | - | - | - | - | - |
|  | MW09-04 | 03/02/2016 | Good | 3.78 | 7.38 | 0.9788 | 0.33 | - | - | 3.84 | - | - | - | - | - | - | - | 0.0931 |
|  | MW09-064 | 04/02/2016 | Good | 2.53 | - | - | 0.19 | - | - | 0.211 | 0.00761 | 0.004 | - | - | - | - | - | 0.0879 |
|  | MW09-22 | 03/02/2016 | Direct Sample | - | - | - | - | - | <0.010 | - | - | - | 4.09 | - | - | - | - | - |
|  | MW09-23 | 03/02/2016 | Good | 3.30 | - | - | - | - | - | 0.0284 | - | - | 13.1 | - | - | - | - | 0.0916 |
|  | MW09-24 | 02/02/2016 | Good | 8.06 | - | - | - | - | - | - | 0.00519 | 0.004 | - | - | - | - | - | - |

Notes: 	1 CCME guideline exceedances shaded with grey. Green shading denotes reportable detection limit in exceedance of CCME guideline.

2 Calculated CCME guidelines, refer to Table A for details.

3 Due to slow recharge and low well volumes, samples were collected from MW09-01 between February 2 and 3, 2016. Dissolved metals were collected on February 2, 2016. All other parameters were collected on February 3, 2016.

4 Due to slow recharge and low well volumes, field parameters were measured for well MW09-06 on February 3, 2016. Laboratory analyzed samples were collected on February 4, 2016.

“–“ indicates either no exceedance was observed or no analysis was conducted. Refer to Table A for full analytical report.

### Mill Complex

Groundwater in the Mill Complex Area was sampled on February 1, 2016. Samples were obtained from four (4) of the nine (9) wells identified in this area. Five (5) of the wells located in this area were found frozen during the time of sampling (GSI-HA-02A, GSI-HA-03A, GSI-HA-05A, MW09-16, and MW09-17). Drive-points GSI-HA-01A and GSI-HA-04A were sampled directly without purging, all other wells were sampled according program protocols (MW09-18 and MW09-19). A summary of the samples collected is provided in Table 3-1, and analytical results are provide in Table A.

CCME FAL guideline exceedances were observed at three (3) of the four (4) sites sampled in the Mill Complex area, including exceedances of dissolved arsenic (three sites) and dissolved iron (two sites). Where measured (three sites), field dissolved oxygen concentrations were below the minimum CCME FAL guideline. A summary of CCME FAL guideline exceedances is provided in Table 3-3.

Where measured, groundwater turbidity of all samples collected within this area was less than 50 NTU (Table 3-2).

### Brown McDade Pit

Groundwater wells in the Brown McDade Pit area were sampled between February 2 and February 3, 2016. Samples were obtained from two (2) of the thirteen (13) sites identified in this area (CH-P-13-03/50 and MW09-14). All samples collected in the Brown McDade area were direct sampled without purging. Six (6) wells were frozen during the Site visit (CH-P-13-01/10, CH-P-13-03/10, CH-P-13-04/10, CH-P-13-04/35, GLL07-01, and MW09-13), one (1) well was dry (GLL07-02), one (1) well was damaged (CH-P-13-02/10), and one (1) well had insufficient volume to collect a sample (MW09-15). Two (2) wells located within the Brown McDade Pit (CH-P-13-05/50 and GLL07-03) were not accessible during the time of sampling due to safety concerns regarding instability of the pit walls. Access to the Brown McDade Pit was restricted by AAM and Denison Environmental Services (DES) during the Site visit. A summary of the samples collected is provided in Table 3-1, and analytical results are provide in Table A.

CCME FAL guideline exceedances were observed at the two (2) sites sampled in the Brown McDade Pit area, including exceedances of dissolved arsenic (one site), dissolved iron (one site), dissolved selenium (one site), and dissolved uranium (one site). A summary of CCME FAL guideline exceedances is provided in Table 3-3.

Groundwater at sample location CH-P-13-03/50 was extremely turbid (1146 AU; Attenuation Units) during the time of sampling. Due to limited well volumes, groundwater turbidity was not measured at the other sampled location (MW09-14) within the Brown McDade Pit area (Table 3-2).

Similar to previous sampling events, CH-P-13-02/10 was damaged during the time of sampling. The well status had been previously listed as damaged as bentonite was present at the bottom of the well (Hemmera, 2015a). Well repairs and re-development of this location will be again recommended for the summer of 2016, but are limited by the lack of water in the well (i.e., bentonite clay is in solid form).

### Pony Creek

Groundwater wells along Pony Creek were monitored on February 3, 2016. Five (5) of the nine (9) groundwater wells identified in the Pony Creek area were frozen during the time of sampling (GSI-PC-03B, GSI-PC-04B, GSI-PC-05B, MP09-03, and MP09-08). The four (4) remaining wells identified in this area were previously destroyed by placer mining activity (GSI-PC-01B, GSI-PC-02B, MP09-01, and MP09-02) and therefore could not be monitored. Of the five (5) wells reported to be frozen, one (1) was completely buried beneath ice and could not be inspected/monitored during the February 2016 monitoring program. A summary of field measurements collected for each site is provided Table 3-2.

### Seepage Dam

Groundwater wells in the Seepage Dam area were monitored on February 2, 2016. All three (3) of the groundwater wells identified in the Seepage Dam area were frozen during the time of sampling. A summary of field measurements collected for each site is provided Table 3-2.

### Tailings Facility

Groundwater wells in the Tailings Facility area were sampled between February 2 and February 4, 2016. Samples were obtained from ten (10) of the twenty-two (22) sample sites located in this area. Twelve (12) of the twenty-two (22) groundwater wells identified in the Tailings Facility area were frozen at the time of sampling (MP09-04, MP09-05, MP09-11, MP09-12, MP09-14, MW09-05, MW09-07, MW09-08, MW09-11, MW09-20, MW09-21, and W14103083BH03). Of the ten (10) samples collected in the Tailings Facility area, five (5) were collected directly without purging (MP09-09, MP09-10, MW09-01, MW09-02, and MW09-22). All other wells were purged prior to sampling (MW09-03, MW09-04, MW09-06, MW09-23, and MW09-24). A summary of the samples collected is provided in Table 3-1, and analytical results are provide in Table A.

CCME FAL guideline exceedances were observed at all ten (10) sites sampled in the Tailings Facility area, including exceedances of total ammonia (one site), fluoride (six sites), nitrite (two sites), free cyanide (one site), dissolved arsenic (eight sites), dissolved copper (five sites), dissolved iron (three sites), dissolved mercury (two sites), dissolved selenium (two sites), dissolved silver (two sites), and dissolved zinc (five sites). Field dissolved oxygen concentrations were below the minimum CCME FAL guideline level at five (5) of the sampled sites. A summary of CCME FAL guideline exceedances is provided in Table 3-3.

Where measured, groundwater turbidity at sampled locations in the Tailings Facility area was less than 50 NTU at the time of sample (Table 3-2). Although turbidity was not measured for samples collected directly without purging, field notes indicate that groundwater wells MP09-09 and MW09-01 were extremely turbid at the time of sample collection (visual observation).

## Quality Assurance and Quality Control Results

Two (2) duplicate groundwater samples were collected during the February 2016 sampling event. Two (2) travel blanks were provided by the laboratory and accompanied the samples throughout the sampling program, and one (2) field blanks were prepared on site during the sampling program. Detailed results of QA/QC sampling are provided in Table B, including RPD values for all duplicate and sample pairs.

### Field and Travel Blanks

All travel blank analytical results were reported as less than the RDL with the exception of total ammonia (0.0056 mg/L) which was reported in the travel blank included with the second shipment (Table B). The program analytical laboratory (ALS Global) indicated that the detection of low levels of ammonia should not be considered an indication of contamination as low concentrations of ammonia are occasionally detected in travel blanks that are prepared too early in advance of the field program. All other analytical results in both travel blanks were reported below RDL (Table B).

All field blank analytical results were reported as less than the RDL (Table B).

### Field Duplicates

#### MW09-18 and MW16-100

Duplicate and duplicate pair analytical results show that RPD values for nitrate (86.55%), total organic carbon (46.85%), and dissolved cadmium (42.55%) were reported above the acceptable range of variability (i.e. 20%). Field notes/measurements do not identify a potential source of contamination or suggest variability in groundwater quality during the purging process (Table 3-2).

All other duplicate and duplicate pair analytical results show that RPD values for samples MW09-18 and MW16-100 were below the 20% RPD threshold limit (Table B).

#### MW09-04 and MW16-200

All duplicate and duplicate pair analytical results show that RPD values for samples MW09-04 and MW16-200 were below the 20% RPD threshold limit (Table B).

### Quality Assurance and Quality Control Summary

Results for the QA/QC analytical program show minimal evidence of sampling variation during the field collection process, and no evidence of contamination during sample collection or transportation.

Travel blank analytical results suggest no external sources of contamination during the transportation process. Although low levels of total ammonia were detected in one of the travel blanks, ALS has indicated that these results do not suggest an external source of contamination.

Overall, across two (2) field blanks, all analytical results were reported as less than the RDL (Table B).This suggests that on site sources of contamination have had minimal influence on the program analytical results.

Duplicate and duplicate pair analytical results show some variability in nitrate, total organic carbon, and dissolved cadmium concentrations. Overall, variation of three parameters was observed in only one (1) of two (2) sample duplicate pairs (at Station MW09-18). The most notable variation was observed between nitrate concentrations, resulting in 86.55% RPD between samples MW09-18 and MW16-100. All other parameters were below the 20% RPD threshold limit. Field notes/measurements do not identify a potential source of contamination or suggest variability in groundwater quality during the purging process, and therefore a systematic or site-specific bias is not believed to have occurred (Table 3-2). The variation in results observed is considered to be most likely the result of small variations in groundwater quality during sampling, in particular because this site was sampled using disposable bailers, and turbidity at the site was observed to be moderately high (34.1 NTU). Therefore, settling of sediments in the bailer during the sampling process could have contributed to variation in two of the variable parameters that were not field filtered.

# Recommendations

Hemmera/ELR have prepared the following recommendations based on the observations and results of the February 2016 groundwater sampling program.

- Damaged or degraded wells should be repaired, if possible.

CH-P-13-02/10, CH-P-13-03/10, and MW09-01 were noted as damaged during the February 2016 and September 2015 sampling events.

CH-P-13-02/10 was reported as damaged during the time of sampling. Bentonite is present at the bottom of this well and therefore the well status had been listed as damaged. Camera footage obtained at this sample site in July 2015 (Hemmera, 2015b) confirmed the presence of bentonite and filter pack (filter sand) at the bottom of well. Bentonite was also observed seeping in the top portion of the well screen (Hemmera, 2015b). The camera footage from July 2015 suggested that the issue may be the result of improper well installation. Attempts made during the September 2015 event (Hemmera, 2015a) to remove the bentonite plug were unsuccessful. Waterra tubing was lowered into the well to attempt to force the bentonite into the tubing to remove it, however the bentonite was frozen in the well. It is recommended that this method be re-attempted in the late spring or summer of 2016. It should be noted that if bentonite is seeping into the top portion of the well screen, removal of the blockage may only provide a temporary solution.

Although suspected as frozen during the February 2016 sampling event, sample location CH-P-13-03/10 was dry during previous sampling programs. During a 2014 sampling event, the upper PVC stick-up of this well became detached from the well casing, allowing sand/filter pack material to enter the well. Camera footage obtained in July 2015 (Hemmera, 2015b) from the sample site confirmed the presence of sand at the bottom of the well. Hemmera/ELR recommends that the well be re-developed to remove the sand when conditions allow. The following methods should be considered: 1) Injecting water into the well and using air lift method to clear the well (using air compressor). Well logs should be reviewed prior to implementing this method to assess the hydraulic conductivity of the surrounding formation. A low hydraulic conductivity is required in order to saturate the sand and mobilize using an air compressor. This method may require a substantial volume of water. 2) Use a vacuum truck to remove sand. Vacuum head would need to be small enough to fit in the casing.

MW09-01 could not be sampled during previous events due to an excessive quantity of tailings present in the groundwater. Although samples were obtained from well MW09-01 during the February 2016 sampling event, groundwater at this location is extremely turbid. The well has a large gash/opening at the top of PVC. Tailings likely enter the well through this opening during periods of high water. The opening at the top of PVC was temporarily sealed during the September 2015 site visit (Hemmera, 2015a) using nitrile gloves and electrical tape. A more permanent fix should be established for this location (such as replacement of the top portion of the PVC well casing). This would require cutting the existing well and adding a coupler, PVC extension, and proper seal or j-plug. This well should also be re-developed to clear debris from the well screen. Due to low volumes and slow recharge it may be necessary to add water to this well in order to suspend the tailing material during the re-development process. Repairs and re-development of this well is recommended for the summer of 2016.

# Closure

We have appreciated the opportunity of working with you on this project and trust that this report is satisfactory to your requirements. Please feel free to contact the undersigned regarding any questions or further information that you may require.

Report prepared by:

ELR

Aaron Nicholson, B.Sc., EP

Environmental Scientist

aaron@elr.ca

Report senior reviewed by:

ELR

Chris Jastrebaski, M.Sc., R.P.Bio., P.Biol.

Project Manager

chris@elr.ca

Report senior reviewed by:

Hemmera Envirochem Inc.

Jason Wilkins, P.Ag., EP, CSAP

Director, Land Development and Projects

jwilkins@hemmera.com

# References

ASTM Standard D4448-01. 2013. Standard Guide for Sampling Groundwater Monitoring Wells. ASTM International, West Conshohocken, PA, 2013, www.astm.org.

ASTM Standard D6452-99 2012 Guide for Purging Methods for Wells used for Groundwater Quality Investigations. ASTM International, West Conshohocken, PA, 2012, www.astm.org.

Canadian Council of Ministers of the Environment (CCME). 2014. Canadian Water Quality Guidelines for the Protection of Aquatic Life. Accessed online at http://st-ts.ccme.ca/, July 2014.

Hemmera Envirochem and Ecological Logistics & Research Ltd. (Hemmera). 2015a. Mount Nansen September Groundwater and Sampling Program. Report prepared for Yukon Government Assessment and Abandon Mines Branch.

Hemmera Envirochem and Ecological Logistics & Research Ltd. (Hemmera). 2015b. Mount Nansen June Groundwater and Sampling Program. Report prepared for Yukon Government Assessment and Abandon Mines Branch.

Hemmera Envirochem and Ecological Logistics & Research Ltd. (Hemmera). 2015c. Mount Nansen March Groundwater and Sampling Program. Report prepared for Yukon Government Assessment and Abandon Mines Branch.

Hemmera Envirochem and Ecological Logistics & Research Ltd. (Hemmera). 2014a. Mount Nansen June Groundwater and Sampling Program. Report prepared for Yukon Government Assessment and Abandon Mines Branch.

Hemmera Envirochem and Ecological Logistics & Research Ltd. (Hemmera). 2014b. Mount Nansen October Groundwater and Sampling Program. Report prepared for Yukon Government Assessment and Abandon Mines Branch.

Rice, E.W., Baird, R.B., Eaton, A.D., and Clesceri, L.S. 2006. Standard Methods for the Examination of Water and Wastewater. 22nd Edition. American Water Works Association.

Yukon Government. 2011. Protocol for the Contaminated Sites Regulation under the Environment Act. Protocol No.7: Groundwater Monitoring Well Installation, Sampling and Decommissioning. Prepared pursuant to Part 6 – Administration, Section 21, Contaminated Sites Regulations, OIC 2002/171.

tables

Appendix A

Site Photographs

Appendix B

Field Forms

Appendix C

Laboratory Reports

Appendix D

Response to Client Comments