Summit Plot Data for Site Investigation/Reconnaissance, September 2013

| Plot # | UTM | Elevation (m) | General Location | Ecosystem | Slope | Aspect | Percent vegetation cover | Parent Material | Soil Characteristics | Drainage | Water Table (cm) | Permafrost Table (cm) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 388888 E, 6881443 N | 1214 | Waste Rock Dump | Anthropogenic | level | none | 15 | Anthropogenic | 80% coarse fragments (crushed gravel), sandy clay loam | Well | None | None | Natural revegetation varies. Sandy substrate is supporting higher density of vegetation growth. |
| 2 | 389438 E, 6880471 N | 1105 | South of Tailings Pond | Shrubby snowpatch fen | 25% | North | 100 | Organic veneer / Eolian | 20 cm poorly decomposed (fibric) material / 45 cm sand, | Poor | 30* | 45* | Permafrost table at 35 cm upslope |
| 3 | 387560 E, 6880965 N | 1275 | Adjacent to exploration line located west of Mine | Shrubby snowpatch fen1 | 20% | Northeast (38o) | 100 | Organic veneer / Collvium | 12 cm poorly (fibric) to moderately (mesic) decomposed material / 65 cm gravelly sandy loam | Imperfect to Poor | 40* | 65* | Comparison of ecotype for adjacent excavation line |
| 4 | 389488 E, 6881001 N | 1154 | North of Tailings Pond | Forested upland | 27% | Southeast (150o) | 100 | Colluvium | Sandy clay loam, colluvial material | Well | None | None | Deep tephra (approximately 13 cm) |
| 5 | 389309 E, 6881412 N | 1219 | East of Pit | Shrubby snowpatch fen1 | 20% | Northeast (44o) | N/A | Organic | Organic, moderately well to well decomposed (mesic to humic) | Poor | Frozen | 30 | Hummocky mounded terrain |
| 6 | 391539 E, 6880871 N | 1024 | Victoria Creek pumphouse | Forested Upland | level | none | 70 | Eolian | Sand | Well | None | None | Victoria Creek pumphouse |
| 7 | 411006 E, 6886033 N | 916 | 40 km east of mine | Forested snowpatch fen1 | 10% | Southeast (150o) | 100 | Glaciolacustrine | Silty clay | Imperfect | 30* | 40* | Glaciated area |

*Water table and permafrost tables are measured below ground surface which is defined as the surface of the mineral layer. Any organic layers are considered above ground surface.

Site Investigation Locations for Reclamation – Survey Plots

| Location | Species Planted | % Success | Comments |
| --- | --- | --- | --- |
| Below seepage pond | Willow and balsam poplar | 0% | Blackened vegetation from tailings/ seepage pond overflow |
| North west of mill site | Balsam poplar | 0% | Bare ground, very compacted |
| Adjacent to Back Creek | Willow and balsam poplar | <5% | Mounded location has some growth |

Wildlife sign observed within or directly adjacent to the Mount Nansen site:

- Six ptarmigan in shrub regrowth to the west above the Mill Site

- Detached caribou leg north of the camp, on the undisturbed hillside

- Moose antlers in the regrowth shrub to the west above the Mill Site

- Wolf tracks and scat to the west of the Mill Site, on the track through the undisturbed hillside

Warner, B.G., and C.D.A. Rubec. 1997. The Canadian Wetland Classification System. Second edition. By the Wetlands Research Centre, University of Waterloo, Ontario.