| Table 5. Canadian volcanoes listed in five threat groups, with required level of monitoring indicated. Recommended monitoring levels, and format of table, are from Ewert et al. (2005). |  |
| --- | --- |
| Volcano | Recommended Level of Monitoring |
| Level 4: Well monitored in real time |  |
| Mt. Meager Mt. Garibaldi | Monitoring should provide the ability to track detailed changes in real-time and to develop, test and apply models of ongoing and expected activity. Seismic: 12-20 stations within 20 km of vent, including several near-field sites. Network includes numerous three-component stations and mix of other instrument types, including digital broadband stations, acoustic sensors, and accelerometers. Borehole instruments where practicable. Deformation: Routine surveys along with sufficient continuous stations (GPS, tiltmeters, and/or borehole dilatometers) to track closely geodetic changes in space and time and do detailed source modeling. Gas: Frequent airborne or campaign gas measurements. Arrays of continuous sensors and other types of gas measurements as appropriate for the volcano. Hydrologic: Level-3 coverage along with real-time monitoring of hill-slope soil moisture, stream discharge, etc., as appropriate. Systems for lahar early detection where warranted. |
| Mt. Cayley Mt. Price Mt. Edziza |  |
| Level 3: Basic real-time monitoring |  |
| Nass River Mt. Silverthrone | Monitoring should provide the ability detect and track pre-eruptive and eruptive changes in real-time, with a basic understanding of what is occurring. Seismic: Network with 3-4 near-field stations and a total of at least six within 20 km of vent. Deformation: Routinely repeated surveys. At least six continuous stations (GPS and/or tiltmeters) in vicinity of volcano. LIDAR-derived images available for active features. Gas: Frequent airborne or campaign measurements of gas emissions (annually to monthly, as appropriate) along with support of 1-2 telemetered continuous sensors. Hydrologic: Level-2 coverage along with continuous-sensing probes in features of primary interest, including water wells. LIDAR-derived DEMs for lahar-runout modeling. Remote sensing: Level 2 coverage along with routine use of multi-channel thermal-infrared data from ASTER-class satellite. Thermal and/or SAR overflights, as indicated by other monitoring data. Where practicable, remote video camera in operation…continued on next page. |

| Level 2: ﻿Limited monitoring for change detection |  |
| --- | --- |
| Hoodo Mt. Nazko Cone Clearwater-Quesnel Level Mt. Heart Peaks Surprise Lake Milbanke Sound | Monitoring should provide the ability to detect and track activity frequently enough in near real time to recognize that anomalous activity is occurring. Seismic: Regional network with 1-2 near-field stations in place (within ~10 km of volcano). Geodetic: Two or more surveys for establishing baseline. InSAR observations possible on summerto-summer basis. At least three continuous stations (GPS or tiltmeters) in vicinity of volcano. Gas: Baseline of carbon-dioxide emission rate (or other gas as appropriate to the volcano). Hydrologic: Comprehensive database on temperatures and chemistry of springs and fumaroles. Remote-Sensing: Regular processing and review of near-real-time meteorological satellite images (AVHRR, GOES), and/or review of non-real-time research satellite images (e.g., MODIS) by an observatory. Baseline inventory of air photos and/or satellite images with high spatial resolution (1 m). |
| Level 1: Minimal monitoring |  |
| Fort Selkirk Iskut-Unuk Crow Lagoon Tuya-Teslin Satah-Baldface Chilcotin Basalts Monmouth Creek-Watts Point Cheakamus-Elaho Bridge River-Salal Ck Dark Mt. Watson Lake Llangorse Mt. The Thumb Bell-Irving | Monitoring should provide the ability to detect that an eruption is occurring or that gross changes are occurring/have occurred near a volcano. Seismic: Volcano lies within a regional network; no near-field stations are in place but at least one station is within 50 km of the volcano. Or, a single near-field station is present, but no regional network exists. Remote sensing: Baseline inventory exists of Landsat-class satellite images. Routine scans for eruption clouds are conducted by meteorological agencies. |