| ID | Table 5. Rock Chip Descriptions. Samples located at MMI soil pits. Refer to Table 3 for UTM. |
| --- | --- |
| P114 | white qtzite with limonite staining throughout. One face schist 2 mm. |
| P125 | Outcrop in creek. With qtzite with sericite lamellae. Limonite faces throughout. Some lamellae |
|  | show parting and incipient qtz crylstal growth. |
| P167 | Angular chocolate brown qtzite with weakly developed limonite cross fractures. |
| Y9 | 10 cm angular qtzite. |
| Y11 | 3 pieces angular phyllite/qtzite. |
| Y14 | laminated angular qtzite with 4% sericite along lamellae and throughout. Weak limonite streaks. Few vugs |
| Y26 | Outcrop 3 m long. Finely laminated qtzite with dark grey lamina<1 mm. Sericite along lamellae. |
|  | Outcrop has qtz veins to 2 cm wide with limonite vugs. E end oc silicified schist/qtzite. Irregular 5 deg slope. |
| Y26A | 2 cm qtz vein with schist walls |
| Y26B | schist/qtzite with sericite. |
| Y38 | Angular slab pale pink finely laminated qtzite with dark lamellae and much sericite. Minor limonite. |
| Y39 | Angular qtz with sericite schist walls. 5% vugs. |
| Y40 | Angular slab pale pink laminated qtzite with common sericite. 2 cross fracs with limonite and vugs. |
| Y43 | 4 angular pieces. Laminated qtzite with vuggy limonitic & sericitic partings and cross fracs. |
| Y55A | Angular slab impure qtzite with limonitic cross fracs. Sericite throughout. |
| Y55B | Angular slab 1 cm qtz vein with weak limonitic stain and few fracs. |
| Y61 | Slab qtzite/scist with limonitic lamellae. |
| Y68 | 3 pieces limonitic sericitic qtzite. |
| Y70 | Angular qtzite with irregular limonitic vugs to 5 mm and streaky vugs to 3 cm long. |
| Y72 | Angular qtz with limonitic and sericitic vugs and fine fractures. |
| Y73 | 2 angular pieces chloritized "granite" with fine limonitic streaks throughout. |
| Y78 | 2 angular pieces: one qtz, one laminated qtzite/sericite schist. |
| Y83A | Sericite schist/qtzite with strong limonitic lamellae |
| Y83B | Same as Y83A with 5 mm qtz with vuggy limonitic walls. |
| Y87 | Subangular phyllite/schist with limoniticvuggy irregular fractures with some qtz. Darker than above. |
| Y88 | 3 pieces qtzite with limonitic lamellae and few cross fracs. |
| Y95 | Angular dark qtzite with Mn and Fe stains and lamellae. |
| Y96 | Angular qtzite with much limonitic lamellae and vugs. Sericite common. |
| Y97 | 3 slabs sericite qtzite with limonitic fracs. |
| Y98 | 5 pieces sericitic qtzite with limonitic lamellae. |
| Y100 | 2 pieces angular dark schist not sampled. 1 piece crackled qtz with irregular limonitic fracs. |
| Y106 | Angular white qtzite with 4 prominent irregular sub parallel limonitic fracs with irregular cockscomb qtz. |
| Y107 | 5 pieces angular chloritic qtzite. |
| Y120 | 5 pieces angular sericited qtzite with some clay alteration. Very limonitic with irregular vugs, streaks and lamellae. |
| Y124 | Strongly lineated dark grey phyllite |
| Y125A | Angular dark grey phyllite with qtzite lamellae and low limonitic fracs. |
| Y125B | Angular qtzite/qtz with faint lamellar texture. Cross fracs common. No limonite. |
| Y129 | Angullar qtz/qtzite with weak cross fracs. |
| Y131 | 5 pieces slabby angular weakly clay altered with tight cross fracs limonite. Pale colour. |
| Y140 | Qtz 3 cm with chloritized wallrock. Almost bxia texture. |
| Y142 | 8 pieces fine grained angular pale metasediment(?) Limonite staining throughout. |
| Y144 | Angular qtz with chlorite wall on one side. Limonitic vugs and fracs throughout. |
| Y145 | 8 pieces angular limonite stained throughout fine grained sed(?) with strong foliation. Mn present. Clay alteration? |
| Y157 | Dark lamellae schist with irregular limonitic fracs |
| Y152 | Angular dark phyllite with weak limonitc lamellae |

Samples in bold type were assayed. See Table 4.