LA-ICPMS methods

Zircon grains were separated from rocks using standard techniques, annealed at 900oC for 60 hours in a muffle furnace, and mounted in epoxy and polished until their centers were exposed. Cathodoluminescence (CL) images were obtained with a JEOL JSM-1300 scanning electron microscope and Gatan MiniCL. Zircon was analyzed by laser ablation inductively coupled plasma mass spectrometry (LA-ICPMS) using a ThermoElectron X-Series II quadrupole ICPMS and New Wave Research UP-213 Nd:YAG UV (213 nm) laser ablation system. In-house analytical protocols, standard materials, and data reduction software were used for acquisition and calibration of U-Pb dates and a suite of high field strength elements (HFSE) and rare earth elements (REE). Zircon was ablated with a laser spot of 25 µm wide using fluence and pulse rates of 5 J/cm2 and 10 Hz, respectively, during a 45 second analysis (15 sec gas blank, 30 sec ablation) that excavated a pit ~25 µm deep. Ablated material was carried by a 1.2 L/min He gas stream to the nebulizer flow of the plasma. Dwell times were 5 ms for Si and Zr, 200 ms for 49Ti and 207Pb, 80 ms for 206Pb, 40 ms for 202Hg, 204Pb, 208Pb, 232Th, and 238U and 10 ms for all other HFSE and REE. Background count rates for each analyte were obtained prior to each spot analysis and subtracted from the raw count rate for each analyte. Ablations pits that appear to have intersected glass or mineral inclusions were identified based on Ti and P. U-Pb dates from these analyses are considered valid if the U-Pb ratios appear to have been unaffected by the inclusions. Analyses that appear contaminated by common Pb were rejected based on mass 204 being above baseline. For concentration calculations, background-subtracted count rates for each analyte were internally normalized to 29Si and calibrated with respect to NIST SRM-610 and -612 glasses as the primary standards. Temperature was calculated from the Ti-in-zircon thermometer (Watson et al., 2006). Because there are no constraints on the activity of TiO2, an average value in crustal rocks of 0.8 was used.

For U-Pb and 207Pb/206Pb dates, instrumental fractionation of the background-subtracted ratios was corrected and dates were calibrated with respect to interspersed measurements of zircon standards and reference materials. A zircon standard was used to monitor time-dependent instrumental fractionation based on two analyses for every 10 analyses of unknown zircon. Plešovice zircon (Sláma et al., 2008) was used for 27 samples and Zirconia (327 Ma, unpublished data, Boise State University) was used for the other 10 samples. A secondary correction to the 206Pb/238U dates was made based on results from the zircon standards that were treated as unknowns and measured once for every 10 analyses of unknown zircon. These secondary standards were Seiland (530 Ma, unpublished data, Boise State University) and Zirconia for 19 samples, Seiland only for 10 samples, and Seiland and Temora (417 Ma, unpublished data, Boise State University) for eight samples. These results showed a linear age bias that varied from +3.0 to -2.3% that is related to the 206Pb count rate. The secondary correction is thought to mitigate matrix-dependent variations due to contrasting compositions and ablation characteristics between the Plešovice zircon and other standards (and unknowns).

Radiogenic isotope ratio and age error propagation for all analyses includes uncertainty contributions from counting statistics and background subtraction. Age interpretations are based on 206Pb/238U dates. Errors on the dates are given at 2. Weighted mean dates are calculated from 34 of the 37 samples using Isoplot 3.0 (Ludwig, 2003) based on groups of analyses that yielded equivalent dates (i.e., probability of fit >0.05). An average of 31 analyses per sample are in the calculations. Weighted mean dates are not calculated for the other three samples due to a lack of equivalent dates. A weighted mean date is first calculated using errors on individual dates that do not include a standard calibration uncertainty, and then a standard calibration uncertainty was propagated into the error on the weighted mean date. This uncertainty is the local standard deviation of the polynomial fit to the interspersed primary standard measurements versus time for the time-dependent, relatively large 206Pb/238U fractionation factor. These uncertainties range from 0.8 to 1.9% (2) for 206Pb/238U. Errors on the weighted mean dates range from 1.0 to 2.1% (2), with an average of 1.5%.

# References

Ludwig, K.R., 2003. User's manual for Isoplot/Ex rev. 3.00:  a Geochronological Toolkit for Microsoft Excel., Special Publication 4. Berkeley Geochronology Center, Berkeley, 70 p.

Sláma, J. et al., 2008. Plešovice zircon — A new natural reference material for U-Pb and Hf isotopic microanalysis. Chemical Geology, vol. 249, no., p. 1-35.

Watson, B., E., Wark, D.A. and Thomas, J.B., 2006. Crystallization thermometers for zircon and rutile. Contributions to Mineralogy and Petrology, vol. 151, no., p. 413-433.